| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1098-v2 |
| --- | --- |

| Title: | Non-CE7: Context reduction in residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Chen F. Le Léannec T. Poirier | Tel: Email: | +33(0)256856325 ya.chen@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the context modelling and reduces the number of contexts for CABAC in VVC. In this contribution, the CABAC context modelling for the regular coded bins par_level_flag and abs_level_gtx_flag are investigated. With 24 contexts reduction, the BD-Rate is reported as 0.00%, 0.00%, and 0.00% for AI, RA and LDB respectively.

# Introduction

CABAC context modelling for regular coded bins sig_coeff_flag, par_level_flag and abs_level_gtx_flag is based on the sum of absolute values of partially reconstructed coefficients in a local neighborhood of current coefficient, as recalled here.

Figure 1: local neighborhood (x0,…,x4) of a current transform coefficient (X), as used in VTM-5.0 and division of luma and chroma transform blocks into region based on the diagonal (diag=x+y) index.

More precisely, the context for each regular coded bin is determined according to the following (JVET-K0072):

- significance flag (90 = 54 + 36 models): ctxIdSig = f( diag, sumAbs1, state )

- parity flag (32 = 21 + 11 models): ctxIdPar = f( diag, sumAbs1 − numSig, last )

- greaterx flag (32 = 21 + 11 models): ctxIdGtx = ctxIdPar

where:

- diag is defined as the diagonal index of the coefficient at position (x,y): diag=x+y

- sumAbs  =  sum of absolute values in local neighborhood (template)

- sumAbs1 = sum of partially reconstructed absolute levels after first pass

- numSig  = number of transform coefficients with sig_coeff_flag == 1

The local neighborhood is used for both luma and chroma transform blocks. In particular, the chroma TBs are divided into 2 regions, respectively corresponding to coefficients for which diag=0 and diag>0.

In VTM-5.0, there are 96 contexts (63 for Luma and 33 for Chroma) for both the context model index for par_level_flag and abs_level_gtx_flag in VVC is listed in Table 1.

Table 1. The context model index of par_level_flag and abs_level_gtx_flag

| Luma |  |
| --- | --- |
| Diagonal position | Context model index |
| d=0 | 16, 17,18, 19, 20 |
| d<3 | 11, 12, 13, 14, 15 |
| d<10 | 6, 7, 8, 9, 10 |
| d>=10 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |
| Chroma |  |
| Diagonal position | Context model index |
| d=0 | 6, 7, 8, 9, 10 |
| d>0 | 1, 2, 3, 4, 5 |
| Last non-zero coefficient | 0 |

# Proposed Solution

Local neighborhood removed in both chroma TB region (x+y>0 and x+y=0)

In this contribution, which was firstly proposed in JVET-M0107[1], the context modelling process for par_level_flag and abs_level_gtx_flag is modified for complexity reduction. As shown in Table 2, the local neighborhood is replaced by a single CABAC context for each of these bins, in each chroma TB region. This leads to 24 contexts less than in VTM-5.0.

Table 2. The proposed context model index of par_level_flag and abs_level_gtx_flag

| Chroma |  |
| --- | --- |
| Diagonal position | Context model index |
| d=0 | 2 |
| d>0 | 1 |
| Last non-zero coefficient | 0 |

## The corresponding derivation process of ctxInc for the syntax elements par_level_flag and abs_level_gtx_flag in the current specification is adapted.

- The variable locNumSig and locSumAbsPass1 is derived by invoking the derivation process for the variables locNumSig and locSumAbsPass1 specifies in clause 9.5.4.2.7 with colour component index cIdx, the luma location ( x0, y0), the current coefficient scan location (xC, yC ), the binary logarithm of the transform block width log2TbWidth, and the binary logarithm of the transform block height log2TbHeight as input.

- The variable ctxOffset is set equal to Min( locSumAbsPass1 − locNumSig, 4 ).

- The variable d is set equal to xC + yC.

- If xC is equal to LastSignificantCoeffX and yC is equal to LastSignificantCoeffY, ctxInc is derived as follows:

ctxInc = ( cIdx  = =  0  ?  0  :  21 )	(9-55)

- Otherwise, if cIdx is equal to 0, ctxInc is derived as follows:

ctxInc = 1 + ctxOffset + ( d  = =  0  ?  15  :  ( d < 3  ?  10  :  ( d < 10  ?  5  :  0 ) ) )	(9-56)

- Otherwise (cIdx is greater than 0), ctxInc is derived as follows:

ctxInc = 22 + ctxOffset + ( d  = =  0  ?  51  :  0 )	(9-57)

# Simulation results

The proposed method is implemented on the top of VTM-5.0. The common test conditions defined in [2] is used in the simulation. Table 3 below shows the BD-Rate of the proposed context reduction method of transform coefficients coding relative to VTM-5.0.

Table 3: Proposal performances over VTM-5.0.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.29% | 0.33% | 100% | 100% |
| Class A2 | 0.01% | 0.31% | 0.31% | 101% | 101% |
| Class B | 0.00% | 0.17% | 0.16% | 101% | 104% |
| Class C | 0.01% | 0.19% | 0.14% | 101% | 103% |
| Class E | 0.00% | 0.12% | 0.04% | 100% | 100% |
| Overall | 0.00% | 0.21% | 0.19% | 101% | 102% |
| Class D | 0.01% | 0.25% | 0.21% | 101% | 102% |
| Class F (optional) | 0.00% | 0.31% | 0.31% | 100% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.07% | 0.34% | 101% | 101% |
| Class A2 | 0.03% | 0.07% | 0.36% | 101% | 102% |
| Class B | 0.01% | 0.12% | 0.04% | 100% | 103% |
| Class C | -0.03% | 0.08% | 0.22% | 100% | 98% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.09% | 0.21% | 100% | 101% |
| Class D | 0.04% | 0.33% | 0.66% | 100% | 100% |
| Class F (optional) | 0.04% | 0.34% | 0.34% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.44% | -0.14% | 102% | 106% |
| Class C | 0.00% | 0.02% | 0.18% | 102% | 110% |
| Class E | 0.02% | -0.47% | 1.89% | 101% | 107% |
| Overall | 0.00% | 0.07% | 0.47% | 102% | 107% |
| Class D | 0.03% | 0.40% | 0.44% | 101% | 102% |
| Class F (optional) | 0.01% | 0.48% | -0.26% | 102% | 107% |

# Conclusions

This contribution proposes some ways to reduce the complexity of the regular coded bins entropy coding and the number of CABAC contexts in the same time. This is achieved through the removal of the contexts for the par_level_flag and abs_level_gtx_flag.

As can be seen, a significant reduction of the context used in VTM-5.0 can be achieved without major penalty in terms of coding efficiency.

# Reference

[1] F. Le Léannec, Y. Chen, JVET-M0107, “CE7-related: reduced local neighborhood usage for transform coefficients coding”, 13th meeting, Marrakech, MA, 9–18 Jan. 2019.

[2]	F. Bossen, J. Boyce, X. Li, et al. “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).