| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1126-v2 |
| --- | --- |

| Title: | On chroma Qp mapping tables signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Informational |  |  |
| Author(s) or Contact(s): | Sergey Ikonin Roman Chernyak Alexander Karabutov Sean McCarthy Fangjun Pu Philippe de Lagrange, | Tel: Email: | +7-906-035-18-23 sergey.ikonin@huawei.com Sean.McCarthy@dolby.com philippe.delagrange@interdigital.com |
| Source: | Huawei Technologies Co., LTD Dolby Laboratories, Inc. InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

The contributions JVET-O0186, JVET-O0298, JVET-O0433, JVET-O0562 and JVET-O0650 are proposing different methods of chroma mapping table signalling. In the track discussion, several questions were identified and recommended for offline study. This document provides information on syntax and semantics details for the related contributions.

# Introduction

The contributions JVET-O0562, JVET-O0650 operate with piece-wise representation of mapping function; JVET-O0186 operates with discreet table representation using signalling of non-increasing points; and the contributions JVET-O0298, JVET-O0433 operate with discreet mapping table representation as well but signal increment of the function in each point from the certain range.

In this report, we investigate the syntax & semantics details such as amount of bits, number of syntax elements, and text description of each category.

## Typical mapping tables signalling

Besides the default SDR chroma mapping table currently used in VVC, the following contributions provide new candidate default chroma mapping tables selected for bit cost evaluation and/or subjective quality: JVET-O0186for SDR; JVET-N0221 for HDR PQ; and JVET-O0433 for HDR HLG. Figure 1 illustrates the possible mapping tables as well as flat (1x1) table.

Figure 1. Typical mapping tables

The cost of signalling tables mentioned above was studied for the different methods. Table 1 summarizes obtained bit cost for selected typical mapping tables.

Table 1. Bit cost evaluation for several proposals for candidate mapping tables.

| Candidate default mapping tables | JVET-O0650 | JVET-O0186 | JVET-O0433 |
| --- | --- | --- | --- |
| SDR (VTM5.0) | 67 | 31 | 27 |
| SDR (JVET-O0186) | 59 | 24 | 22 |
| HDR PQ (QpC_709Cr from JVET-N0221) | 115 | 57 | 48 |
| HDR HLG (JVET-O0433) | 77 | 53 | 47 |

## Affect on specification

Each method studied has different impact on the VVC specification. Table 1 and Table 2 summarizes syntax table and semantic sections of the proposals with respect signalling of only one chroma table.

Table 2. Syntax table of the proposals (a part related to one table signalling).

| JVET-O0650 | JVET-O0186 | JVET-O0433 |
| --- | --- | --- |
| num_points_in_qp_table[ i ]; ue(v). for( j = 0; j < num_points_in_qp_table[ i ]; j++ ){. delta_qp_in_val[ i ][ j ]; ue(v). delta_qp_out_val[ i ][ j ]; ue(v). }. | cqp_flat_points_minus1; ue(v). cqp_fp0; ue(v). for ( i = 1; i <= cqp_flat_points_minus1; i++ ). cqp_delta_fp_minus1[ I ]; ue(v). | sps_qpc_cb_start_idx; u(7). sps_qpc_cb_end_idx; u(7). for( i = sps_qpc_cb_start_idx; i <= sps_qpc_cb_end_idx; i++ ). sps_qpc_cb_delta[ i ]; u(1). |

Table 3. Semantics of the proposals (a part related to one table signalling)

| JVET-O0650 | JVET-O0186 | JVET-O433 |
| --- | --- | --- |
| num_points_in_qp_table [ i ] specifies the number of points used to describe the chroma QP mapping table. The value of num_points_in_qp_table [ i ] shall be in the range of 0 to 69 + QpBdOffsetC, inclusive. delta_qp_in_val[ i ][ j ] specifies a delta value used to derive the input coordinate of the j-th pivot point of the i-th chroma QP mapping table. delta_qp_out_val[ i ][ j ] plus specifies a delta value used to derive the output coordinate of the j-th pivot point of the i-th chroma QP mapping table. The i-th chroma QP mapping tables cQPTable[ i ] for i = 0..same_qp_table_for_cb_cr ? 0 : 1 are derived as follows: qpCtr = 0 qpVal[ i ][ 0 ] = −QpBdOffsetC + delta_qp_in_val[ i ][ 0 ] cQPTable[ i ][ 0 ] = −QpBdOffsetC + delta_qp_out_val[ i ][ 0 ] for( j = 1; j < num_points_in_qp_table[ i ]; j++ ) { qpVal[ i ][ j ] = qpVal[ i ][ j – 1 ] + delta_qp_in_val[ i ][ j ] cQPTable[ i ][ j ] = cQPTable[ i ][ j – 1 ] + delta_qp_out_val[ i ][ j ] } ChromaQpTable[ i ][ qpVal[ i ][ 0 ] ] = cQPTable[ i ][ 0 ] if( qpVal[ i ][ 0 ] < −QpBdOffsetC ) { for( k = qpVal[ i ][ 0 ] − 1; k >= −QpBdOffsetC; k − − ) ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 69, ChromaQpTable[ i ][ k + 1 ] − 1 ) } for( j = 0 ; j < num_points_in_qp_table[ i ] − 1; j++ ) { for( k = cQpTable[ j ] + 1, m = 1; k <= cQPTable[ j + 1 ]; k++, m++ ) sh = delta_qp_in_val [j + 1 ] >> 1 ChromaQpTable[ i ][ k ] = ChromaQpTable[ i ][ cQpTable[ j ] ] + ( delta_qp_out_val[j + 1] * m + sh ) / delta_qp_in_val [j + 1] if( qpVal[ i ][ num_points_in_qp_table[ i ] − 1 ] != 69 ) for( k = qpVal[ i ][ num_points_in_qp_table[ i ] − 1 ] + 1; k <= 69; k++ ) ChromaQpTable[ i ][ k ] = Clip3( −QpBdOffsetC, 69, ChromaQpTable[ i ][ k − 1 ] + 1 ) … Further, the following changes are made to the chroma QP derivation: The following constraint is added: The value of qpVal[ i ][ j ] shall be greater than the value qpVal[ i ][ j – 1] for j = 1..num_points_in_qp_table[ i ]. | cqp_flat_points_minus1 plus 1 specifies the number of points of where mapping function is non-increasing. cqp_fp0 specifies the fist element of set of points where mapping function is non-increasing cqp_delta_fp_minus1[ i ] plus 1 specifies the delta value between i-th and (i-1)-th element of set of points where mapping function is non-increasing. The variable cQpFlatSize is derived as follows: cQpFlatSize = cqp_flat_points_minus1 + 1 The variable cQpFlat[] is derived as follows: cQpFlat[ 0 ] = cqp_fp0; for( i = 1; i < cQpFlatSize; i++ ) { cQpFlat[ i ] = cqp_delta_fp_minus1[ i ] + 1 + cQpFlat[ i - 1 ] } The chroma QP mapping table cqpMappingTable[] is derived as follows: cqpMappingTable [ 0 ] = 0; for( i = 1; i <= maxQP; i++ ) { incStep = 1 for ( j = 0; j < cQpFlatSize; j++){ if ( i = = cQpFlat[ j ] ) incStep = 0 } cqpMappingTable [ i ] = cqpMappingTable [ i –1 ] + incStep; } | sps_default_qpc_table_flag equal to 1 specifies that a default chroma quantization parameter QpC table specified by sps_default_qpc_idc is used. sps_default_qpc_table_flag equal to 0 specifies that default QpC table is not used. sps_qpc_cb_start_idx specifies the starting index from which the syntax array sps_qpc_cb_delta is signaled. The value of sps_qpc_cb_start_idx shall be in the range of 0 to 69, inclusive. sps_qpc_cb_end_idx specifies the ending index to which the syntax array sps_qpc_cb_delta is signaled. The value of sps_qpc_cb_end_idx shall not be less than sps_qpc_cb_start_idx and shall be in the range of 0 to 69, inclusive. sps_qpc_cb_delta[ i ] specifies the delta values between QpCb[ i ] and QpCb[i − 1] for Cb components. The value of sps_qpc_cb_delta[ i ] shall be in the range of 0 to 1. The variable QpCb[ i ], with i = 0..69, is derived as follows: - For i = 0..sps_qpc_cb_start_idx, QpCb [ i ] is set equal i. - For i = sps_qpc_cb_start_idx + 1..sps_qpc_cb_end_idx, the following applies: QpCb[ i ] = QpCb[i − 1]+ sps_qpc_cb_delta[ i ] - For i = sps_qpc_cb_end_idx + 1..69, the foolowing applies: deltaEnd = sps_qpc_cb_end_idx− QpCb[sps_qpc_cb_end_idx] QpCb[ i ] =i − deltaEnd |

# References

- S. Ikonin, R. Chernyak, and E. Alshina, “AHG15: Signalling of chroma Qp mapping table”, JVET-O0186, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- S. Paluri, J. Zhao, J. Lim, and S. H. Kim, “AHG15: Chroma Quantization QpC Parameter Signalling”, JVET-O0298, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- F. Pu, T. Lu, P. Yin, S. McCarthy, W. Husak, and T. Chen, “AHG15: chroma quantization parameters QpC table”, JVET-O0433, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- F. Bossen, and A. Segall, “AHG15: Flexible luma-to-chroma quantization parameter tables”, JVET-O0562, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- A. K. Ramasubramonian, G. V. der Auwera, D. Rusanovskyy, and M. Karczewicz, “AHG15: On signalling of chroma QP tables”, JVET-O0650, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- T. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, and T. Chen, “Chroma Quantization Parameter Qpc Table for HDR Signal”, JVET-N0221, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 14th Meeting: Geneva, CH, 19–27 March 2019.