| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1130-v3 |
| --- | --- |

| Title: | AHG8/AHG12: Decoding multiple independent layers with single-layer decoding process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela (Nokia) Stephan Wenger Byeongdoo Choi (Tencent) Emmanuel Thomas (TNO) Vadim Seregin, Muhammed Coban (Qualcomm) Yong He (InterDigital) Lulin Chen (MediaTek) | Email: | miska.hannuksela@nokia.com stewe@stewe.org bdchoi@tencent.com emmanuel.thomas@tno.nl vseregin@qti.qualcomm.com mcoban@qti.qualcomm.com Yong.He@InterDigital.com lulin.chen@mediatek.com |
| Source: | Nokia, Tencent, TNO, Qualcomm, InterDigital Communications, Inc., MediaTek Inc. |  |  |

_____________________________

# Abstract

This proposal enables using of a single decoder instance and the single-layer VVC decoding process for decoding multiple independent layers. The decoding is performed as if all NAL units resided in a single layer only.

# Introduction

This contribution contains the results of the offline work requested by the HLS BOG review on JVET-O0334, noted in the BOG minutes (JVET-O1076) as follows:

The proponent and other interested participants were requested to work on completing the layered approach based on O0334, taking into account the comments noted above, possibly taking into account related aspects in O0497 and O0393. Revisit after the output of the offline work is available.

Section 5, originally included in version 1 of the contribution, contains an approach where output layer sets are specified and HRD parameters are specific to output layer sets. Version 2 of the contribution avoids the use of the output layer sets and is contained in Sections 2 and 4 of the contribution.

# Overview of the proposal

This proposal enables using of a single decoder instance and the single-layer VVC decoding process for decoding multiple independent layers. The decoding is performed as if all NAL units resided in a single layer only.

The proposal can be used with any types of independent layers, such as "independent coded region" layers, and texture or depth layers, e.g. for 3DoF+. The type of a layer has no normative impact in the decoding process.

The proposal keeps all aspects related to independent layers in VVC Draft 5 unchanged. For example, it is still possible to decode each independent layer with a separate decoder instance.

Encoding is performed so that POC values differ across layers, i.e. at most one picture is present for a given POC value among all layers. Fig.1 shows an example of the sequence structure with independently coded region layers. Any prediction structure within an independently coded region layer can be supported by the RPL signaling, based on the POC delta indication.

Figure 1. An example of a sequence structure with independently coded region layers

The proposal has the following ingredients:

- A VPS flag (inbl_one_pic_per_poc_flag) to indicate that there is one and only one picture per each POC value among all independent layers and that the independent layers can be decoded as if there were in the same layer.

- "Collective" HRD parameters can be signalled for the bitstream that contains multiple independent layers as follows:

  - When inbl_one_pic_per_poc_flag is equal to 1, the profile-tier-level and the HRD parameters applying collectively to the independent layers are indicated as follows in the VPS:

    - A single profile-tier-level syntax structure that applies to the bitstream (as if there were only a single layer)

    - max_dec_pic_buffering_minus1, max_num_reorder_pics, and max_latency_increase_plus1 that apply collectively to the bitstream (rather than per layer)

    - hrd_parameters( ) that apply to the bitstream

  - Independent layer set nesting SEI message is specified for carrying SEI messages that apply collectively to the independent layers of the bitstream. It can be used to carry buffering period and picture timing SEI messages that apply to the bitstream (rather than per layer).

- Possibility to control by external means that the entire bitstream is decoded as if it were a single-layer bitstream. Otherwise, the current layer-wise operation of VVC is carried out.

- The layout information of independently coded region layers can be signaled in the proposed independently coded region layer SEI message. Mapping information to connect each independently coded region to each layer is also included in the SEI message.

Two options are proposed in this contribution:

  - Fig. 2 shows a simple example of independently coded regions with a layer mapping information, as a first option, which includes the offset value of the left-top position and the coded region size of each layer.

  - A second option, Fig. 3, is to indicate the 4-neighbour information of each layer, top, right, bottom and left which constitutes the layout information.

  - With any of these options, it is asserted that the granularity of the picture size of a layer is finer than that of the sub-picture in the sub-picture partitioning approach as a single layer.

When the independently coded region layer SEI message is present in the bitstream, the layout information signalled in this SEI message is used in the rendering process for composition.The composed picture for displaying is constructred by merging one or more output pictures of the independently coded region layers.

Figure 2. An example of independently coded region partitioning (Option 1 signaling)

Figure 3. An example of 4 layers connected by 4-neighbor information (Option 2 signaling)

# Responses to the comments in the HLS BoG review on July 4th

The list below provides the open comments and questions as noted in the BoG report (JVET-O1076-v1) and our responses to them.

- Different sub-pictures within an AU have different POC values. Why? Different sub-pictures with different POC values are decoded by one decoder but have different decoding times. In that case, what is the picture output process? Then it works without using layer ID.

Response: The decoding process could be implemented without references to layer ID. However, layer ID is extensively used in the proposed signalling.

- Potentially more POC LSBs might be needed for signalling of POC values, which may affect coding efficiency. How would these affect delta POC based scaling of MVs and sample values?

Response: Since the scaling is done using delta POCs and since POCs in this proposal effectively get multiplied by the number of layers (when compared to the POCs of conventional single-layer coding), the scaling of MVs and sample values is not affected.

- When merging different sub-pictures layers into one layer, rewriting of slice header would be needed due to the use of different POC values for sub-pictures originated from the same picture.

Response: There is no need to merge different sub-pictures into one layer. The decoding operates as if the pictures were in the same layer, but the neither the slice header nor the layer ID values in the NAL unit header are rewritten.

- Can different sub-pictures originated from the same picture have different values of picture_output_flag? It seems they can be just required to be the same.

Response: This constraint was added to the proposed text.

- How about HDR behavior? From the CPB behavior point of view, the single-layer process would conceptually work, just the picture rate is n times as great as if no sub-pictures and no layering were used. From the DPB point of view, it could be similarly as the CPB behavior, and in addition to that, the output process for the entire picture after merging the decoded sub-pictures could be specified normatively; this is currently not addressed.

Response: The HRD signalling and behavior were added to the proposed text.

- Some encoder constraints are needed to enable sub-picture based sub-bitstream extraction.

Response: A sub-bitstream extraction process was added in this contribution. Encoding constraints are enforced by requiring the output of the sub-bitstream to be a conforming bitstream.

- The reference picture marking needs to be layer based, i.e., layer ID is not just metadata.

Response: The refence picture marking in clause 8.3.3 of VVC Draft 5 is already layer-specific. Thus, no changes are needed in the reference picture marking text to support this proposal.

- If a layered approach is specified, would we still envision the support of decoding based on the original spatial resolution and picture rate?

- Can sub-pictures originated from the same picture have different tile/brick configurations? No, the intent is that they have the same tile/brick configuration, but currently there is no constraint for this.

  - Why?

Response: While some of the envisioned sub-picture layering use cases would not need different tile/brick configurations, no reasons were discovered why to constrain those. Hence, the proposal does not constrain tile/brick configurations.

- How is the cropping window defined? Do they refer to the same SPS or different SPSs?

Response: In VVC Draft 5, SPSs (and cropping windows therein) apply layer-wise. No change is proposed.

# Proposed specification text

## VPS syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| inbl_one_pic_per_poc_flag | u(1) |
| if( inbl_one_pic_per_poc_flag ) { |  |
| inbl_hrd_param_present_flag | u(1) |
| if( inbl_hrd_param_present_flag ) { |  |
| inbl_num_units_in_tick | u(32) |
| inbl_time_scale | u(32) |
| } |  |
| inbl_max_sub_layers_minus1 | u(3) |
| profile_tier_level( inbl_max_sub_layers_minus1 ) |  |
| inbl_sub_layer_ordering_info_present_flag | u(1) |
| for( j = ( inbl_sub_layer_ordering_info_present_flag ? 0 : inbl_max_sub_layers_minus1 ); j <= inbl_max_sub_layers_minus1; j++ ) { |  |
| inbl_max_dec_pic_buffering_minus1[ j ] | ue(v) |
| inbl_max_num_reorder_pics[ j ] | ue(v) |
| inbl_max_latency_increase_plus1[ j ] | ue(v) |
| } |  |
| if( inbl_hrd_param_present_flag ) |  |
| hrd_parameters( inbl_max_sub_layers_minus1 ) |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Additions to VPS semantics

inbl_one_pic_per_poc_flag equal to 0 specifies that coded pictures at different layers may have the same PicOrderCnt value. ols_one_pic_per_poc_flag equal to 1 specifies that the bitstream consists of independent layers and there is one and only one coded picture with a particular PicOrderCnt value in the same CVS among all layers.

inbl_hrd_param_present_flag equal to 1 specifies that the syntax elemnts inbl_num_units_in_tick, inbl_time_scale, and the syntax structure hrd_parameters( ) are present in the VPS RBSP syntax structure. inbl_timing_info_present_flag equal to 0 specifies that the syntax elemnts inbl_num_units_in_tick, inbl_time_scale, and the syntax structure hrd_parameters( ) are not present in the VPS RBSP syntax structure.

inbl_num_units_in_tick and inbl_time_scale are specified identically to num_units_in_tick and time_scale, respectively, and apply to the hrd_parameters( ) present in the VPS and to the buffering period and picture timing SEI messages that are nested within independent layer set nesting SEI message.

inbl_max_sub_layers_minus1 specifies the maximum number of temporal sub-layers that may be present.

inbl_sub_layer_ordering_info_preset_flag equal to 1 specifies that inbl_max_dec_pic_buffering_minus1[ j ], inbl_max_num_reorder_pics[ j ], and inbl_max_latency_increase_plus[ j ] are present for inbl_max_sub_layers_minus1 + 1 sub-layers. inbl_sub_layer_ordering_info_present_flag equal to 0 specifies that the values of inbl_max_dec_pic_buffering_minus1[ inbl_max_sub_layers_minus1 ], inbl_max_num_reorder_pics[ inbl_max_sub_layers_minus1 ], and inbl_max_latency_increase_plus1[ inbl_max_sub_layers_minus1 ] apply to all sub-layers.

inbl_max_dec_pic_buffering_minus1[ j ], inbl_max_num_reorder_pics[ j ], and inbl_max_latency_increase_plus[ j ] are specified identically to sps_max_dec_pic_buffering_minus1[ j ], sps_max_num_reorder_pics[ j ], and sps_max_latency_increase_plus1[ j ] for a bitstream where all the layers present.

## Additions to the decoding process in clause 8.1.1

…

For each CVS in the bitstream, the variable TargetLayerId, which identifies the target layer to be decoded, and the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, are specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetLayerId, TargetLayerId is set by the external means.

–	Otherwise, if some external means, not specified in this Specification, is available to indicate that the entire bitstream containing all layers is decoded, DecodeAllLayersFlag is set equal to 1.

–	Otherwise, TargetLayerId is set equal to vps_included_layer_id[ 0 ].

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

If TargetLayerId is provided by external means or inferred, the following applies:

–	for each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10 is applied with the CVS, TargetLayerId, and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode.

–	After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

–	Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

Otherwise (DecodeAllLayersFlag is equal to 1), the following applies:

–	It is a requirement for bitstream conformance that inbl_one_pic_per_poc_flag is equal to 1.

–	After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

–	Separate instances of variables starting with an upper case letter are derived and kept for each layer. Clause 8.1.2 is repeatedly invoked for each coded picture in the bitstream in decoding order. The upper case variables pertaining to the NuhLayerId of the coded picture provided as input to the decoding process are used in the decoding process.

## Independent layer set nesting SEI message

The independent layer set nesting SEI message provides a mechanism to associate SEI messages applying collectively to all independent layers of the bitstream. An independent layer set nesting SEI message contains one or more SEI messages.

| inbl_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Independent layer set nesting SEI | Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested. |

## Independently coded region layer SEI message

Independently coded region layer SEI message syntax

Option 1

| independent_coded_region_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| composed_pic_width_in_luma_samples | ue(v) |
| composed_pic_height_in_luma_samples | ue(v) |
| num_independent_regions_in_pic_minus1 | ue(v) |
| for( i = 0; i <= num_independent_regions_in_pic_minus1; i++) { |  |
| region_layer_id[ i ] | u(7) |
| if( i > 0 ) { |  |
| layer_region_offset_x_in_luma_samples[ i ] | ue(v) |
| layer_region_offset_y_in_luma_samples[ i ] | ue(v) |
| } |  |
| layer_region_width_in_luma_samples[ i ] | ue(v) |
| layer_region_height_in_luma_samples[ i ] | ue(v) |
| } |  |
| } |  |

Independently coded region layer SEI message semantics

An independently coded region layer SEI message provides the location and size information of a independently coded region covered by a layer.

When the independently coded region layer SEI message is present, each region in a picture is independently coded in a layer, and the location and size information of a region corresponding to a layer is used for composition of a composed output picture.

The composed output picture is constructred by merging one or more output pictures of the independently coded region layers.

num_independent_regions_in_pic_minus1 plus 1 specifies the number of independently coded regions in a picture. The value of num_independent_regions_in_pic_minus1 shall be in the range of 0 to 125, inclusive.

region_layer_id[ i ] specifies the value of NuhLayerId corresponding to the i-th independently coded region. One or more NAL units with nuh_layer_id equal to region_layer_id[ i ] shall be present in the CVS associated with this SEI message.

composed_pic_width_in_luma_samples specifies the width of a picture composed by one or more independently coded regions in units of luma samples. composed_pic_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

composed_pic_height_in_luma_samples specifies the height of a picture composed by one or more independently coded regions in units of luma samples. composed_pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

layer_region_offset_x_in_luma_samples[ i ] specifies the horizontal offset, in units of luma samples, of the top-left corner luma sample of the i-th independently coded region relative to the top-left corner luma sample of the composed picture. When not present, the value of layer_region_offset_x_in_luma_samples[ i ] is inferred to be equal to 0. layer_region_offset_x_in_luma_samples[ i ] shall be an integer multiple of MinCbSizeY.

layer_region_offset_y_in_luma_samples[ i ] specifies the vertical offset, in units of luma samples, of the top-left corner luma sample of the i-th independently coded region relative to the top-left corner luma sample of the composed picture. When not present, the value of layer_region_offset_y_in_luma_samples[ i ] is inferred to be equal to 0. layer_region_offset_y_in_luma_samples[ i ] shall be an integer multiple of MinCbSizeY.

layer_region_width_in_luma_samples[ i ] specifies the width of the i-th independently coded region in units of luma samples. layer_region_width_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

layer_region_height_in_luma_samples[ i ] specifies the height of the i-th independently coded region in units of luma samples. layer_region_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of MinCbSizeY.

Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Independently coded region layer | The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains an SEI message of the same type or starts a new CVS. |

Option 2

| independent_coded_region_layer_info( payloadSize ) { | Descriptor |
| --- | --- |
| boundary_identifier_length_minus1 | ue(v) |
| boundary_identifier_north | u(v) |
| boundary_identifier_east | u(v) |
| boundary_identifier_south | u(v) |
| boundary_identifier_west | u(v) |
| } |  |

boundary_ identifier_length_minus1 plus 1 specifies the number of bits used to represent the syntax elements boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ] and boundary_identifier_west[ i ]. The value of boundary_id_length_minus1 shall be in the range of 0 to 15, inclusive.

boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ], boundary_identifier_west[ i ] specifies the identifier respecitvely at the north, east, south and west boundary of the decoded picture of the i-th layer within the output picture. The length of the boundary_identifier_north[ i ], boundary_identifier_east[ i ], boundary_identifier_south[ i ] and boundary_identifier_west[ i ] syntax elements is boundary_id_length_minus1 + 1 bits.

When the values of the pair boundary_identifier_north[ i ] and boundary_identifier_south[ j ] or the values of the pair boundary_identifier_east[ i ] and boundary_identifier_west[ j ] are equal the decoded picture of the i-th layer and the decoded picture of the j-th layer are adjacent in the output picture and they share a common boundary respectively at the boundary north/south and east/west. Two decoded pictures adjacent by the north/south boundary are aligned on their west boundary in the output picture. Two decoded pictures adjacent by the east/west boundary are aligned on their north boundary in the output picture.

Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Independently coded region layer | The access unit containing the SEI message and up to but not including the next access unit, in decoding order, that contains an SEI message of the same type or starts a new CVS. |

# Proposed specification text (output layer set option)

## Overview

This specification text originally appeared in JVET-O1130-v1. The proposal has the following ingredients:

- "Collective" HRD parameters can be signalled for the bitstream that contains multiple independent layers as follows:

  - Output layer sets are specified in VPS to indicate which independent layers can be grouped together

  - A flag per each output layer set to indicate that there is one and only one picture per each POC value among all layers of the output layer set. When the flag is equal to 1,

    - A single profile-tier-level syntax structure that applies to the output layer set (as if there were only a single layer)

    - max_dec_pic_buffering_minus1, max_num_reorder_pics, and max_latency_increase_plus1 that apply collectively to the output layer set (rather than per layer)

    - hrd_parameters( ) that apply to the output layer set

  - Output layer set nesting SEI message is specified for the purpose to carry buffering period and picture timing SEI messages that apply to the output layer set

- Possibility to provide an output layer set index as an externally controlled variable. If the variable is provided by external means, a sub-bitstream extraction process that extracts all layers of the output layer set is invoked. Otherwise, the current layer-wise operation of VVC is carried out.

## Definitions

The following definitions are proposed similarly to the respective definitions in HEVC:

output layer: A layer of an output layer set that is output when TargetOlsIdx is equal to the index of the output layer set.

output layer set (OLS): A set of layers, where one or more layers in the set of layers are indicated to be output layers.

## VPS syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(8) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_included_layer_id[ i ] | u(7) |
| vps_reserved_zero_bit | u(1) |
| } |  |
| vps_constraint_info_present_flag | u(1) |
| vps_reserved_zero_7bits | u(7) |
| if( vps_constraint_info_present_flag ) |  |
| general_constraint_info( ) |  |
| vps_num_ols | ue(v) |
| if( vps_num_ols > 0 ) { |  |
| ols_timing_info_present_flag | u(1) |
| if( ols_timing_info_present_flag ) { |  |
| ols_num_units_in_tick | u(32) |
| ols_time_scale | u(32) |
| } |  |
| for( i = 0; i < vps_num_ols; i++ ) { |  |
| for( j = 0; j <= ols_max_layers_minus1; j++ ) |  |
| ols_layer_idx_included_flag[ i ][ j ] | u(1) |
| ols_one_pic_per_poc_flag[ i ] | u(1) |
| if( ols_one_pic_per_poc_flag[ i ] ) { |  |
| ols_max_sub_layers_minus1[ i ] | u(3) |
| profile_tier_level( ols_max_sub_layers_minus1[ i ] ) |  |
| ols_sub_layer_ordering_info_present_flag[ i ] | u(1) |
| for( j = ( ols_sub_layer_ordering_info_present_flag[ i ] ? 0 : ols_max_sub_layers_minus1[ i ] ); j <= ols_max_sub_layers_minus1[ i ]; j++ ) { |  |
| ols_max_dec_pic_buffering_minus1[ i ][ j ] | ue(v) |
| ols_max_num_reorder_pics[ i ][ j ] | ue(v) |
| ols_max_latency_increase_plus1[ i ][ j ] | ue(v) |
| } |  |
| if( ols_timing_info_present_flag ) { |  |
| ols_hrd_param_present_flag[ i ] | u(1) |
| if( ols_hrd_parame_present_flag[ i ] ) |  |
| hrd_parameters( ols_max_sub_layers_minus1[ i ] ) |  |
| } |  |
| } |  |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

## Additions to VPS semantics

vps_num_ols specifies the number of output layer sets.

ols_timing_info_present_flag equal to 1 specifies that the syntax elemnts ols_num_units_in_tick, ols_time_scale, and ols_hrd_param_present_flag[ i ] are present in the VPS RBSP syntax structure. ols_timing_info_present_flag equal to 0 specifies that the syntax elemnts ols_num_units_in_tick, ols_time_scale, and ols_hrd_param_present_flag[ i ] are not present in the VPS RBSP syntax structure.

ols_num_units_in_tick and ols_time_scale are specified identically to num_units_in_tick and time_scale, respectively, and apply to the hrd_parameters( ) present in the VPS and to the buffering period and picture timing SEI messages that are nested within output layer set nesting SEI message.

ols_layer_idx_included_flag[ i ][ j ] equal to 0 specifies that the j-th layer is not included in the i-th output layer set. ols_layer_idx_included_flag[ i ][ j ] equal to 1 specifies that the j-th layer is included in the i-th output layer set.

ols_one_pic_per_poc_flag[ i ] equal to 0 specifies that coded pictures at different layers included in the i-th output layer set may have the same PicOrderCnt value. ols_one_pic_per_poc_flag[ i ] equal to 1 specifies that there is one and only one coded picture with a particular PicOrderCnt value in the same CVS among all layers of the i-th output layer set.

ols_max_sub_layers_minus1[ i ] specifies the maximum number of temporal sub-layers that may be present in the i-th output layer set.

ols_sub_layer_ordering_info_preset_flag[ i ] equal to 1 specifies that ols_max_dec_pic_buffering_minus1[ i ][ j ], ols_max_num_reorder_pics[ i ][ j ], and ols_max_latency_increase_plus[ i ][ j ] are present for ols_max_sub_layers_minus1[ i ] + 1 sub-layers. ols_sub_layer_ordering_info_present_flag[ i ] equal to 0 specifies that the values of ols_max_dec_pic_buffering_minus1[ ols_max_sub_layers_minus1[ i ] ], ols_max_num_reorder_pics[ ols_max_sub_layers_minus1[ i ] ], and ols_max_latency_increase_plus1[ ols_max_sub_layers_minus1[ i ] ] apply to all sub-layers.

ols_max_dec_pic_buffering_minus1[ i ][ j ], ols_max_num_reorder_pics[ i ][ j ], and ols_max_latency_increase_plus[ i ][ j ] are specified identically to sps_max_dec_pic_buffering_minus1[ j ], sps_max_num_reorder_pics[ j ], and sps_max_latency_increase_plus1[ j ] for a bitstream where the layers of the i-th output layer set are present and layers not included in the i-th output layer set are not present.

ols_hrd_param_present_flag[ i ] equal to 1 specifies that the syntax structure hrd_parameters( ) is present for the i-th output layer set in the VPS RBSP syntax structure. ols_hrd_param_present_flag[ i ] equal to 0 specifies that the syntax structure hrd_parameters( ) is not present for the i-th output layer set in the VPS RBSP syntax structure.

## Additions to the decoding process in clause 8.1.1

…

For each CVS in the bitstream, the variable TargetLayerId, which identifies the target layer to be decoded, and the variable HighestTid, which identifies the highest temporal sub-layer to be decoded, are specified as follows:

–	If some external means, not specified in this Specification, is available to set TargetLayerId, TargetLayerId is set by the external means.

–	If some external means, not specified in this Specification, is available to set TargetOlsIdx, TargetOlsIdx is set by the external means.

–	Otherwise, TargetLayerId is set equal to vps_included_layer_id[ 0 ].

–	If some external means, not specified in this Specification, is available to set HighestTid, HighestTid is set by the external means.

–	Otherwise, HighestTid is set equal to sps_max_sub_layers_minus1.

If TargetLayerId is provided by external means or inferred, the following applies:

–	for each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10.1 is applied with the CVS, TargetLayerId, and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode.

–	After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

–	Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order.

Otherwise (TargetOlsIdx is provided by external means), the following applies:

–	For each CVS in the bitstream, the sub-bitstream extraction process as specified in clause 10.2 is applied with the CVS, TargetOlsIdx, and HighestTid as inputs, and the output is assigned to a bitstream referred to as CvsToDecode.

–	After that, the instances of CvsToDecode of all the CVSs are concatenated, in decoding order, and the result is assigned to the bitstream BitstreamToDecode.

–	Separate instances of variables starting with an upper case letter are derived and kept for each layer included in the output layer set with the index TargetOlsIdx. Clause 8.1.2 is repeatedly invoked for each coded picture in BitstreamToDecode in decoding order. The upper case variables pertaining to the NuhLayerId of the coded picture provided as input to the decoding process are used in the decoding process.

## Sub-bitstream extraction process for an output layer set

The sub-bitstream extraction process of clause 10 is moved under clause 10.1 and clause 10.2 is introduced as follows:

Inputs to this process are a bitstream, the index of a target output layer set olsIdxTarget, and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, olsIdxTarget equal to any value in the range of 0 to vps_num_ols_minus1, inclusive, and tIdTarget equal to any value in the range of 0 to 6, inclusive, as inputs, and that satisfies the following condition shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget and NuhLayerId equal to vps_included_layer_id[ layerIdx ] with any layerIdx value in the range of 0 to vps_max_layers_minus1, inclusive, such that ols_layer_idx_included_flag[ olsIdxTarget ][ layerIdx ] is equal to 1.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0.

The output sub-bitstream is derived as follows:

–	Remove all NAL units with TemporalId greater than tIdTarget.

–	Remove all NAL units with NuhLayerId not equal to any of the values vps_included_layer_id[ layerIdx ] for all  layerIdx values in the range of 0 to vps_max_layers_minus1, inclusive, such that ols_layer_idx_included_flag[ olsIdxTarget ][ layerIdx ] is equal to 1.

## Output layer set nesting SEI message

The ouput layer set nesting SEI message provides a mechanism to associate SEI messages with output layer sets. An output layer set nesting SEI message contains one or more SEI messages.

| ols_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| nesting_num_ops_minus1 | ue(v) |
| for( i = 0; i <= nesting_num_ops_minus1; i++ ) { |  |
| nesting_max_temporal_id_plus1[ i ] | u(3) |
| nesting_ols_idx[ i ] | ue(v) |
| } |  |
| } |  |
| while( !byte_aligned( ) ) |  |
| nesting_zero_bit /* equal to 0 */ | u(1) |
| do |  |
| sei_message( ) |  |
| while( more_rbsp_data( ) ) |  |
| } |  |

nesting_num_ops_minus1 plus 1 specifies the number of pairs of nesting_max_temporal_id_plus1[ i ] and nesting_ols_idx[ i ] present in this SEI message.

nesting_max_temporal_id_plus1[ i ] and nesting_ols_idx[ i ] specify that the nested SEI messages apply to the output of the sub-bitstream extraction process, when invoked by setting a target output layer set olsIdxTarget equal to nesting_ols_idx[ i ], and a target highest TemporalId value tIdTarget equal to nesting_max_temporal_id_plus1[ i ] − 1.

Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| Output layer set nesting SEI | Depending on the nested SEI messages. Each nested SEI message has the same persistence scope as if the SEI message was not nested. |

# Patent rights declaration(s)

Nokia, Tencent, TNO, Qualcomm, InterDigital Communications, Inc., and MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).