| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1153-v3 |
| --- | --- |

| Title: | Non-CE3: Revised intra chroma mode coding simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, Marta Karczewicz (Qualcomm) Jiaqi Zhang, Xiaozhen Zheng, Chuanmin Jia, Shanshe Wang, Siwei Ma (Peking University, DJI). Steve Keating, Karl Sharman (Sony Europe). | Email: | bray@qti.qualcomm.com geertv@qti.qualcomm.com martak@qti.qualcomm.com zhangjiaqi163mail@163.com xiaozhen.zheng@dji.com steve.keating@sony.com karl.sharman@sony.com |
| Source: | Qualcomm Inc, Peking University, DJI, Sony Europe. |  |  |

_____________________________

# Abstract

This contribution proposes to merge the proposal JVET-O0671 (using single binarization table for sps_cclm_enabled_flag=0 and 1), and JVET-O0416/JVET-O0512 (removing the context selection dependency), the results in CTC configurations are reported over VTM-5.0 as anchor.

Table 1 --  Average BD rate results for CTC

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.00% | -0.04% | -0.02% | 99% | 96% |
| RA | 0.01% | -0.08% | -0.19% | 99% | 96% |

# Introduction

This is a joint proposal combining JVET-O0671 and JVET-O0416/JVET-O0512 for simplifying the intra chroma prediction mode coding.

Firstly, for chroma intra mode coding, the current VVC specification text associates DM_CHROMA mode with intra_chroma_pred_mode=4 for sps_cclm_enabled_flag=0 and intra_chroma_pred_mode=7 for sps_cclm_enabled_flag= 1, as described below in Table 2 and Table 3 (Table 8-5 and 8-6 in the draft) [1].

Table 2 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 0

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |

Table 3 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 81 | 81 | 81 | 81 | 81 |
| 5 | 82 | 82 | 82 | 82 | 82 |
| 6 | 83 | 83 | 83 | 83 | 83 |
| 7 | 0 | 50 | 18 | 1 | X |

Secondly, two different binarization tables are described for intra chroma mode coding, depending on sps_cclm_enabled_flag value, as described in Table 4 and Table 5 (Table 9-14 and 9-15 of the draft) [1].

Table 4 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 0

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 0 |
| 0 | 100 |
| 1 | 101 |
| 2 | 110 |
| 3 | 111 |

Table 5 – Binarization for intra_chroma_pred_mode when sps_cclm_enabled_flag is equal to 1

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 7 | 0 |
| 4 | 10 |
| 5 | 1110 |
| 6 | 1111 |
| 0 | 11000 |
| 1 | 11001 |
| 2 | 11010 |
| 3 | 11011 |

Thirdly, As summarized in Table 1, the context derivation of the fourth bin (binIdx equal to 3) is relied on the value of the third bin (binIdx equal 2). When the bin value at binIdx equal to 2 is ‘0’(chroma prediction mode is either 0,1,2,3), uses bypass mode. Otherwise, when the bin value at binIdx equal to 2 is ‘1’ (chroma prediction mode is either INTRA_L_CCLM, INTRA_T_CCLM), the fourth bin (binIdx equal to 3) uses context based binary coding.

Table 6 --  context derivation table in VVC draft

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypassss | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |

# Proposal 1

As the initial proposal of JVET-O0671, it is proposed to have intra_chroma_pred_mode=4 irrespective of sps_cclm_enabled_flag value. Thus the modified Table 8-6 of VVC draft would be as specified in Table 6.

Table 7 – Specification of IntraPredModeC[ xCb ][ yCb ] depending on intra_chroma_pred_mode[ xCb ][ yCb ] and lumaIntraPredMode when sps_cclm_enabled_flag is equal to 1

| intra_chroma_pred_mode[ xCb ][ yCb ] | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | 66 | 0 | 0 | 0 | 0 |
| 1 | 50 | 66 | 50 | 50 | 50 |
| 2 | 18 | 18 | 66 | 18 | 18 |
| 3 | 1 | 1 | 1 | 66 | 1 |
| 4 | 0 | 50 | 18 | 1 | X |
| 5 | 81 | 81 | 81 | 81 | 81 |
| 6 | 82 | 82 | 82 | 82 | 82 |
| 7 | 83 | 83 | 83 | 83 | 83 |

# Proposal 2

As the initial proposal of JVET-O0671, on top of proposal 1, we propose to use a single binarization table for sps_cclm_enabled_flag=0 and 1. The modified binarization table is shown below in Table 7.

Table 8 - unified binarization table.

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 00 |
| 0 | 0100 |
| 1 | 0101 |
| 2 | 0110 |
| 3 | 0111 |
| 5 | 10 |
| 6 | 110 |
| 7 | 111 |

Here, the first bin indicates whether it is regular (0) or LM modes (1). If it is regular mode, then we derive the next bins the same way as described in Table 4. If it is LM mode, then the next bin indicates whether it is LM_CHROMA (0) or not. If it is not LM_CHROMA, next 1 bin indicates whether it is LM_L (0) or LM_A (1).  For this case, when sps_cclm_enabled_flag is 0, the first bin of the binarization table for the corresponding intra_chroma_pred_mode can be discarded prior to the entropy coding. Or, in other words, the first bin is inferred to be 0 and hence not to be coded. It also implies that for sps_cclm_enabled_flag=0, the binarization is unchanged. This single binarization table works for both sps_cclm_enabled_flag 0 and 1 case.

Please note that, with this new binarization, DM_CHROMA (4) uses 1 bin more than the original binarization, LM_CHROMA uses the same number of bins, while the other 6 remaining modes uses 1 less bin.

For our current implementation, we have used two contexts (Table 8.a): red: context 1, green: context 2, black: bypass coded. Please note that the original chroma mode coding uses three contexts (1 more context than our proposal). Please note that, there is no context selection dependency on any of the bins depending on whether bin at binIdx equal to 0 is 0 or 1, which is inspired from the contribution JVET-O0416/JVET-O0512.

Table 9 - Contexts for new binarization.

| Value of intra_chroma_pred_mode | Bin string |
| --- | --- |
| 4 | 00 |
| 0 | 0100 |
| 1 | 0101 |
| 2 | 0110 |
| 3 | 0111 |
| 5 | 10 |
| 6 | 110 |
| 7 | 111 |

Accordingly, In the VTM5 draft [1], in the Table 9.17, for intra chroma mode coding, we propose the following changes for the context selections of bin (Table 8.b).

Table 10 - Context selection for bins related to intra chroma mode coding

| binIdx | 0 | 1 | 2 | 3 | 4 | >=5 |
| --- | --- | --- | --- | --- | --- | --- |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | 0 | bypass | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 0 | 0 | 1 | 2 | bypass | bypass | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 2 = = 1 | 0 | 1 | 2 | 2 | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 0 | na | 1 | bypass | na | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 0 = = 0 | 0 | 1 | bypass | bypass | na | na |
| intra_chroma_pred_mode[ ][ ] sps_cclm_enabled_flag = = 1 && bin at binIdx equal to 0 = = 1 | 0 | 1 | bypass | na | na | na |

# Experimental results

Proposal 2 has been implemented on top of VTM5.0. The experimental results under the AI, and RA configurations are reported in Table-3 for QP values {22, 27, 32, 37}, as described for CTC [2]. From the results, it can be reported that the proposed scheme provides similar performance as VTM-5.0.

Table: CTC results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.15% | -0.19% | 100% | 100% |
| Class A2 | 0.01% | 0.06% | -0.02% | 99% | 99% |
| Class B | 0.01% | -0.03% | -0.09% | 100% | 102% |
| Class C | 0.00% | -0.09% | 0.11% | 99% | 90% |
| Class E | 0.01% | 0.05% | 0.07% | 99% | 84% |
| Overall | 0.00% | -0.04% | -0.02% | 99% | 96% |
| Class D | 0.00% | -0.04% | -0.14% | 100% | 103% |
| Class F | 0.00% | -0.03% | 0.03% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-5.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.19% | -0.16% | 99% | 99% |
| Class A2 | 0.03% | 0.31% | 0.16% | 97% | 97% |
| Class B | 0.04% | -0.06% | -0.38% | 99% | 98% |
| Class C | -0.02% | -0.29% | -0.22% | 99% | 90% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.08% | -0.19% | 99% | 96% |
| Class D | 0.06% | -0.31% | 0.02% | 100% | 102% |
| Class F | 0.01% | 0.02% | 0.07% | 101% | 100% |

# Acknowlegements

The authors would like to thank HHI for performing the crosscheck.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 5),” 14th JVET Meeting, Geneva, CH, Mar. 2019, JVET-N1001.

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Peking University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

SZ DJI Technology Co., Ltd., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).