| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1161-v1 |
| --- | --- |

| Title: | An implementation of JVET-O0568 based on the IBC buffer design of JVET-O0127 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Li Zhang, Kai Zhang, Hongbin Liu, Yue Wang (Bytedance Inc.) Jingya Li, Chong Soon Lim (Panasonic) | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. and Panasonic Corporation |  |  |

_____________________________

# Abstract

JVET-O0127 presents an IBC reference buffer design that reduces the number of operations for block vector checking; and JVET-O0568 proposes an alternative VPDU memory reuse scheme that shows bitrate saving compared with the current design. This document presents an implementation of the alternative VPDU memory reuse scheme using the IBC reference buffer design in JVET-O0127. The implementation is compatible to JVET-O0568, so that it preservs the advantages of both JVET-O0127 and JVET-O0568. Compared with the VTM-5.0 design, the BD-rate changes for class F and 4:2:0 TGM are: AI: -0.78%/-1.82%; RA: -0.55%/-1.10% and the number of operations to check a block vector for a 4x4 block can be reduced from 44 to 22.

In addition, during the discussion of JVET-O0568, IBC with 64x128/128x64 block size was mentioned. This document also reports the performance change when 64x128/128x64 IBC blocks are disabled at both the encoder and decoder. The BD-rate numbers for classF and 4:2:0 TGM are: AI: 0.00%/0.00%; RA: 0.00%/0.00%.

# Introduction

In the current VVC design, a decoder does not need to check whether a block vector is valid or not. The block vector constrains are placed at the encoder side. However, putting some simple checks to the decoder may help to identify illegal bitstreams at a decoder and also prevent an encoder from generating non-conforming bitstreams easily. JVET-O0127 proposes to have an IBC reference buffer and uses an array, named isDecoded to keep track of availability of reference samples. At the beginning of decoding a CTU row, the isDecoded array is all set to 0; at the beginning of decoding a VPDU, the corresponding part isDecoded is set to 0; after decoding a CU, the corresponding part is set to 1. Given a block vector (bv_x, bv_y), the reference block’s position relative to the IBC buffer can be calculated as

refPos_x = (currPos_x + bv_x) % IBCBufferWidth					    (1)

refPos_y = (currPos_y + bv_y) % IBCBufferHeight					    (2)

Given an IBC block vector is determined as invalid if any of the following conditions are met.

- If the refPosTL_y + IBC block height > IBCBufferHeight.

- If any reference sample is not decoded in reference block.

The design is compatible to the current design while significantly reduce the number of operations to perform block vector checking. The table 1 in [1] shows the operation reduction to check a block vector for a 4x4 block, compared with VTM-5.0. The number of operations for a 4x4 block is reduced to 22, while the number in VTM-5.0 is 44. The complexity is significantly reduced.

JVET-O0568 proposes an alternative 64x64 VPDU memory placement for IBC reference. For the case of CTU size being 128x128, by keeping two most recently decoded VPDU in the 1st VPDU row and one most recently decoded VPDU in the 2nd VPDU row, IBC reference area can be placed closer to the CU being decoded compared with the current VVC design; and thus lead to BD-rate improvement. An example of VPDU placement for IBC reference is shown in figure 1.

Figure 1. An example of 64x64 VPDU placement proposed in JVET-O0568.

This proposal shows that the VPDU placement proposed in JVET-O0568 can be implemented using the method proposed in JVET-O0127 so that advantages of both JVET-O0568 and JVET-O0127 can be kept.

# Proposed scheme

To support the VPDU placement in JVET-O0568 using marking unavailable process, we can keep track of the most recently decoded VPDU using its position (xPrev64x64, yPrev64x64) relative to the top-left position of the picture. Before decoding a new VPDU, the marking process is as follows:

- If yPrev64x64%64 is equal to 0, mark VPDU (xPrev64x64 – 128, yPrev64x64) as unvailable.

- Otherwise (i.e., yPrev64x64%64 is equal to 1), mark VPDU (xPrev64x64 – 64, yPrev64x64) as unvailable.

To implement the above process using the JVET-O0127 design, instead of maintaining an isDecoded array corresponding to 2x2 VPDUs, we maintain a isDecoded array corresponding to 4x2 VPDUs. Because isDecoded uses 1-bit for a 4x4 luma region, with the modified setting, the size of the isDecoded array is 4*2*64*64/(4*4)=2048bit or 256bytes. isDecoded is maintained as follows:

- At the beginning of decoding a CTU row, set the whole isDecoded[x][y] array to 0.

- At the beginning of decoding a VPDU,

  - if yPrev64x64%64 is equal to 0, isDecoded[x][y] array with index x = ((xPrev64x64-128)&255)/4, .., ((xPrev64x64-128)&255)/4 + 16 and y = (yPrev64x64&127)/4, .., (yPrev64x64&127)/4 + 16, is set to 0.

  - Otherwise, isDecoded[x][y] array with index x = ((xPrev64x64-64)&255)/4, .., ((xPrev64x64-64)&255)/4 + 16 and y = (yPrev64x64&127)/4, .., (yPrev64x64&127)/4 + 16, is set to 0.

  - Update variables of xPrev64x64 and yPrev64x64.

- When finishing decode a WxH CU at (xCb, yCb), isDecoded[x][y] with index x=(xCb&255)/4, .., (xCb&255)/4 + (W-1)/4 and y=(yCb&127)/4, .., (yCb&127)/4 + (H-1)/4, is set to 1.

The process to check if a block vector (bv_x, bv_y) for a WxH CU at (xCb, yCb) is valid or not is the same as that in JVET-O127, i.e.

A block vector is determined as invalid if any of the following conditions are met.

- Let refPosTL_y = yCb + bv_y. If the refPosTL_y + CbHeight > 128, the block vector is considered as invalid.

- For each NxN sublock (N=4 for luma block, 2 for chroma block for 420 input sequences) in the reference block, determine 4 corner positions of the subblock as (subRefPosTL_x, subRefPosTL_y), (subRefPosTR_x, subRefPosTR_y), (subRefPosBL_x, subRefPosBL_y) and (subRefPosBR_x, subRefPosBR_y).

  - If any of the values of isDecoded(subRefPosTL_x>>shift, subRefPosTL_y>>shift), isDecoded(subRefPosTR_x, subRefPosTR_y), isDecoded(subRefPosBL_x, subRefPosBL_y) and isDecoded(subRefPosBR_x>>shift, subRefPosBR_y>>shift) is 0, the block vector is considered as invalid.

Compared with original design of JVET-O0127, the cost is that one check is added to tell if the VPDU belongs to the even-number or odd-number VPDU row. So, the additional cost is one check per 64x64 block.

# Experimental results

The design described in section 2 has been implemented based on VTM-5.0 software, where a dedicated array is allocated at the decoder to store IBC reference samples. All IBC predictions are from the dedicated IBC reference array so that it can guarantee that all IBC reference at the decoder is within the IBC reference region as expected. Note that the current software does not operate VPDU by VPDU, so supporting 128x64 or 64x128 IBC blocks may need software re-structure. But from the decoding process point of view, there is no problem to support IBC blocks larger than 64x64. In the experiments, we simply disable IBC sizes larger than 64x64 at the encoder. The results for classF/TGM4:2:0 compared with VTM-5.0 anchor are summarized in table 1.

Table 1. Performance of the proposed implementation.

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | -0.78% | -0.71% | -0.75% | 102% | 99% | -0.55% | -0.67% | -0.57% | 101% | 103% |
| 4:2:0 TGM | -1.82% | -1.85% | -1.87% | 100% | 98% | -1.10% | -1.15% | -1.15% | 101% | 99% |

To show how much performance change is due to large IBC block sizes, we also test the performance of VTM-5.0 when IBC blocks larger than 64x64 are disabled at both encoder and decoder. The results compared with VTM-5.0 anchor are summarized in table 2.

Table 2. Performance of VTM-5.0 when 128x64 and 64x128 IBC are disabled, compared with VTM-5.0.

|  | AI |  |  |  |  | RA |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 98% | 0.00% | 0.00% | -0.01% | 100% | 101% |
| 4:2:0 TGM | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | -0.01% | -0.03% | 100% | 99% |

# Conclusions

In this document, we have shown that the VPDU memory rearrangement for IBC reference proposed in JVET-O0568 can be implemented with the method proposed in JVET-O0127, so that the advantages of both methods can be kept. The number of operations remain as 22 for a 4x4 block as indicated in JVET-O0127. The performance improvement provided by JVET-O0568 can also be kept.

Thus, we recommend adoption of both JVET-O0568 and JVET-O0127, or this proposal as a combination, into VVC.

# References

[1] J. Li and C.-S. Lim, “CE8-related: IBC modifications,” JVET-O0127, at the 15th JVET meeting, Gothenburg, SE, Jul., 2019.

[2] J. Xu, L. Zhang, K. Zhang, H. Liu and Y. Wang, “Non-CE8: An Alternative VPDU Memory Reuse for IBC,” JVET-O0568, at the 15th JVET meeting, Gothenburg, SE, Jul., 2019.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).