| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O1174 |
| --- | --- |

| Title: | Crosscheck of JVET-O1120: Non-CE5: Hadamard transform domain in-loop filter |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check |  |  |
| Author(s) or Contact(s): | Jacob Ström Torshamnsgatan 23 164 80 Stockholm Sweden | Tel: Email: | +46 702672192 jacob.strom (at) ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

A cross-check has been carried out on JVET-O1120 test 3. It was reported that bit rates match for AI and RA but there are small differences in bit rate and PSNR figures between the results presented in JVET-O1120 and the crosschecked results for ArenaOfValor and RaceHorses class C for the LDB configuration for test3. It is further reported that encoder and decoder run times match where tested.

# Encoder and decoder run times

The decoder run times are created by measuring the decoder run times of the test and comparing against the decoder run times of a previously run anchor. Hence the decoder run times in the cross checks are a bit inaccurate, but they seem to correspond well enough with the run times reported in JVET-O1120 so no more exact measurement was deemed necessary.

As for our encoder run time measurements, we selected a cluster configuration that gave maximum speed rather than exact encoder run times. Instead we did a separate run time measurement of class D RA, which agrees well with the figures in JVET-O1120. We therefore consider both decoder and encoder run times to match.

# Results

## Test 3

AI results

|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | 0.50% | 0.39% | 104% | 110% |
| Class A2 | -0.39% | 0.32% | 0.27% | 103% | 108% |
| Class B | -0.29% | 0.42% | 0.44% | 104% | 108% |
| Class C | -0.46% | 0.26% | 0.39% | 107% | 98% |
| Class E | -0.39% | 0.30% | 0.35% | 101% | 98% |
| Overall | -0.36% | 0.36% | 0.38% | 104% | 104% |
| Class D | -0.35% | 0.66% | 0.50% | 104% | 94% |
| Class F | -0.28% | 0.22% | 0.19% | 103% | 107% |

RA results

|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.29% | -0.29% | 0.09% | 100% | 103% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.39% | 0.16% | -0.16% | 102% | 102% |
| Class C | -0.48% | 0.00% | -0.02% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.37% | 0.28% | 0.24% | 100% | 96% |
| Class F | -0.25% | 0.15% | 0.33% | 103% | 100% |

LD results

|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.29% | 0.53% | 0.52% | 97% | 99% |
| Class E | -0.12% | 1.30% | 1.03% | 93% | 90% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.43% | 0.15% | 1.08% | 99% | 92% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

RA results run time check

|  | Over VTM-5.0 (host timing without MD5) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.37% | 0.28% | 0.24% | 103% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).