| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothernburg, SE, 3–12 July 2019 | Document: JVET-O1177-v1 |
| --- | --- |

| Title: | Frame_field_info text combining JVET-O0041, JVET-O0041, and JVET-O0043 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chad Fogg | Tel: Email: | chadfogg@gmail.com |
| Source: | Movie Labs |  |  |

_____________________________

Abstract

This text combines the changes necessary to combine JVET-O0041, JVET-O0042, JVET-O0043 and the decisions made in HLS BoG (JVET-O1076).

[ note to VVC editors: the length of these new bitstream elements is temporarily long in order to help editors understand the intended purpose. It is expected that the VVC text editors will shorten the variables appropriately.  Yellow highlights point out potential topic for consideration ]

# Draft text of VVC changes

## Frame field info SEI message

[ note to VVC editors: alter frame-field information SEI message syntax according to JVET-O0042 and JVET-O1076  ]

| frame_field_info( payloadSize ) { | Descriptor |
| --- | --- |
| num_output_pictures_in_coded_picture_interval | ue(v) |
| ffi_field_seq_flag | u(1) |
| if( ffi_field_seq_flag == 1 ) { |  |
| bottom_pic_flag | u(1) |
| field_is_paired_with_next_opposite_output_field_flag | u(1) |
| } else { |  |
| output_fields_from_frame_flag | u(1) |
| if( output_fields_from_frame_flag == 1 ) |  |
| output_top_field_first_flag | u(1) |
| } |  |
| source_scan_type | u(2) |
| duplicate_flag | u(1) |
| } |  |

[ Frame-field information SEI message semantics ]

The frame-field information SEI message may be used to indicate how the associated picture should be displayed, the source scan type of the associated picture, and whether the associated picture is a duplicate of a previous picture, in output order, of the same layer.

When field_seq_flag is equal to 1 for a CVS, a frame-field information SEI message shall be present for each coded picture in the CVS.

num_output_pictures_in_coded_picture_interval indicates the number of picture periods, including pictures repeated by the output process in Annex C, that the current coded picture occupies during the output process. When not present, num_output_pictures_in_coded_picture_interval is assumed to be equal to 1. When frame_field_info  is present for the current picture, the value DeltaToDivisor is equal to num_output_pictures_in_coded_picture_interval. If the coded picture is not intended to be output (when pic_output_flag == 0),  then the value of num_output_pictures_in_coded_picture_interval shall be 0.

ffi_field_seq_flag shall have the same value and interpretation as field_seq_flag in the seq_parameter_set_rbsp.

[editor: ffi_field_seq_flag is included in frame_field_info so that elements in this SEI message can be parsed independently of coding headers].

bottom_pic_flag equal to 1 indicates that the current field picture has bottom parity. bottom_pic_flaq equal to 0 indicates the current field picture has top parity.

field_paired_with_next_opposite_output_field_flag equal to 1 indicates the current coded field picture is paired with the next field picture in output order that has opposite parity of the current coded picture picture indicated by bottom_pic_flag. field_paired_with_next_opposite_output_field_flag equal to 0 indicates that the current coded field picture is paired with the previous field in output order that has opposite parity to of the current coded picture indicated by bottom_pic_flag. This element does not affect the normative output process of this standard, but can be used in the pairing of fields in post decoder processing.

output_fields_from_frame_flag equal to 1 indicates that the current coded frame sequence picture will be output as two field pictures.  field_output_from_flag_flag equal to 0 indicates that the coded frame sequence picture is output as a frame picture.

output_top_field_first_flag equal to 1 indicates the top field of the current coded frame sequence picture is output first, and the bottom field is output second. output_top_field_first_flag equal to 0 indicates that the bottom field of the current coded frame sequence picture is output first, and the top field is output second.

When fixed_pic_rate_within_cvs_flag is equal to 1, frame doubling is indicated by the combination of num_output_pictures_in_coded_picture_interval equal to 2, field_seq_flag equal to 0, and output_fields_from_frame_flag equal to 0,  the frame should be displayed two times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65, and frame tripling is indicated by the combination of num_output_pictures_in_coded_picture_interval equal to 3 and field_seq_flag equal to 0, and output_fields_from_frame_flag equal to 0, the frame should be displayed three times consecutively on displays with a frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation E-65.

NOTE 2 – Frame doubling can be used to facilitate the display, for example, of 25 Hz progressive-scan video on a 50 Hz progressive-scan display or 30 Hz progressive-scan video on a 60 Hz progressive-scan display. Using frame doubling and frame tripling in alternating combination on every other frame can be used to facilitate the display of 24 Hz progressive-scan video on a 60 Hz progressive-scan display.

The nominal vertical and horizontal sampling locations of samples in top and bottom fields for 4:2:0, 4:2:2 and 4:4:4 chroma formats are shown in Figure D.1, Figure D.2 and Figure D.3, respectively.

The field_is_paired_with_next_oppossite_output_field_flag is an association indicator for coded fields that provides hints to associate fields of complementary parity together as frames. The parity of a field can be top or bottom, and the parity of two fields is considered complementary when the parity of one field is top and the parity of the other field is bottom.

NOTE 3 – When frame_field_info is not present for the current picture, then in many cases default values may be inferred or indicated by other means. In the absence of other indications of the intended display type of a picture, the decoder should infer the num_output_pictures_in_coded_picture_interval equal to 1 when frame_field_info is not present.

source_scan_type equal to 1 indicates that the source scan type of the associated picture should be interpreted as progressive. source_scan_type equal to 0 indicates that the source scan type of the associated picture should be interpreted as interlaced. source_scan_type equal to 2 indicates that the source scan type of the associated picture is unknown or unspecified. source_scan_type equal to 3 is reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders conforming to this version of this Specification shall interpret the value 3 for source_scan_type as equivalent to the value 2.

The following applies to the semantics of source_scan_type:

–	If general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 1, the value of source_scan_type shall be equal to 0 when present, and should be inferred to be equal to 0 when not present.

–	Otherwise, if general_progressive_source_flag is equal to 1 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 1 when present and should be inferred to be equal to 1 when not present.

–	Otherwise, when general_progressive_source_flag is equal to 0 and general_interlaced_source_flag is equal to 0, the value of source_scan_type shall be equal to 2 when present and should be inferred to be equal to 2 when not present.

duplicate_flag equal to 1 indicates that the current picture is indicated to be a duplicate of a previous picture in output order. duplicate_flag equal to 0 indicates that the current picture is not indicated to be a duplicate of a previous picture in output order.

NOTE 4 – The duplicate_flag should be used to mark coded pictures known to have originated from a repetition process such as 3:2 pull-down or other such duplication and picture rate interpolation methods. This flag would commonly be used when a video feed is encoded as a field sequence in a "transport pass-through" fashion, with known duplicate pictures tagged by setting duplicate_flag equal to 1.

NOTE 5 – When field_seq_flag is equal to 1 and duplicate_flag is equal to 1, this should be interpreted as an indication that the access unit contains a duplicated field of the previous field in output order with the same parity (indicated by bottom_pic_flag) as the current field.

Table D.2 – Interpretation of frame_field_info

| picture structure (informative) | Indicated Display Output of Picture | field_seq_flag | num_output_pictures_in_coded_picture_interval | bottom_pic_flag | field_paired_with_next_opposite_output_field_flag | output_fields_from_frame_flag | output_top_field_first_flag |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | (progressive) Frame | 0 | 1 | - | - | 0 | - |
| 1 | Top field | 1 | 1 | 0 | 0 or 1 | - | - |
| 2 | Bottom field | 1 | 1 | 1 | 0 or 1 | - | - |
| 3 | Top field, bottom field, in that order | 0 | 2 | - | - | 1 | 1 |
| 4 | Bottom field, top field, in that order | 0 | 2 | - | - | 1 | 0 |
| 5 | Top field, bottom field, top field repeated, in that order | 0 | 3 | - | - | 1 | 1 |
| 6 | Bottom field, top field, bottom field repeated, in that order | 0 | 3 | - | - | 1 | 0 |
| 7 | Frame doubling | 0 | 2 | - | - | 0 | - |
| 8 | Frame tripling | 0 | 3 | - | - | 0 | - |
| 9 | Top field paired with previous bottom field in output order | 1 | 1 | 0 | 0 | - | - |
| 10 | Bottom field paired with previous top field in output order | 1 | 1 | 1 | 0 | - | - |
| 11 | Top field paired with next bottom field in output order | 1 | 1 | 0 | 1 | - | - |
| 12 | Bottom field paired with next top field in output order | 1 | 1 | 1 | 1 | - | - |
|  | repeat more than 2 pictures |  | > 3 |  |  |  |  |

[ note to VVC editors: this construct can indiate of repeated field sequence pictures, such as requested in JCTVC-J0444, though one of the purposes of duplicate_flag was to handle such scenarios by allowing the encoder to code repeated field pictures in field sequences (where the coded repeated field picture consists of all skipped CTU's) and then marking that picture with duplicate_flag = 1. We are also deliberately not included an indicator to output frame pictures from field coded pictures in a field sequence: the functionality of pic_struct 9,10,11,12 ]

[ note to VVC editors:: Update clause C.3.3 in the VVC workding draft (JVET-N1001) according to the red highlihted text below in order to better distinguish between pictures that are repeated and coded pictures  ]

## Changes to Annex C

C.3.3 Picture output

The processes specified in this clause happen instantaneously at the CPB removal time of access unit n, CpbRemovalTime[ n ].

When picture n has PictureOutputFlag equal to 1, its DPB output time DpbOutputTime[ n ] is derived as follows:

DpbOutputTime[ n ] = CpbRemovalTime[ n ] + ClockTick * picDpbOutputDelay –	(C-12)
				picDpbOutputDelta[ OpTid ]

where picDpbOutputDelay is the value of pic_dpb_output_delay and picDpbOutputDelta is the value of picDpbOutputDelta[ OpTid ] derived according to cpb_removal_delay_minus1[ OpTid ], and cpb_removal_delay_delta_idx[ OpTid ] in the picture timing SEI message associated with access unit n, and cpb_removal_delay_delta[ cpb_removal_delay_delta_idx[ OpTid ] ] in the buffering period SEI message associated with access unit n.

The output of the current picture is specified as follows:

[ note to editors: from JVET-O0043 and JVET-O1076.]

–	pictures are output with pattern (including any repetitions) according to the payload of frame_field_info SEI message (if present) associated with the picture being output.

–	If PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is equal to CpbRemovalTime[ n ], the current picture is output.

–	Otherwise, if PictureOutputFlag is equal to 0, the current picture is not output, but will be stored in the DPB as specified in clause C.3.4.

–	Otherwise (PictureOutputFlag is equal to 1 and DpbOutputTime[ n ] is greater than CpbRemovalTime[ n ] ), the current picture is output later and will be stored in the DPB (as specified in clause C.3.4) and is output at time DpbOutputTime[ n ] unless indicated not to be output by the decoding or inference of no_output_of_prior_pics_flag equal to 1 at a time that precedes DpbOutputTime[ n ].

When output, the picture is cropped, using the conformance cropping window specified in the active SPS for the picture.

[ note to editors: The red highlihted text below also tries to distinguish between pictures that are repeated and coded pictures ]

When picture n is a picture that is output and is not the last picture of the bitstream that is output, the value of the variable DpbOutputInterval[ n ] is derived as follows:

DpbOutputInterval[ n ] = DpbOutputTime[ nextCodedPicInOutputOrder ] − DpbOutputTime[ n ]	(C-13)

where nextCodedPicInOutputOrder is the coded picture that follows coded picture n in output order and has PictureOutputFlag equal to 1.

## Changes to Annex E

[ Editor: Update clause E.3.2 in the VVC workding draft (JVET-N1001) according to the red highlihted text below in order to better distinguish between pictures that are repeated and coded pictures  : ]

elemental_duration_in_tc_minus1[ i ] plus 1 (when present) specifies, when HighestTid is equal to i, the temporal distance, in clock ticks, between the elemental units that specify the HRD output times of consecutive pictures in output order as specified below. The value of elemental_duration_in_tc_minus1[ i ] shall be in the range of 0 to 2047, inclusive.

For each picture n that is output and not the last picture in the bitstream (in output order) that is output, the value of the variable DpbOutputElementalInterval[ n ] is specified by:

DpbOutputElementalInterval[ n ] = DpbOutputInterval[ n ]  DeltaToDivisor	(E-65)

where DpbOutputInterval[ n ] is specified in Equation C-13 and DeltaToDivisor is specified in Table E-5 based on the value of frame_field_info for the CVS containing picture n. Entries marked "-" in Table E-5 indicate a lack of dependence of DeltaToDivisor on the corresponding syntax element.

When HighestTid is equal to i and fixed_pic_rate_general_flag[ i ] is equal to 1 for a CVS containing picture n, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C-1 (using the value of ClockTick for the CVS containing picture n) towhen one of the following conditions is true for the following picture in output order nextCodedPicInOutputOrder that is specified for use in Equation C-13:

–	picture nextCodedPicInOutputOrder is in the same CVS as picture n.

–	picture nextCodedPicInOutputOrder is in a different CVS and fixed_pic_rate_general_flag[ i ] is equal to 1 in the CVS containing picture nextCodedPicInOutputOrder, the value of ClockTick is the same for both CVSs, and the value of elemental_duration_in_tc_minus1[ i ] is the same for both CVSs.

When HighestTid is equal to i and fixed_pic_rate_within_cvs_flag[ i ] is equal to 1 for a CVS containing picture n, the value computed for DpbOutputElementalInterval[ n ] shall be equal to ClockTick * ( elemental_duration_in_tc_minus1[ i ] + 1 ), wherein ClockTick is as specified in Equation C-1 (using the value of ClockTick for the CVS containing picture n) when the following picture in output order nextCodedPicInOutputOrder that is specified for use in Equation C-13 is in the same CVS as picture n.

Table E-5 – Divisor for computation of DpbOutputElementalInterval[ n ]

| frame_field_info is present for the current coded picture | DeltaToDivisor |
| --- | --- |
| No | 1 |
| Yes | num_output_pictures_in_coded_picture_interval |

[ Note to VVC editors: it doesn't seem useful to have a table for just two entries. Should the purpose served by the table above be a few sentences instead ?]

# VTM source code change suggests

[ Note to VVC VTM software coordinators: to support the output of fields and frames according to frame_field_info() SEI message, the following changes to the VTM reference decoder are suggested ]

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/blob/master/source/Lib/DecoderLib/DecLib.cpp

Change the determination of isField such that the equivalent of pic_struct 1,2,3,4,5,6,9,10,11,12 output field pictures to file, while pic_struct 0,7,8 output frames to file.

Change from:

[..]

if(!m_SEIs.empty())

{

// Check if any new Picture Timing SEI has arrived

SEIMessages pictureTimingSEIs = getSeisByType(m_SEIs, SEI::PICTURE_TIMING);

if (pictureTimingSEIs.size()>0)

{

SEIPictureTiming* pictureTiming = (SEIPictureTiming*) *(pictureTimingSEIs.begin());

isField    = (pictureTiming->m_picStruct == 1) || (pictureTiming->m_picStruct == 2) || (pictureTiming->m_picStruct == 9) || (pictureTiming->m_picStruct == 10) || (pictureTiming->m_picStruct == 11) || (pictureTiming->m_picStruct == 12);

isTopField = (pictureTiming->m_picStruct == 1) || (pictureTiming->m_picStruct == 9) || (pictureTiming->m_picStruct == 11);

}

}

//Set Field/Frame coding mode

m_pcPic->fieldPic = isField;

m_pcPic->topField = isTopField;

[..]

Change to:

- field picture output to file would be field for the equivlanent of pic_struct 1,2,3,4,5,6,9,10,11,12

- determination of top field first would be for pic_struct 3,5.

https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/blob/master/source/App/DecoderApp/DecApp.cpp

Change from:

Picture* pcPic = *(iterPic);

if( numPicsNotYetDisplayed>2 && pcPic->fieldPic ) //Field Decoding

{

PicList::iterator endPic   = pcListPic->end();

endPic--;

iterPic   = pcListPic->begin();

while (iterPic != endPic)

{

Picture* pcPicTop = *(iterPic);

iterPic++;

Picture* pcPicBottom = *(iterPic);

if ( pcPicTop->neededForOutput && pcPicBottom->neededForOutput &&

(numPicsNotYetDisplayed >  numReorderPicsHighestTid || dpbFullness > maxDecPicBufferingHighestTid) &&

(!(pcPicTop->getPOC()%2) && pcPicBottom->getPOC() == pcPicTop->getPOC()+1) &&

(pcPicTop->getPOC() == m_iPOCLastDisplay+1 || m_iPOCLastDisplay < 0))

{

// write to file

numPicsNotYetDisplayed = numPicsNotYetDisplayed-2;

if ( !m_reconFileName.empty() )

{

const Window &conf = pcPicTop->cs->sps->getConformanceWindow();

#if !JVET_N0063_VUI

const Window  defDisp = (m_respectDefDispWindow && pcPicTop->cs->sps->getVuiParametersPresentFlag()) ? pcPicTop->cs->sps->getVuiParameters()->getDefaultDisplayWindow() : Window();

#endif

const bool isTff = pcPicTop->topField;

bool display = true;

if( m_decodedNoDisplaySEIEnabled )

{

SEIMessages noDisplay = getSeisByType( pcPic->SEIs, SEI::NO_DISPLAY );

const SEINoDisplay *nd = ( noDisplay.size() > 0 ) ? (SEINoDisplay*) *(noDisplay.begin()) : NULL;

if( (nd != NULL) && nd->m_noDisplay )

{

display = false;

}

}

if (display)

{

m_cVideoIOYuvReconFile.write( pcPicTop->getRecoBuf(), pcPicBottom->getRecoBuf(),

m_outputColourSpaceConvert,

false, // TODO: m_packedYUVMode,

#if JVET_N0063_VUI

conf.getWindowLeftOffset(),

conf.getWindowRightOffset(),

conf.getWindowTopOffset(),

conf.getWindowBottomOffset(),

#else

conf.getWindowLeftOffset()   + defDisp.getWindowLeftOffset(),

conf.getWindowRightOffset()  + defDisp.getWindowRightOffset(),

conf.getWindowTopOffset()    + defDisp.getWindowTopOffset(),

conf.getWindowBottomOffset() + defDisp.getWindowBottomOffset(),

#endif

NUM_CHROMA_FORMAT, isTff );

}

}

// update POC of display order

m_iPOCLastDisplay = pcPicBottom->getPOC();

// erase non-referenced picture in the reference picture list after display

if ( ! pcPicTop->referenced && pcPicTop->reconstructed )

{

pcPicTop->reconstructed = false;

}

if ( ! pcPicBottom->referenced && pcPicBottom->reconstructed )

{

pcPicBottom->reconstructed = false;

}

pcPicTop->neededForOutput = false;

pcPicBottom->neededForOutput = false;

}

}

}

Change to:

- For equivalent frame_field_info to pic_struct 3,4,5,6: output fields from frame coded picture, including repeats for pic_struct cases 5,6.

- repeat frames for pic_struct 7,8, or any case of num_output_pictures_in_coded_picture_interval

# References

JVET-N1001:  Versatile Video Coding ([working] Draft 5).

# Patent rights declaration(s)

MovieLabs does not have any current or pending patent rights relating to the technology described in this contribution.