| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2023-v3 |
| --- | --- |

| Title: | Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Ling Li Alexey Filippov | Email: | geertv@qti.qualcomm.com l.li@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Abstract

This document describes Core Experiment 3 (CE3) on intra prediction and mode coding. The experiments are conducted after the 15th JVET meeting and the results, including cross-checks, will be reported to the 16th JVET meeting.

# Introduction

In CE3, a total of 6 tests are defined on the matrix intra prediction (MIP) topic.

# Test conditions and guidelines

CTC conditions as agreed at the 15th JVET meeting (Gothenburg, SE) for SDR will be used for the evaluation of objective performance (JVET-N1010 remains valid). The anchor is the ‘VVC Test Model’ or VTM. Both the ‘All Intra’ and ‘Random Access’ test configurations shall be used for all sub-CEs and providing the results for each of these configurations is mandatory. The VTM version 6.0 software shall be used with ‘vtm’ configuration files to produce the VTM anchor. Tools shall be implemented into the VTM version 6.0 software.

Proposals will primarily be compared with respect to objective compression efficiency while complexity will also be assessed.

BD-rates, encoding and decoding time comparisons, shall be reported by all proponents for at least the following case: #2 vs. #1. Additional test results may be provided, for example: #2 vs. #3.

Numbering is defined as follows for the case where ‘ToolA’ is replacing an existing ‘ToolB’ in VTM:

1.           VTM,						VTM anchor

2.           VTM + ToolA,			VTM with ToolA enabled (as a “tool on” test)

3.           VTM − ToolB            	VTM with ToolB disabled

Note: If ‘ToolB’ does not exist, then #3 = #1.

Source codes corresponding with the test descriptions contained in this document shall be uploaded to the following CE3 GitLab project according to the agreed timeline:

https://vcgit.hhi.fraunhofer.de/jvet-o-ce3/VVCSoftware_VTM.git

# Timeline

| T1 | 2 August 2019 (3 weeks after the JVET meeting) | CE test description finalized |
| --- | --- | --- |
| T2 | 2 September 2019 (Test model software release + 2 weeks) | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | 10 September 2019 (3 weeks before the next JVET meeting) | Software of CE tests is finalized; final cross-check begins |
| T4 | 17 September 2019 (Regular document deadline -1 week) | CE contribution document upload, including specification text, complete test results |
|  | 21 September 2019 (Regular document deadline -3 days) | Cross-check reports to be sent to CE coordinators |
|  | 24 September 2019 (Regular document deadline) | CE summary report upload |

# MIP-related test descriptions

## Contribution documents

JVET-O0161 Non-CE3: Simplification on MIP boundary sample downsampling process [Z. Zhang, K. Andersson, R. Sjöberg, J. Ström, P. Wennersten, R. Yu (Ericsson)]

JVET-O0323 Non-CE3: Unification of shifting for MIP mode [J.-Y. Huo (Xidian Univ.), S. Wan (NPU), Y.-Z. Ma (Xidian Univ.), J.-K. Guo (NPU), H.-X. Wang, F.-Z. Yang (Xidian Univ.), Y.-F. Yu, Y. Liu, M. Li (OPPO)]

JVET-O0408 Non-CE3: On rounding shift of MIP [K. Kondo, M. Ikeda, T. Suzuki (Sony)]

JVET-O0571 Non-CE3: Up-sampling with a fixed order in MIP [H. Liu, L. Zhang, K. Zhang, Z. Deng, J. Xu (Bytedance)]

## Defined tests

| Test # | Description | Source | Tester | Cross-checker |
| --- | --- | --- | --- | --- |
| 3-1.1 | MIP downsampling by averaging two samples from the original boundary | JVET-O0161 | Z. Zhang (Ericsson) | J. Pfaff (HHI) A. Filippov (Huawei) |
| 3-1.2 | MIP downsampling by N-tap filtering N samples from the original boundary |  | Z. Zhang (Ericsson) | J. Pfaff (HHI) V. Rufitskiy (Huawei) |
| 3-2.1 | The table of weight shift (sW) of MIP is removed. The variable (sW) is fixed value. It doesn’t depend on both MipSizeId and modeId. | JVET-O0323 JVET-O0408 | J.-Y. Huo (Xidian Univ.) K. Kondo (Sony) | J. Pfaff (HHI) |
| 3-2.2 | The table of weight shift (sW) of MIP is simplified. The variable (sW) depends on only MipSizeId. It doesn’t depend on modeId. |  | J.-Y. Huo (Xidian Univ.) K. Kondo (Sony) | J. Pfaff (HHI) |
| 3-3.1 | Up-sampling is performed firstly in the vertical direction and secondly in the horizontal direction | JVET-O0571 | Hongbin Liu (Bytedance) | C. Rosewarne (Canon) |
| 3-3.2 | Up-sampling is performed firstly in the horizontal direction and secondly in the vertical direction |  | Hongbin Liu (Bytedance) | C. Rosewarne (Canon) |

### CE3-1: MIP downsampling process (JVET-O0161 Ericsson)

Aspect two in JVET-O0161 presents a MIP downsampling process to derive reduced boundary bdryred for matrix multiplication by averaging two sample values from the original boundary. The position of the samples to be used for averaging align with the positions of the samples of reduced prediction predred in linear interpolation direction.

Figure 1 shows an example of the bdryred derivation for a 32 × 16 block. The length of the top reduced boundary LenW is equal to the length of the left reduced boundary LenH which is equal to 4.  Mleft = H ∕ LenH ∕ 2 = 16 ∕ 4 ∕ 2 = 2. Mtop = W ∕ LenW ∕ 2 = 32 ∕ 4 ∕ 2 = 4.

Fig 1. An example of bdryred derivation for a 32 × 16 block.

In the CE test, the proposed MIP downsampling method in JVET-O0161 aspect two and a variation of long taps downsampling filter will be tested:

- MIP downsampling by averaging two samples from the original boundary (same as JVET-O0161 aspect two)

The samples to be used for averaging align with samples of reduced prediction predred in linear interpolation direction. One example of bdryred derivation for a 32 × 16 block is shown in figure 1.

The downsampling process is applied to boundary sizes equal to 8, 16, 32 and 64.

- MIP downsampling by N-tap filtering N samples from the original boundary

For boundary sizes equal to 8, N = 2, the filter coefficients to use are [1 1] / 2

For boundary sizes equal to 16, N = 3, the filter coefficients to use are [1 2 1] / 4

For boundary sizes equal to 32, N = 5, the filter coefficients to use are [1 2 2 2 1] / 8

For boundary sizes equal to 64, N = 9, the filter coefficients to use are [1 2 2 2 2 2 2 2 1] / 16

The N samples to be used in the MIP downsampling process are denoted P0, P1…PN-1.

P0 and PN-1 align with the positions of the samples of reduced prediction predred in linear interpolation direction. P1…PN-2 are samples between P0 and PN-1.

Figure 2 shows an example of the N-tap downsampling filter for a 32 × 8 block.

Fig 2. An example of bdryred derivation for a 32 × 8 block.

Figure 3 shows an example of the N-tap downsampling filter for a 64 × 16 block.

Fig 3. An example of bdryred derivation for a 64 × 16 block.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-1.1 | MIP downsampling by averaging two samples from the original boundary | Z. Zhang (Ericsson) | J. Pfaff (HHI) A. Filippov (Huawei) |
| 3-1.2 | MIP downsampling by N-tap filtering N samples from the original boundary | Z. Zhang (Ericsson) | J. Pfaff (HHI) V. Rufitskiy (Huawei) |

### CE3-2: On rounding shift of MIP (JVET-O0323, JVET-O0408 Xidian Univ., NPU, OPPO, Sony)

In this test, it is considered to reduce the table size regarding weighted shift (sW). In the matrix calculation, the predicted sample predMip[ x ][ y ] is computed in the MIP using below equation as shown in JVET-O0925:

oW = ( 1 << ( sW − 1 ) ) – fO * (i = 0inSize - 1p i )

predMip[ x ][ y ] = ( (i = 0inSize - 1mWeight i  y*incH*predC+x*incW *p i ) + oW ) >> sW ) + pTemp[ 0 ]

Where, the variable sW for downshift is found by referring a table. The table sizes are currently derived depending on MipSizeId and modeId of MIP.

To simplify the table size, two methods are tested. One is that the sW is fixed value. The other method is that the sW is derived depending on only MipSizeId.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-2.1 | The table of weight shift (sW) of MIP is removed. The variable (sW) is fixed value. It doesn’t depend on both MipSizeId and modeId. | J.-Y. Huo (Xidian Univ.) K. Kondo (Sony) | J. Pfaff (HHI) |
| 3-2.2 | The table of weight shift (sW) of MIP is simplified. The variable (sW) depends on only MipSizeId. It doesn’t depend on modeId. | J.-Y. Huo (Xidian Univ.) K. Kondo (Sony) | J. Pfaff (HHI) |

### CE3-3: Up-sampling with a fixed order in MIP (JVET-O0571 Bytedance)

This contribution proposes to perform the up-sampling in a fixed up-sampling order for the prediction signal up-sampling process in MIP. Two different orders are tested.

1. Test #1: up-sampling is performed firstly in the vertical direction and secondly in the horizontal direction.
2. Test #2: up-sampling is performed firstly in the horizontal direction and secondly in the vertical direction.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 3-3.1 | Up-sampling is performed firstly in the vertical direction and secondly in the horizontal direction | Hongbin Liu (Bytedance) | C. Rosewarne (Canon) |
| 3-3.2 | Up-sampling is performed firstly in the horizontal direction and secondly in the vertical direction | Hongbin Liu (Bytedance) | C. Rosewarne (Canon) |

# Participants

The following table enumerates participants with tools in CE3 as well as participating cross-checkers and interested persons.

| Name | Company | Email | CE3-x Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Jonathan Pfaff Heiko Schwarz Benjamin Bross Philipp Merkle Santiago de Luxán Hernández Philipp Helle Bjoern Stallenberger Christian Helmrich | HHI | jonathan.pfaff@hhi.fraunhofer.de heiko.schwarz@hhi.fraunhofer.de benjamin.bross@hhi.fraunhofer.de philipp.merkle@hhi.fraunhofer.de santiago.de.luxan@hhi.fraunhofer.de philipp.helle@hhi.fraunhofer.de bjoern.stallenberger@hhi.fraunhofer.de christian.helmrich@hhi.fraunhofer.de |  | 3-1, 3-2 |
| Alexey Filippov Sergey Ikonin Jianle Chen Anand Meher Kotra Xiang Ma Vasily Rufitskiy Sychev Maxim Biao Wang Seungwook Hong Haitao Yang Semih Esenlik Yin Zhao | Huawei | Alexey.Filippov@huawei.com Sergey.Ikonin@huawei.com jianle.chen@huawei.com anand.meher.kotra@huawei.com maxiang6@huawei.com Vasily.Rufitskiy@huawei.com Sychev.Maxim@huawei.com biao.wang@huawei.com seuhong@outlook.com haitao.yang@huawei.com semih.esenlik@huawei.com yin.zhao@huawei.com |  | 3-1 |
| Steve Keating Karl Sharman Masaru Ikeda Kenji Kondo | Sony | Steve.Keating@sony.com karl.sharman@sony.com masaru.ikeda@sony.com Kenji.Kondo@sony.com | 3-2 |  |
| Zhi Zhang Rickard Sjoberg | Ericsson | zhi.a.zhang@ericsson.com rickard.sjoberg@ericsson.com | 3-1 |  |
| Chris Rosewarne Patrice Onno Guillaume Laroche Jonathan Taquet Christophe Gisquet Jonathan Gan | Canon | chris.rosewarne@canon.com.au Patrice.Onno@crf.canon.fr guillaume.laroche@crf.canon.fr jonathan.taquet@crf.anon.fr Christophe.Gisquet@crf.canon.fr jonathan.gan@canon.com.au |  | 3-3 |
| Liu Hongbin Kai Zhang Li Zhang Weijia Zhu Cynthia Zhangna Deng Zhipin | Bytedance | liuhongbin.01@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com weijia.zhu@bytedance.com zhangna.cynthia@bytedance.com zhipin.deng@bytedance.com | 3-3 |  |
| Fannie Yu | OPPO | yuyuanfang@oppo.com | 3-2 |  |
| Junyan Huo Yanzhuo Ma | Xidian University | jyhuo@mail.xidian.edu.cn yzma@mail.xidian.edu.cn | 3-2 |  |
| Shuai Wan | Northwestern Polytechnical University | swan@nwpu.edu.cn | 3-2 |  |