| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 15th Meeting: Gothenburg, SE, 3–12 July 2019 | Document: JVET-O2027-v2 |
| --- | --- |

| Title: | Description of Core Experiment 7 (CE 7): Quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Muhammed Coban Cheung Auyeung | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com cauyeung@tencent.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization, transform coefficient/residual coding. This core experiment evaluates the impact of:

- Context modelling/selection/coding order and context-coded bin limits for syntax elements related to transform coefficients/residuals

The objectives of this CE include:

- Investigation of the impact of context selection and signalling schemes for transform coefficient/residual levels on the coding efficiency and complexity.

- Investigation of the impact of context-coded bin limits on coding efficiency and complexity.

# Participants List

| Affiliation | Person, Email address |
| --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Geert Van der Auwera <geertv@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> Yung-Hsuan Chao <yunghsua@qti.qualcomm.com> |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> Benjamin Bross <benjamin.bross@hhi.fraunhofer.de> |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Andrew Segall <asegall@sharplabs.com> Frank Bossen <frank.bossen@me.com> |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> Xiaoyu Xiu <xiaoyuxiu@kwai.com> Tsung-Chuan Ma <tsung-chuanma@kwai.com> Jhuhong Jheng <jhuhong-jheng@kwai.com> |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |
| MediaKind | Julien Le Tanou <julien.letanou@mediakind.com> |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> |
| InterDigital | Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> Fabien Racape <fabien.racape@interdigital.com> Ya Chen <Ya.chen@interdigital.com> Edouard Francois <Edouard.francois@interdigital.com> Philippe de Lagrange <Philippe.delagrange@interdigital.com> Le Léannec Fabrice <Fabrice.Leleannec@interdigital.com> |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Jonathan Gan <jonathan.gan@cisra.canon.com.au> |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Yusuke Kato <kato.yusuke003@jp.panasonic.com> |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> Jane Zhao <jie.zhao@lge.com> |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |
| Comcast | Dan Grois <grois@ieee.org> |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |
| Tencent | Xin Zhao <xinzzhao@tencent.com> Xiang Li <xlxiangli@tencent.com> Cheung Auyeung <cauyeung@tencent.com> |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |
| NHK | Shunsuke Iwamura <iwamura.s-gc@nhk.or.jp > |
| Tongji University | Tao Lin <lintao@tongji.edu.cn> |
| Alibaba | Jie Chen <jiechen.cj@alibaba-inc.com> Ru-Ling Liao <ruling.lrl@alibaba-inc.com> Mohammed Golam Sarwer <m.sarwer@alibaba-inc.com> |
| Apple | Krishna Rapaka <krapaka@apple.com> |

# Tools under Test

The following tools will be considered in this CE. Details are described in sec. 6.

|  | Proponent | Document | Tool description |
| --- | --- | --- | --- |
| 1 | Mediatek | JVET-O0295 | Coding order of syntax elements and context modelling for transform skip residual coding |
| 2 | Alibaba | JVET-O0619 | Coding order of syntax elements and switch to bypass coding for transform skip residual coding |
| 3 | Qualcomm | JVET-O0623 | Coding order of syntax elements for transform skip residual coding |
| 4 | Qualcomm | JVET-O0559 | Binarization for transform skip residual coding |
| 5 | Huawei | JVET-O0231 | Modified signalling for chroma coded block flags and joint chroma residual coding |
| 6 | Mediatek | JVET-O0928 | Context reduction for sig_coeff_flag in transform coefficient coding |
| 7 | InterDigital | JVET-O1098 | Context reduction for regular transform coefficient coding |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-6.0 software (outcome of the Gothenburg meeting).  Results have to be reported for the VTM-6.0 software and the common test conditions (standard configuration files).

All tools are tested against the VTM-6.0 anchor (same set of tools, except the tool to be tested).

Results have to be provided for two ranges of QP values:

- Standard QPs (37, 32, 27, 22): AI, RA, and LD configurations as specified in CTC (all frames);

- Low QPs (17, 12, 7, 2): AI, RA, and LD configurations with 100 frames only (i.e., by using the encoder option “--FramesToBeEncoded=100”).

For category 1 and 2 tests (transform skip residual coding), additional results for screen coding content sequences (class TGM with YUV 4:2:0 format) have to be provided (using the same configuration as for class F CTC).

For category 4 tests (related to joint chroma coding), additional HDR results have to be provided (for HDR common test conditions). For HDR, only tests for the standard QP set (22, 27, 32, 37) are required (visual testing is not required).

## Category 1 and 2 test bin to bit ratio measurements

Bin to bit ratio measurements for context coded as well as bypass coded bins of transform coefficient coding have to be provided for Category 1 and 2 tests using VTM-6.0 decoder SW with

#define RExt__DECODER_DEBUG_BIT_STATISTICS                1

setting. The following stats have to be collected (JVET-N0049).

- The peak context coded bin rate, bypass bin rate and bit-rate of a coded sequence

- The average context coded bin rate, bypass bin rate and bit-rate over a coded sequence.

- The average context coded bin rate, bypass bin rate and bit-rate consumed by the transform coefficients coding over a coded sequence.

JVET-N0049 SW’s maximum context coded bins for transform coefficient syntax elements as well as overall peak context coded bins rate feature will be integrated into VTM-6.0 decoder SW. The stats for the following syntax elements for transform coding have to be used for reporting transform coefficient coding stats.

- Context-coded bins: sig_coeff_flag, abs_level_gt1_flag, par_level_flag, abs_level_gt3_flag

- Bypass-coded bins: abs_remainder, dec_abs_level

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results. For Category 4 tests with HDR, proponents have to provide results for the HDR metrics specified in JVET-N1011.

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

## Test plan

The core experiment is split into the following tests:

Category 1: Coding order and bypass switch for transform skip residual coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-1.1 | S.-T. Hsiang (Mediatek) | Re-positioning of parity flag (JVET-O0295) |  |
| CE7-1.2a | Y.-H- Chao (Qualcomm) | 2-pass variant (JVET-O0623) |  |
| CE7-1.2b | Y.-H- Chao (Qualcomm) | re-positioning of the parity flag after gtx scan (JVET-O0623) |  |
| CE7-1.2c | Y.-H- Chao (Qualcomm) | Rice-Golomb coding after bypass switch (JVET-O0623) for 2 pass scan |  |
| CE7-1.2d | Y.-H- Chao (Qualcomm) | Rice-Golomb coding after bypass switch (JVET-O0623) for 3 pass scan |  |
| CE7-1.3a | M. Sarwer (Alibaba) | Swap position of parity_flag and abs_gt3_flag (JVET-O0619) |  |
| CE7-1.3b | M. Sarwer (Alibaba) | Simplified bypass coding / unification of bypass switch for transform skip and regular transform coefficient coding (JVET-O0619) |  |
| CE7-1.3c | M. Sarwer (Alibaba) | Combination of CE7-1.3a and CE7-1.3b |  |

Category 2: Context modelling and binarization for transform skip residual coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-2.1 | S.-T. Hsiang (Mediatek) | CE7-1.1 + Improved context modelling for abs_gtx_flags and par_level_flag (JVET-O0295) |  |
| CE7-2.2a | Y.-H. Chao (Qualcomm) | QP dependent binarization (JVET-O0559) |  |
| CE7-2.2b | Y.-H. Chao (Qualcomm) | QP-dependent binarization + varying QP offset (JVET-O0559) |  |

Category 3: Context reduction for regular transform coefficient coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| CE7-3.1a | S.-T. Hsiang (Mediatek) | Context reduction for sig_coeff_flag (JVET-O0295) on 6 context subsets | C. Auyeung (Tencent) |
| CE7-3.1b | S.-T. Hsiang (Mediatek) | Context reduction for sig_coeff_flag (JVET-O0928) on 9 context subsets |  |
| CE7-3.2 | F. Le Léannec (InterDigital) | Context reduction by reducing the template for chroma blocks (JVET-O1098) |  |
| CE7-3.3a | S.-T. Hsiang (Mediatek), F. Le Léannec (InterDigital) | Combination of CE7-3.1a and CE7-3.2 |  |
| CE7-3.3b | S.-T. Hsiang (Mediatek), F. Le Léannec (InterDigital) | Combination of CE7-3.1b and CE7-3.2 |  |

Category 4: Chroma cbf coding

| CE7-4.1 | S. Esenlik (Huawei) | Coding of chroma cbf’s and joint chroma residual coding flag (JVET-O0231) |  |
| --- | --- | --- | --- |

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | 3 weeks after meeting | - Final CE description uploaded |
| --- | --- | --- |
| T2 | 2 weeks after VTM-5 release | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | 3 weeks before next JVET meeting or T2 + 1 week (whichever is later) | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |
| T4 | regular document deadline – 1 week | - CE contribution documents including specification text and complete test results are uploaded to the JVET document repository |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

# Detailed Description of Test

## Subblock processing flow and context modelling (JVET-O0295)

In CE7-1.1, the syntax elements abs_level_gtx_flag[n][j], j = 1..4, is signalled in a single pass. The syntax par_level_flag is signaled in a separate pass after signalling abs_level_gtx[n].

In CE7-2.1, abs_level_gtx_flag is entropy coded conditioned on the two neighbouring level values. par_level_flag is entropy coded conditioned on the partially reconstructed absolute level value of the current coefficient. This test is conducted on top of CE7-1.1.

## Context reduction for sig_coeff_flag (JVET-O0928)

In the proposed tests, the number of the context variables in a context subset associated with a TB zone is reduced to 2 for some context subsets. The 1st context variable is selected when locSumAbsPass1 is equal to 0. The 2nd context variable is selected, otherwise. Context reduction is applied to 6 context subsets in CE7-3.1 and to 9 context subset in CE7-3.1b, as illustarted in Figure 1.

Figure 1: Illustration of the tests for context reduction for sig_coeff_flag.

## Context reduction by reduced usage of local neighbourhood (JVET-O1098)

This CE test proposes to remove the usage of the local neighbourhood for the context derivation for syntax elements par_level_flag and abs_level_gtx_flag in some regions of the chroma transform blocks (TB).

As shown in Table 1, the local neighbourhood is replaced by a single CABAC context for each of these bins, in each chroma TB region.

Table 1: proposed context model index of par_level_flag and abs_level_gtx_flag

| Chroma |  |
| --- | --- |
| Diagonal position | Context model index |
| d=0 | 2 |
| d>0 | 1 |
| Last non-zero coefficient | 0 |

Therefore, the index of the contexts used for coding par_level_flag and abs_level_gtx_flag is determined as follows:

ctxInc = (diag = = 0 ? 1 : 0)

where diag is determined as follows based on the scan location (xC,yC) of the considered coefficient:

diag=xC+yC .

As a consequence, the sum of partially reconstructed coefficient in the local neighbourhood of current coefficient, denoted locSumAbsPass1 in VVC draft specification, needs no more being computed for chroma TBs. Additionally, 24 contexts are removed compared to the current VVC draft specification with proposed method.

Figure 2: proposed modification in the context derivation for par_level_flag and abs_level_gtx_flag in chroma TBs

## Modified chroma coded block flag signalling (JVET-O0231)

The current contribution proposes an alternative way of coding the coded block flags for Cb and Cr blocks. Based on the statistical observation, a first flag (tu_cbf_cb_or_cr) indicating whether Cb or Cr  blocks have residual, and a second flag (tu_cbf_cb_and_cr) indicating that both Cb and Cr blocks have residual are included in the syntax. The joint CbCr residual flag will be signalled when tu_cbf_cb_or_cr flag is true.

According to the proposal, the following is the modified syntax table applicable to VVC draft 6:

| tu_cbf_cb_or_cr[ x0 ][ y0 ] | ae(v) |
| --- | --- |
| if( tu_cbf_cb_or_cr[ x0 ][ y0 ] ) { |  |
| if( sps_joint_cbcr_enabled_flag ) |  |
| tu_joint_cbcr_residual[ x0 ][ y0 ] | ae(v) |
| if( !tu_joint_cbcr_residual[ x0 ][ y0 ] ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA) |  |
| tu_cbf_cb_and_cr[ x0 ][ y0 ] | ae(v) |
| if( !tu_cbf_cb_and_cr[ x0 ][ y0 ] ) |  |
| tu_cbf_cb [ x0 ][ y0 ] | ae(v) |

tu_joint_cbcr_residual and tu_cbf_cb_and_cr flags are inferred to be zero when not present in the bitstream. The difference between the modified syntax and the original proposal is as follows:

For non-intra CU: No difference. The tu_cbf_cb_and_cr is present in bitstream only if tu_joint_cbcr_residual is true. tu_joint_cbcr_residual indicates that both Cb and Cr blocks have residual and coded jointly.

For intra CU: tu_cbf_cb_and_cr is present in bitstream even if tu_joint_cbcr_residual is false, due to the adoption of JVET-O0105.

## Interleaved coefficient coding, parity flag re-ordering and bypass binarization in TS residual coding (JVET-O0623)

This contribution proposes to reduce the number of passes when encoding transform-skip coefficients as well as re-positioning of the parity flag and different binarization after switch to bypass coding. The following 3 aspects are proposed and tested.

- 2 scan passes: In the first pass, sig flag, sign flag, gt1, parity, and remaining gtx flags (gt3, gt5, gt7, gt9) are coded. In the second pass, the remainder is coded.  If the context bin limit is reached in the first pass, all the syntax elements will be bypass coded.

- Re-positioning of parity flag: encode/parse parity flag after gtx flags.

- Binarization after switch to bypass coding:

  - For 2 pass scan: sig flag, sign flag, gt1, parity, and remaining gtx flags (gt3, gt5, gt7, gt9) are coded in the first pass. If no available regular coded bins are left, all remaining sig and sign flags are bypass coded for the rest of the coefficients. If the regular bin count is reached after coding of a gt1 flag, the remaining present par, gtx (gt3, gt5, gt7, gt9) flags for that coefficient are bypass coded. If  the regular bin count is reached before coding of a gt1 flag, the remaining present gt1, par, gtx (gt3, gt5, gt7, gt9) flags for that coefficient are not coded. After the coefficient where the regular bin count is reached, the gt1, par, gtx (gt3, gt5, gt7, gt9) flags will not be coded in the first pass. In the second pass, for the coefficient with gt9 flag equal to 1, the remainder is coded. For significant coefficients after (including) reaching the regular bin count, abs(coeff) – 1 will be encoded with Rice-Golomb code.

  - For 3 pass scan: sig flag, sign flag are coded in the first pass, and gt1, parity, and remaining gtx flags (gt3, gt5, gt7, gt9) are coded in the second pass. If the regular bin count is reached after coding of a gt1 flag, the remaining present par, gtX flags for that coefficient are bypass coded. After the coefficient where the regular bin count is reached, the gt1, par, gtx (gt3, gt5, gt7, gt9) flags will not be coded. In the third pass, for the coefficient with gt9 equal to 1, the remainder is coded. For significant coefficients after (including) reaching the regular bin count, abs(coeff) – 1 will be encoded with Rice-Golomb code.

Subtests:

- 2 scan passes

- Re-positioning the parity flag

- Binarization after bypass switch for 2 pass scan

- Binarization after bypass switch for 3 pass scan

## QP dependent binarization in TS residual coding (JVET-O0559)

In this proposal, a new binarization method for transform skip residual coding is proposed, where the binarization of absolute coefficient levels is adjusted based on the dynamic range of the quantized residuals. In addition, a variable QPoffset is introduced which modifies QP to be QP – QPoffset. The maximum dynamic range (which depends on the QP value), is divided into m intervals. The following binarization is used: The syntax elements sig_coeff_flag, abs_level_gt1_flag and coeff_sign_flag are transmitted. When abs_level_gt1_flag is equal to 1, the interval index and the remainder inside the interval is transmitted.

Subsets:

- QP dependent binarization

- Varying QP offset across TS coded blocks for anchor and proposed QP dependent binarization (delta QP offset signaling for both anchor and proposed binarization). Note: this is to demonstrate the performance of proposed binarization.

## Modified coding order and bypass switch for transform skip residual coding (JVET-O0619)

In the test 1, the position of parity flag is moved to second pass and the position of the greater than 3 flag is moved to the first pass.

- Pass 1: following flags are signalled

  - sig_coeff_flag

  - coeff_sign_flag

  - greater 1 flag (abs_level_gtx_flag[0])

  - greater than 3 flag (abs_level_gtx_flag[1])

- Pass 2: following flags are signalled

  - parity (par_level_flag) flag

  - greater than 5 flag (abs_level_gtx_flag[2])

  - greater than 7 flag (abs_level_gtx_flag[3])

  - greater than 9 flag (abs_level_gtx_flag[4])

- Pass 3: by-pass coding of remaining absolute level (abs_remainder) using Golomb-Rice coding

In order to unify the by-pass coding process of transform and TS residual coding, test 2 proposes to modify the by-pass coding method of TS residual coding.

Below is the pseudo code:

Pass 1: for(n = 0; n <= numSbCoeff  - 1 && remainingCtxBin >= 4; n++ )

decode sig_coeff_flag

decode coeff_sign_flag

decode abs_level_gtx_flag[0] // gt1 flag

decode par_level_flag

iFirstPassBypassPos = n;

Pass 2: for(n = 0; n < iFirstPassBypassPos && remainingCtxBin >= 4; n++ )

decode abs_level_gtx_flag[1] // gt3

decode abs_level_gtx_flag[2] // gt5

decode abs_level_gtx_flag[3] // gt7

decode abs_level_gtx_flag[4] // gt9

iSecondPassBypassPos = n;

Pass 3 (a): for(n = 0; n < iFirstPassBypassPos; n++ )

baseLevel = (n >= iSecondPassBypassPos ? 2 : 10);

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

Pass 3 (b): for(n = iFirstPassBypassPos; n <= numSbCoeff  - 1; n++ )

baseLevel = 0

decode_bypass_sig_flag

if(sig_flag) {

rice = cctx.templateAbsSumTS(n, coeff, baseLevel);

decode abs_remainder_using_RG_Coding

decode coeff_sign_flag

}

The Rice parameter is derived based on locSumAbs and baseLevel.

locSumAbs  += (10 – baseLevel)

locSumAbs = clip3 (0  , 31, locSumAbs)