| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0240 |
| --- | --- |

| Title: | AHG9: Additional constant slice header parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Kashyap Kammachi-Sreedhar Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes signalling of patterns of reference picture list indices and slice QP delta values in the constant slice header parameters of PPS.

# Introduction

It is observed that the common test conditions result into the use of periodic patterns for reference picture list indices and slice QP delta values. It is therefore proposed to include the signalling of reference picture list indices and slice QP delta values in the constant slice header parameters of PPS. A pattern of reference picture list indices and slice QP delta values is included in the PPS. The correct entry within the pattern is selected for decoding based on the POC correspondence between an entry in the pattern and the current picture.

The table below provides an estimated rate-distortion performance for the random access common test conditions:

|  | Random access |  |  |
| --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |
|  | Y | U | V |
| Class A1 | -0.06 % | -0.06 % | -0.06 % |
| Class A2 | -0.05 % | -0.05 % | -0.05 % |
| Class B | -0.25 % | -0.26 % | -0.26 % |
| Class C | -0.56 % | -0.56 % | -0.55 % |
| Class E |  |  |  |
| Overall | -0.26 % | -0.26 % | -0.25 % |
| Class D | -1.60 % | -1.60 % | -1.61 % |
| Class F | -0.94 % | -0.94 % | -0.94 % |

# Overview of the proposal

## PPS syntax

The following changes are proposed (with color coding to indicate the correspondence between the overview and the syntax):

- The previously reserved value 3 of pps_ref_pic_list_sps_idc[ i ] is specified to indicate a constant pattern-based assignment of reference picture list indices.

- pps_slice_qp_delta_constant_flag is specified to indicate a constant pattern-based assignment of slice QP delta values.

- Relative POC values in the pattern are signalled with fixed-length codes (pps_const_sh_poc_lsb[ i ]).

- For the pattern of reference picture list indices, the following is specified:

- pps_irap_no_rpl_flag specifies if the pattern starts with the reference picture list indices for the initial IRAP picture.

- The reference picture list index pattern may have an initial part that is not used repetitively. The number of entries in the initial part is indicated with pps_rpl_idx_initial.

- The reference picture list indices in the pattern are provided in pps_rpl_idx[ i ][ j ] (for j equal to 0 and 1).

- For the pattern of slice QP delta values, the following is specified:

- The slice QP delta pattern may have an initial part that is not used repetitively. The number of entries in the initial part is indicated with pps_qp_delta_initial_minus1 + 1.

- The slice QP delta values in the pattern are provided in pps_qp_delta[ i ].

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| pps_slice_qp_delta_constant_flag | u(1) |
| if( pps_ref_pic_list_sps_idc[ 0 ] = = 3 ¦ ¦ pps_ref_pic_list_sps_idc[ 1 ] = = 3 ¦ ¦ pps_slice_qp_delta_constant_flag = = 1 ) { |  |
| pps_const_sh_period_minus1 | ue(v) |
| pps_const_sh_poc_lsb_len_minus1 | ue(v) |
| for( i = 0; i <=pps_const_sh_period_minus1 + 1; i++ ) |  |
| pps_const_sh_poc_lsb[ i ] | u(v) |
| } |  |
| if( pps_ref_pic_list_sps_idc[ 0 ] = = 3 ¦ ¦ pps_ref_pic_list_sps_idc[ 1 ] = = 3 ) { |  |
| pps_rpl_idx_len_minus1 | ue(v) |
| pps_irap_no_rpl_flag | u(1) |
| pps_rpl_idx_initial | ue(v) |
| for( i = pps_irap_no_rpl_flag; i <= 1 + pps_rpl_idx_initial + pps_const_sh_period_minus1; i++ ) { |  |
| if( pps_ref_pic_list_sps_idc[ 0 ] = = 3 ) |  |
| pps_rpl_idx[ i ][ 0 ] | u(v) |
| if( rpl1_idx_present_flag && pps_ref_pic_list_sps_idc[ 1 ] = = 3 ) |  |
| pps_rpl_idx[ i ][ 1 ] | u(v) |
| } |  |
| } |  |
| if( pps_slice_qp_delta_constant_flag ) { |  |
| pps_qp_delta_initial_minus1 | ue(v) |
| for( i = 0; i <= 1 + pps_qp_delta_initial_minus1 + pps_const_sh_period_minus1; i++ ) |  |
| pps_qp_delta[ i ] | se(v) |
| } |  |
| } |  |
| … |  |

## Derivation of the slice header parameters

The derivation of ConstSHIdx for indexing pps_rpl_idx[ ConstSHIdx ][ 0 ], pps_rpl_idx[ ConstSHIdx ][ 1 ], and pps_qp_delta[ ConstSHIdx ] is performed as follows:

- When a picture is an IRAP picture, ConstSHIdx is set equal to 0.

- When a picture is the first picture in a sequence that references the same PPS and is not an IRAP picture, ConstSHIdx is set equal to 1.

- A POC value is derived from pps_const_sh_poc_lsb[ i ] relative to the POC of the previous IRAP picture or the first picture in a sequence that referenced the same PPS.

- If the derived POC value is equal to the POC value of the current picture, the correct value of ConstSHIdx has been found.

- Otherwise, ConstSHIdx is incremented by 1 (with wraparound when the length of the pattern is reached) and the process continues with step 3.

This process is asserted to be robust against temporal sub-layer pruning.

# Rate-distortion performance in random access common test conditions

The accompanying .xlsm file provides an estimate of the compression gain in random access common test conditions (RA CTC). The gain is estimated as follows:

- Deriving the additional bitrate caused by the additions in PPS syntax.

- Deriving the reduced bitrate caused by absent syntax elements in picture headers and slice headers.

- The obtained bitrate values are used as the tested bitstreams in rate-distortion calculations against VTM-7 anchor bitstreams.

- The derivation does not take the impact of byte alignment into account. Thus, the derived rate-distortion performance values should be considered as estimates rather than exact values.

An excerpt of a "flattened" PPS syntax element value table applying to RA CTC is provided below.

| pps_slice_qp_delta_constant_flag | 1 |
| --- | --- |
| if( pps_ref_pic_list_sps_idc[ 0 ] = = 3 ¦ ¦ pps_ref_pic_list_sps_idc[ 1 ] = = 3 ¦ ¦ pps_slice_qp_delta_constant_flag = = 1 ) { |  |
| pps_const_sh_period_minus1 | 15 |
| pps_const_sh_poc_lsb_len_minus1 | 4 |
| for( i = 0; i <=pps_const_sh_period_minus1 + 1; i++ ) |  |
| pps_const_sh_poc_lsb[ 0 ] | 0 |
| pps_const_sh_poc_lsb[ 1 ] | 16 |
| pps_const_sh_poc_lsb[ 2 ] | 8 |
| pps_const_sh_poc_lsb[ 3 ] | 4 |
| pps_const_sh_poc_lsb[ 4 ] | 2 |
| pps_const_sh_poc_lsb[ 5 ] | 1 |
| pps_const_sh_poc_lsb[ 6 ] | 3 |
| pps_const_sh_poc_lsb[ 7 ] | 6 |
| pps_const_sh_poc_lsb[ 8 ] | 5 |
| pps_const_sh_poc_lsb[ 9 ] | 7 |
| pps_const_sh_poc_lsb[ 10 ] | 12 |
| pps_const_sh_poc_lsb[ 11 ] | 10 |
| pps_const_sh_poc_lsb[ 12 ] | 9 |
| pps_const_sh_poc_lsb[ 13 ] | 11 |
| pps_const_sh_poc_lsb[ 14 ] | 14 |
| pps_const_sh_poc_lsb[ 15 ] | 13 |
| pps_const_sh_poc_lsb[ 16 ] | 15 |
| } |  |
| if( pps_ref_pic_list_sps_idc[ 0 ] = = 3 ¦ ¦ pps_ref_pic_list_sps_idc[ 1 ] = = 3 ) { |  |
| pps_rpl_idx_len_minus1 | 4 |
| pps_irap_no_rpl_flag | 1 |
| pps_rpl_idx_initial | 4 |
| for( i = pps_irap_no_rpl_flag; i <= 1 + pps_rpl_idx_initial + pps_const_sh_period_minus1; i++ ) { |  |
| if( pps_ref_pic_list_sps_idc[ 0 ] = = 3 ) |  |
| pps_rpl_idx[ 1 ][ 0 ] | 16 |
| pps_rpl_idx[ 2 ][ 0 ] | 17 |
| pps_rpl_idx[ 3 ][ 0 ] | 18 |
| pps_rpl_idx[ 4 ][ 0 ] | 19 |
| pps_rpl_idx[ 5 ][ 0 ] | 4 |
| pps_rpl_idx[ 6 ][ 0 ] | 5 |
| pps_rpl_idx[ 7 ][ 0 ] | 6 |
| pps_rpl_idx[ 8 ][ 0 ] | 7 |
| pps_rpl_idx[ 9 ][ 0 ] | 8 |
| pps_rpl_idx[ 10 ][ 0 ] | 9 |
| pps_rpl_idx[ 11 ][ 0 ] | 10 |
| pps_rpl_idx[ 12 ][ 0 ] | 11 |
| pps_rpl_idx[ 13 ][ 0 ] | 12 |
| pps_rpl_idx[ 14 ][ 0 ] | 13 |
| pps_rpl_idx[ 15 ][ 0 ] | 14 |
| pps_rpl_idx[ 16 ][ 0 ] | 15 |
| pps_rpl_idx[ 17 ][ 0 ] | 0 |
| pps_rpl_idx[ 18 ][ 0 ] | 1 |
| pps_rpl_idx[ 19 ][ 0 ] | 2 |
| pps_rpl_idx[ 20 ][ 0 ] | 3 |
| if( rpl1_idx_present_flag && pps_ref_pic_list_sps_idc[ 1 ] = = 3 ) |  |
| pps_rpl_idx[ i ][ 1 ] | u(v) |
| } |  |
| } |  |
| if( pps_slice_qp_delta_constant_flag ) { |  |
| pps_qp_delta_initial_minus1 | 0 |
| for( i = 0; i <= 1 + pps_qp_delta_initial_minus1 + pps_const_sh_period_minus1; i++ ) |  |
| pps_qp_delta[ 0 ] | −5 |
| pps_qp_delta[ 1 ] | −1 |
| pps_qp_delta[ 2 ] | 2 |
| pps_qp_delta[ 3 ] | 5 |
| pps_qp_delta[ 4 ] | 6 |
| pps_qp_delta[ 5 ] | 7 |
| pps_qp_delta[ 6 ] | 7 |
| pps_qp_delta[ 7 ] | 6 |
| pps_qp_delta[ 8 ] | 7 |
| pps_qp_delta[ 9 ] | 7 |
| pps_qp_delta[ 10 ] | 5 |
| pps_qp_delta[ 11 ] | 6 |
| pps_qp_delta[ 12 ] | 7 |
| pps_qp_delta[ 13 ] | 7 |
| pps_qp_delta[ 14 ] | 6 |
| pps_qp_delta[ 15 ] | 7 |
| pps_qp_delta[ 16 ] | 7 |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).