| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0244-v2 |
| --- | --- |

| Title: | [AHG12]: Misc improvements to tile and rectangular slice signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Hendry Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com dr.hendry@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the tile and rectangular signalling:

- Signal a flag to specify whether the explicitly signalled tile row is the same as the explicitly signalled tile column. If true, tile row information can be derived from tile column information.

- Signal a flag for each rectangular slice in the picture to indicate if the slice height in tiles is the same as the slice width in tiles. If this flag is false, then the slice height is explicitly signalled.

- Signal rectangular slice width in tile only when number of tile column is greater than 1; likewise, signal rectangular slice height only when the number of tile row is greater than 1.

- Signal the number of rectangular slices in a tile only when the number of CTU rows within the tile is greater than 1.

- It is asserted that the second condition for the presence of tile_idx_delta[ i ] (i.e., i < num_slices_in_pic_minus1) is not really needed as it is always true anyway. Therefore, it is proposed to remove it.

- It is asserted that the derivation of CtbToTileColBd[ ] and CtbToTileRowBd[ ] have out of bound / out of range problems. Detail fixes to the asserted problems are given below

# Introduction

The current signalling method for tiles and slices can be improved to benefit some of the typical situations. A case in point is when a picture is partitioned into multiple tiles, it is a common scenario to have the picture split into equal number of tile columns and tile rows. It is also asserted that the current spec text may have bugs

The current signalling can be improved in several ways as described in the Section 2 below.

# Proposal

In this document we propose 6 aspects that can be considered for improvement. These include:

Aspect 1: Signal a flag to specify whether explicitly signalled tile row is the same as the explicitly singalled tile column. If true, tile row information can be derived from tile column information.

Aspect 2: Signal a flag for each slice in the picture to indicate if the slice height in tiles is the same as the slice width in tiles. If this flag is false, then the slice height is explicitly signalled.

Aspect 3: Signal rectangular slice width in tile only when number of tile column is greater than 1; likewise, signal rectangular slice height only when the number of tile row is greater than 1.

Aspect 4: Signal the number of rectangular slices in a tile only when the number of CTU rows within the tile is greater than 1.

Aspect 5: It is asserted that the second condition for the presence of tile_idx_delta[ i ] (i.e., i < num_slices_in_pic_minus1) is not really needed as it is always true anyway. Therefore, it is proposed to remove it.

Aspect 6: It is asserted that the derivation of CtbToTileColBd[ ] and CtbToTileRowBd[ ] have out of bound / out of range problems. Detail fixes to the asserted problems are given below.

## Aspect 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| exp_tile_rows_same_flag | u(1) |
| if( !exp_tile_rows_same_flag ) { |  |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| } |  |
| … |  |
| } |  |

no_pic_partition_flag equal to 1 specifies that no picture partitioning applied to each picture referring to the PPS. no_pic_partition_flag equal to 0 specifies each picture referring to the PPS may be partitioned into more than one tile or slice.

…

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns_minus1 is inferred to be equal to 0.

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1, inclusive. tile_column_width_minus1[ num_exp_tile_columns_minus1 ] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 as specified in clause 6.5.1. When not present, the value of tile_column_width_minus1[ 0 ] is inferred to be equal to PicWidthInCtbsY − 1.

exp_tile_rows_same_flag equal to 1 specifies that the syntax element num_exp_tile_rows_minus1 and tile_row_height_minus1[ i ] are  not present. exp_tile_rows_same_flag equal to 0 specifies that the syntax element num_exp_tile_rows_minus1 and tile_row_height_minus1[ i ] are  not present.

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0. When not present and the value of no_pic_partition_flag is equal to 0, the value of num_exp_tile_rows_minus1 is inferred to be equal to num_exp_tile_columns_minus1.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1, inclusive. tile_row_height_minus1[ num_exp_tile_rows_minus1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 as specified in clause 6.5.1. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1. When not present, the following applies:

- If no_pic_partition_flag is equal to 0 and exp_tile_rows_same_flag is equal to 1, the value of tile_row_height_minus1[ i ] is inferred to be equal to the value of tile_column_width_minus1[ i ]

- Otherwise, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1

## Aspect 2

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_same_flag[ i ] | u(1) |
| if( !slice_height_in_tiles_same_flag[ i ] ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| … |  |
| } |  |

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th rectangular slice in units of tile columns. The value of slice_width_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileColumns − 1, inclusive. When not present, the value of slice_width_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

slice_height_in_tiles_same_flag[ i ] equal to 1 specifies that the height of the i-th rectangular slice is the same as the width of the i-th rectangular slice. slice_height_in_tiles_same_flag[ i ] equal to 0 specifies that the height of the i-th rectangular slice may not be the same as the width of the i-th rectangular slice.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of tile rows. The value of slice_height_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileRows − 1, inclusive. When not present and slice_height_in_tiles_same_flag is equal to 1, the value of slice_height_in_tiles_minus1[ i ] is inferred to be equal to the value of slice_width_in_tiles_minus1[ i ]. Otherwise, the value of slice_height_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

CTB raster scanning, tile scanning, and subpicture scanning processes

For rectangular slices, the list NumCtuInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the number of CTU in the i-th slice, and the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, and j ranging from 0 to NumCtuInSlice[ i ] − 1, inclusive, specifying the picture raster scan address of the j-th CTB within the i-th slice, are derived as follows:

if( subpics_present_flag  &&  single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i < PicSizeInCtbsY; i ++ ) {
		sliceIdx = CtbToSubPicIdx[ i ]
		CtbAddrInSlice[ sliceIdx ][ NumCtuInSlice[ sliceIdx ] ] = i
		NumCtuInSlice[ sliceIdx ]++
	}
} else {
	tileIdx = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns

if( slice_height_in_tiles_same_flag[ i ] )

slice_height_in_tiles_minus1[ i ] = slice_width_in_tiles_minus1[ i ]
		if( i  = =  num_slices_in_pic_minus1 ) {
			slice_width_in_tiles_minus1[ i ] = NumTileColumns − 1 − tileX
			slice_height_in_tiles_minus1[ i ] = NumTileRows − 1 − tileY
			num_slices_in_tile_minus1[ i ] = 0
		}
		if( slice_width_in_tiles_minus1[ i ]  = =  0  &&  slice_height_in_tiles_minus1[ i ]  = =  0 ) {	(29)
			ctbY = tileRowBd[ tileY ]
			numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ]
			for( j = 0; j < numSlicesInTileMinus; j++ ) {
				AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + slice_height_in_ctu_minus1[ i ] + 1 )
				ctbY  +=  slice_height_in_ctu_minus1[ i ] + 1
				i++
			}
			AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], ctbY, tileRowBd[ tileY + 1 ] )
		} else
			for( j = 0; j  <=  slice_height_in_tiles_minus1[ i ]; j++ )
				for( k = 0; k  <=  slice_width_in_tiles_minus1[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],
						tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )
		if( tile_idx_delta_present_flag )
			tileIdx  +=  tile_idx_delta[ i ]
		else {
			tileIdx  +=  slice_width_in_tiles_minus1[ i ] + 1
			if( tileIdx % NumTileColumns  = =  0 )
				tileIdx  +=  slice_height_in_tiles_minus1[ i ] * NumTileColumns
		}
	}
}

## Aspect 3

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| if( NumTileColumns > 1 ) |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( NumTileRows > 1 ) |  |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| … |  |
| } |  |

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th rectangular slice in units of tile columns. The value of slice_width_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileColumns − 1, inclusive. When not present and the value of NumTileColumns is equal to 1, the value of slice_width_in_tiles_minus1[ i ] is inferred to be equal to 0, otherwise, the value of slice_width_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of tile rows. The value of slice_height_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileRows − 1, inclusive. When not present and the value of NumTileRows is equal to 1, the value of slice_height_in_tiles_minus1[ i ] is inferred to be equal to 0, otherwise, the value of slice_height_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

## Aspect 4

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| … |  |
| } |  |

…

For rectangular slices, the list NumCtuInSlice[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the number of CTU in the i-th slice, SliceTopLeftTileIdx[ i ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, specifying the index of the top-left tile of the slice, and the matrix CtbAddrInSlice[ i ][ j ] for i ranging from 0 to num_slices_in_pic_minus1, inclusive, and j ranging from 0 to NumCtuInSlice[ i ] − 1, inclusive, specifying the picture raster scan address of the j-th CTB within the i-th slice, are derived as follows:

if( subpics_present_flag  &&  single_slice_per_subpic_flag ) {
	for( i = 0; i  <=   sps_num_subpics_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i < PicSizeInCtbsY; i ++ ) {
		sliceIdx = CtbToSubPicIdx[ i ]
		CtbAddrInSlice[ sliceIdx ][ NumCtuInSlice[ sliceIdx ] ] = i
		NumCtuInSlice[ sliceIdx ]++
	}
} else {
	tileIdx = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
		SliceTopLeftTileIdx[ i ] = tileIdx
		…
	}
}

## Aspect 5

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| … |  |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

## Aspect 6

In subclause 6.5.1, for the derivation of CtbToTileColBd[ ctbAddrX ], the loop runs from index 0 to the PicWidthInCtbY, inclusive. It is asserted that the right end of loop has out of bound issue. Likewise, the derivation of CtbToTileRowBd[ ctbAddrY ] also has the same problem.

Furthermore, the condition “if( ctbAddrX  = =  tileColBd[ tileX + 1 ] )” may have an unpredictable behavior when cttbAddrX is the last CTU in the CTU row of the picture since the value of tileColBd[ tileX + 1 ] does not exist as tileX + 1 would be already out of bound. Likewise, the same problem also exists for the condition “if( ctbAddrY  = =  tileRowBd[ tileY + 1 ] )”

If the above asserted problems are valid, the following changes are suggested:

The list CtbToTileColBd[ ctbAddrX ] for ctbAddrX ranging from 0 to PicWidthInCtbsY − 1, inclusive, specifying the conversion from a horizontal CTB address to a left tile column boundary in units of CTBs, is derived as follows:

tileX = 0
for( ctbAddrX = 0; ctbAddrX  <=  PicWidthInCtbsY; ctbAddrX++ ) {
	if( tileX < NumTileColumns – 1  &&  ctbAddrX  = =  tileColBd[ tileX + 1 ] )	(27)
		tileX++
	CtbToTileColBd[ ctbAddrX ] = tileColBd[ tileX ]
}

The list CtbToTileRowBd[ ctbAddrY ] for ctbAddrY ranging from 0 to PicHeightInCtbsY − 1, inclusive, specifying the conversion from a vertical CTB address to a top tile column boundary in units of CTBs, is derived as follows:

tileY = 0
for( ctbAddrY = 0; ctbAddrY  <=  PicHeightInCtbsY; ctbAddrY++ ) {
	if( tileY < NumTileRows – 1  &&  ctbAddrY  = =  tileRowBd[ tileY + 1 ] )	(28)
		tileY++
	CtbToTileRowBd[ ctbAddrY ] = tileRowBd[ tileY ]
}

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).