| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0246-v3 |
| --- | --- |

| Title: | [AHG9]: On virtual boundary signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Hendry Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com dr.hendry@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

In the current design, if the virtual boundary information is not needed, there are two flags (i.e., sps_virtual_boundaries_present_flag and ph_virtual_boundaries_present_flag) still need to be signalled. Particularly, the signalling of ph_virtual_boundaries_present_flag still need to be present in every picture header even when no virtual boundary is needed. It is asserted that this redundant and unnecessary signalling should be fixed.

This contribution proposes a flag (i.e., sps_virtual_boundaries_enabled_flag) in the SPS to specify whether loop filters may be enabled along virtual boundaries in the coded pictures within the CLVS. When this flag is equal to 1, virtual boundary signalling may be present in either SPS or PH, but not in both.

In the first revision of this contribution, update to the author list was made.

In the second revision of this contribution, the sps_virtual_boundaries_disabled_flag was updated to be sps_virtual_boundaries_enabled_flag and necessary update to the text due to this change was made.

# Introduction

In the existing design of the loop filter control across virtual boundaries, the signalling may be present either in SPS or in picture header. Considering that virtual boundaries is usually used only in certain applications / use-case scenarios such as for gradual decoding refresh (GDR) or for certain projection formats for 3600 videos (e.g., cube-map based projection format), when such feature is not used, the signalling of it should be minimal.

When virtual boundary information is not needed, in the current signalling design, one of the following options for signalling is used. It is asserted that both methods introduce redundancy as follows:

- Set the value of sps_virtual_boundaries_present_flag to be equal to 0 and for every picture header, the syntax element ph_virtual_boundaries_present_flag is set equal to 0.

- Set the value of sps_virtual_boundaries_present_flag to be equal to 1, which consequently requires the signalling of syntax elements and set sps_num_ver_virtual_boundaries and sps_num_hor_virtual_boundaries which need to be set equal to 0.

Furthermore, for the second option above, in addition to being redundant, it may also cause problem for decoding process since when sps_virtual_boundaries_present_flag is equal to 1, decoder expects further signalling of the location of the virtual boundaries.

# Proposal

We propose the following:

- Signal a new flag called sps_virtual_boundaries_ enabled_flag in SPS. This flag specifies whether virtual boundary information may be present and be needed for the decoding process of pictures in the CLVS.

- When sps_virtual_boundaries_ enabled_flag is equal to 0, there is no further signalling about virtual boundaries in SPS or in PH.

- When sps_virtual_boundaries_ enabled_flag is equal to 1, virtual boundaries signalling may be present in either SPS or in PH.

  - When it is specified that virtual boundary signalling is present in an SPS, at least one virtual boundary is signalled in the SPS.

  - When it is specified that virtual boundary signaling is present in an PH, at least one virtual boundary is signalled in the PH.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| … |  |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_x[ i ] | u(13) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| … |  |
| } |  |

sps_virtual_boundaries_enabled_flag equal to 1 specifies that virtual boundary signalling may be present either in the SPS or in the picture headers referring to the SPS. sps_virtual_boundaries_enabled_flag equal to 0 specifies that  virtual boundary signalling is not present..

sps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the SPS. sps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the SPS. When there is one or more than one virtual boundaries signalled in the SPS, the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations.

sps_num_ver_virtual_boundaries specifies the number of sps_virtual_boundaries_pos_x[ i ] syntax elements that are present in the SPS. When sps_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

sps_virtual_boundaries_pos_x[ i ] is used to compute the value of VirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. sps_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

sps_num_hor_virtual_boundaries specifies the number of sps_virtual_boundaries_pos_y[ i ] syntax elements that are present in the SPS. When sps_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

When sps_virtual_boundaries_enabled_flag is equal to 1 and sps_virtual_boundaries_present_flag is equal to 1, the sum of sps_num_ver_virtual_boundaries and sps_num_hor_virtual_boundaries shall be greater than 0.

sps_virtual_boundaries_pos_y[ i ] is used to compute the value of VirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. sps_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| … |  |
| if( sps_virtual_boundaries_enabled_flag && !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

ph_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PH. ph_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PH. When there is one or more than one virtual boundaries signalled in the PH, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0.

The variable VirtualBoundariesDisabledFlag is derived as follows:

VirtualBoundariesDisabledFlag = sps_virtual_boundaries_present_flag  | |
	ph_virtual_boundaries_present_flag		(77)

VirtualBoundariesDisabledFlag = 0
if( sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesDisabledFlag = sps_virtual_boundaries_present_flag  | |
				ph_virtual_boundaries_present_flag

ph_num_ver_virtual_boundaries specifies the number of ph_virtual_boundaries_pos_x[ i ] syntax elements that are present in the PH. When ph_num_ver_virtual_boundaries is not present, it is inferred to be equal to 0.

The parameter VirtualBoundariesNumVer is derived as follows:

VirtualBoundariesNumVer = 0
if(sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesNumVer = sps_virtual_boundaries_present_flag ?
		sps_num_ver_virtual_boundaries : ph_num_ver_virtual_boundaries	(7-43)

ph_virtual_boundaries_pos_x[ i ] is used to compute the value of VirtualBoundariesPosX[ i ], which specifies the location of the i-th vertical virtual boundary in units of luma samples. ph_virtual_boundaries_pos_x[ i ] shall be in the range of 1 to Ceil( pic_width_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the vertical virtual boundary VirtualBoundariesPosX[ i ] is derived as follows:

VirtualBoundariesPosX[ i ] = 0
if(sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesPosX[ i ] = (sps_virtual_boundaries_present_flag ?
		sps_virtual_boundaries_pos_x[ i ] : ph_virtual_boundaries_pos_x[ i ]) * 8	(7-44)

The distance between any two vertical virtual boundaries shall be greater than or equal to CtbSizeY luma samples.

ph_num_hor_virtual_boundaries specifies the number of ph_virtual_boundaries_pos_y[ i ] syntax elements that are present in the PH. When ph_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

ph_num_hor_virtual_boundaries specifies the number of ph_virtual_boundaries_pos_y[ i ] syntax elements that are present in the PH. When ph_num_hor_virtual_boundaries is not present, it is inferred to be equal to 0.

The parameter VirtualBoundariesNumHor is derived as follows:

VirtualBoundariesNumHor = 0

if(sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesNumHor = sps_virtual_boundaries_present_flag ?
		sps_num_hor_virtual_boundaries : ph_num_hor_virtual_boundaries	(7-45)

-

-

ph_virtual_boundaries_pos_y[ i ] is used to compute the value of VirtualBoundariesPosY[ i ], which specifies the location of the i-th horizontal virtual boundary in units of luma samples. ph_virtual_boundaries_pos_y[ i ] shall be in the range of 1 to Ceil( pic_height_in_luma_samples ÷ 8 ) − 1, inclusive.

The location of the horizontal virtual boundary VirtualBoundariesPosY[ i ] is derived as follows:

VirtualBoundariesPosY[ i ] = 0
if(sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesPosY[ i ] = (sps_virtual_boundaries_present_flag ?
		sps_virtual_boundaries_pos_y[ i ] : ph_virtual_boundaries_pos_y[ i ]) * 8	(7-46)

The distance between any two horizontal virtual boundaries shall be greater than or equal to CtbSizeY luma samples

When sps_virtual_boundaries_ enabled_flag is equal to 1 and ph_virtual_boundaries_present_flag is equal to 1, the sum of ph_num_ver_virtual_boundaries and ph_num_hor_virtual_boundaries shall be greater than 0.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).