| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0247 |
| --- | --- |

| Title: | [AHG9]: Signalling the prediction weight table in the picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Seethal Paluri Hendry Jie Zhao Seung Hwan Kim 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | seethal.paluri@lge.com dr.hendry@lge.com jie.zhao@lge.com seunghwan3.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In current VVC specification text, when weighted prediction is enabled for a picture, the prediction weight table may be signalled in slice header. Considering that weighted prediction tool is practically applied/efficient at a picture level when there are fades or illumination variation within a video, this contribution proposes to move the signalling of prediction weight table from slice header to picture header. The modified design of the prediction weight table signalling is as follows:

- Unify pps_weighted_pred_flag and pps_weighted_bipred_flag in PPS into one flag and keep the name pps_weighted_pred_flag to specify whether weighted prediction may be applied to pictures referring to the PPS.

- When weighted prediction is enabled for a picture, prediction weight table for reference pictures in the reference picture lists (RPLs) of the picture may be present in the picture header associated with the picture.

- When weighted prediction is enabled, a flag in picture header (i.e., pic_weighted_pred_present_flag) is present to specify whether weighted prediction is applied to the picture or not. When pic_weighted_pred_present_flag is equal to 1, prediction weight table is present in the picture header.

- When weighted prediction is enabled for a picture, all slices of the picture have the same reference picture lists.

- In the prediction weight table signalling, explicitly signal the number of reference pictures to be weighted for L0 and L1. This is designed to remove dependency of the prediction weight table signalling to the number of active reference pictures that may be present in slice header.

# Introduction

It is asserted that weighted prediction tool may be beneficial in the following situation:

- Videos in which there is scene transition such as fade-in and fade-out.

- Videos in which there is sudden and / or gradual illumination change.

In both scenarios above where weighted prediction is mostly useful, it is asserted that weighted prediction would be used for the whole picture instead of specific local region within the picture(s) affected.

While the application of weighted prediction is asserted to be at picture level, yet, in the current VVC specification text prediction weight table, when present, is signalled in slice header (i.e,. if the PPS flag for the weighted prediction is enabled and the slice type is P or if the PPS flag for the weighted biprediction flag is enabled and the slice type is B). Such signalling is redundant and unnecessary. It would be more appropriate if prediction weight table is signalled in picture header rather than in each header of each slices of a picture.

One argument for keeping the signalling of prediction weight table in slice header is to anticipate the case that slices of the picture could have different reference picture lists. However, for the situation where weighted prediction tool is useful (i.e., when there is scene transition and illumination change), it would be common that all slices have the same reference picture lists.

# Proposal

We propose to signal prediction weight table in picture header rather than in slice header. The detail of the proposed signalling is as follows:

- Unify pps_weighted_pred_flag and pps_weighted_bipred_flag in PPS into one flag and keep the name pps_weighted_pred_flag to specify whether weighted prediction may be applied to pictures referring to the PPS.

- When weighted prediction is enabled for a picture, prediction weight table for reference pictures in the reference picture lists (RPLs) of the picture may be present in the picture header associated with the picture.

- When weighted prediction is enabled, a flag in picture header (i.e., pic_weighted_pred_present_flag) is present to specify whether weighted prediction is applied to the picture or not. When pic_weighted_pred_present_flag is equal to 1, prediction weight table is present in the picture header.

- When weighted prediction is enabled for a picture, all slices of the picture have the same reference picture lists.

- In the prediction weight table signalling, explicitly signal the number of reference pictures to be weighted for L0 and L1. This is designed to remove dependency of the prediction weight table signalling to the number of active reference pictures that may be present in slice header.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| … |  |
| pps_weighted_pred_flag | u(1) |
| pps_weighted_bipred_flag | u(1) |
| … |  |
| } |  |

…

pps_weighted_pred_flag equal to 0 specifies that weighted prediction is not applied to P or B slices referring to the PPS. pps_weighted_pred_flag equal to 1 specifies that weighted prediction is applied to P or B slices referring to the PPS. When sps_weighted_pred_flag is equal to 0, the value of pps_weighted_pred_flag shall be equal to 0.

pps_weighted_bipred_flag equal to 0 specifies that explicit weighted prediction is not applied to B slices referring to the PPS. pps_weighted_bipred_flag equal to 1 specifies that explicit weighted prediction is applied to B slices referring to the PPS. When sps_weighted_bipred_flag is equal to 0, the value of pps_weighted_bipred_flag shall be equal to 0.

…

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| … |  |
| } |  |
| if( pps_weighted_pred_flag && pic_rpl_present_flag ) { |  |
| pic_weighted_pred_flag | u(1) |
| if( pic_weighted_pred_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |
| } |  |

…

pic_weighted_pred_flag equal to 0 specifies that weighted prediction is not applied to P or B slices referring to the PH. pic_weighted_pred_flag equal to 1 specifies that weighted prediction is applied to P or B slices referring to the PH. When not present, the value of pic_weighted_pred_flag shall be equal to 0.

…

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_parameter_set_id | ue(v) |
| … |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| … |  |
| } |  |

| pred_weight_table( ) { | Descriptor |
| --- | --- |
| luma_log2_weight_denom | ue(v) |
| if( ChromaArrayType != 0 ) |  |
| delta_chroma_log2_weight_denom | se(v) |
| num_l0_weighted_ref_pics | ue(v) |
| for( i = 0; i < num_l0_weighted_ref_pics; i++ ) |  |
| luma_weight_l0_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumRefIdxActive[ 0 ]; i++ ) |  |
| chroma_weight_l0_flag[ i ] | u(1) |
| for( i = 0; i < NumRefIdxActive[ 0 ]; i++ ) { |  |
| if( luma_weight_l0_flag[ i ] ) { |  |
| delta_luma_weight_l0[ i ] | se(v) |
| luma_offset_l0[ i ] | se(v) |
| } |  |
| if( chroma_weight_l0_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l0[ i ][ j ] | se(v) |
| delta_chroma_offset_l0[ i ][ j ] | se(v) |
| } |  |
| } |  |
| num_l1_weighted_ref_pics | ue(v) |
| for( i = 0; i < num_l1_weighted_ref_pics; i++ ) |  |
| luma_weight_l1_flag[ i ] | u(1) |
| if( ChromaArrayType != 0 ) |  |
| for( i = 0; i < NumRefIdxActive[ 1 ]; i++ ) |  |
| chroma_weight_l1_flag[ i ] | u(1) |
| for( i = 0; i < NumRefIdxActive[ 1 ]; i++ ) { |  |
| if( luma_weight_l1_flag[ i ] ) { |  |
| delta_luma_weight_l1[ i ] | se(v) |
| luma_offset_l1[ i ] | se(v) |
| } |  |
| if( chroma_weight_l1_flag[ i ] ) |  |
| for( j = 0; j < 2; j++ ) { |  |
| delta_chroma_weight_l1[ i ][ j ] | se(v) |
| delta_chroma_offset_l1[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |

…

num_l0_weighted_ref_pics specifies the number of reference pictures in reference picture list 0 that are weighted. The value of num_l0_weighted_ref_pics shall ranges from 0 to MaxDecPicBuffMinus1 + 14, inclusive.

It is a requirement of bitstream conformance that when present, the value of num_l0_weighted_ref_pics shall not be less than the number of active reference pictures for L0 of any slices in the picture associated with the picture header.

num_l1_weighted_ref_pics specifies the number of reference pictures in reference picture list 1 that are weighted. The value of num_l1_weighted_ref_pics shall ranges from 0 to MaxDecPicBuffMinus1 + 14, inclusive.

It is a requirement of bitstream conformance that when present, the value of num_l1_weighted_ref_pics shall not be less than the number of active reference pictures for L1 of any slices in the picture associated with the picture header.

…

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).