| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0270 |
| --- | --- |

| Title: | AHG9: On Picture Header Modifications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to adopt the following changes to the picture header in VVC. The proposed changes may be considered independently.

- Condition gdr_pic_flag in picture header on gdr_enabled_flag in SPS such that gdr_pic_flag is not present and inferred to 0 when gdr_enabled_flag is equal to 0.

- Introduce an se(v) pic_qp_delta syntax element in the picture header and derive SliceQpY = 26 + init_qp_minus26 + pic_qp_delta + slice_qp_delta. The presence of pic_qp_delta is gated by a sps_pic_qp_delta_present_flag syntax element signaled in SPS.

- Replace pic_deblocking_filter_override_present_flag and pic_deblocking_filter_override_flag with a pic_deblocking_filter_override_idc syntax element specifying whether deblocking filter parameters are not overridden in picture header or slice header, deblocking filter parameters may be overridden in picture header or deblocking filter parameters may be overridden in slice header.

- Replace pic_sao_enabled_present_flag and pic_sao_luma_enabled_flag with a pic_sao_enabled_idc syntax element specifying whether SAO is disabled for the whole picture, SAO is enabled for the whole picture or SAO may be enabled per slice.

- Replace pic_alf_enabled_present_flag and pic_alf_enabled_flag with a pic_alf_enabled_idc syntax element specifying whether ALF is disabled for the whole picture, ALF is enabled for the whole picture or ALF may be enabled per slice.

Proposed specification text relative to JVET-Q0041-v2 is provided with the contribution.

# Introduction

At the 16th JVET meeting in Geneva the picture header was adopted to the VVC draft specification. This contribution identifies potential areas of improvement to the picture header and proposes solutions that may be adopted independently from each other. The following areas have been identified:

- gdr_pic_flag is always signaled in the picture header regardless if GDR pictures are enabled in the bitstream or not.

- When the picture header was added to VVC, a number of parameters were moved from the slice header to the picture header. slice_qp_delta was not moved since it is common to change QP per slice. However, the picture header provides a new level of flexibility for adapting the QP value.

- The pic_deblocking_filter_override_present_flag, pic_sao_enabled_present_flag and pic_alf_enabled_present_flag seem to add an unnecessary level of complexity to the picture header signaling. For instance, in order to decode tc and beta offsets for deblocking, four if-statements need to be checked as shown in the following syntax table.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

# Proposal

It is proposed to adopt the changes below to the VVC specification.

Proposed changes on top of the VVC draft (JVET-Q0041-v2) are marked in yellow with removed text in red strike-through.

## Condition gdr_pic_flag in picture header on gdr_enabled_flag in SPS

It is proposed to condition the gdr_pic_flag in the picture header on the gdr_enabled_flag such that no bit is signaled in the picture header when GDR pictures are disabled in the CVS. If this is adopted, it is also proposed to move the no_output_of_prior_pics_flag before the gdr_pic_flag to allow for fixed bit parsing of the no_output_of_prior_pics_flag.

### Proposed picture header syntax and semantics

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if ( gdr_enabled_flag ) { |  |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| } |  |
| … |  |
| } |  |

gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, the value of gdr_pic_flag is inferred to be 0.

## pic_qp_delta

It is proposed to introduce a pic_qp_delta syntax element in the picture header to allow for a more flexible adaption of the quantization parameter (QP). The SliceQpY is proposed to be derived as:

SliceQpY = 26 + init_qp_minus26 + pic_qp_delta + slice_qp_delta

### Proposed syntax and semantics:

| sequence_parameter_set_ rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| min_qp_prime_ts_minus4 | ue(v) |
| sps_pic_qp_delta_present_flag | u(1) |
| sps_weighted_pred_flag | u(1) |
| … |  |
| } |  |

sps_pic_qp_delta_present_flag equal to 1 specifies that pic_qp_delta is present in PHs referring to the SPS. sps_pic_qp_delta_present_flag equal to 0 specifies that pic_qp_delta is not present in PHs referring to the SPS.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( sps_pic_qp_delta_present_flag ) |  |
| pic_qp_delta | se(v) |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| … |  |
| } |  |

pic_qp_delta specifies the initial value of QpY to be used for the coding blocks in the picture and is used to derive  SliceQpY. pic_qp_delta shall be in the range of −63 to +63, inclusive. When not present, pic_qp_delta is inferred to be equal to 0.

| slice_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |  |
| slice_qp_delta | se(v) |  |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |  |
| slice_cb_qp_offset | se(v) |  |
| slice_cr_qp_offset | se(v) |  |
| if( sps_joint_cbcr_enabled_flag ) |  |  |
| slice_joint_cbcr_qp_offset | se(v) |  |
| } |  |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |  |
| cu_chroma_qp_offset_enabled_flag | u(1) |  |
| … |  |  |
| } |  |  |

slice_qp_delta specifies the initial value of QpY to be used for the coding blocks in the slice until modified by the value of CuQpDeltaVal in the coding unit layer. The initial value of the QpY quantization parameter for the slice, SliceQpY, is derived as follows:

SliceQpY = 26 + init_qp_minus26 + pic_qp_delta + slice_qp_delta	(146)

The value of SliceQpY shall be in the range of −QpBdOffset to +63, inclusive.

## Deblocking override in picture header

It is proposed to replace the flags pic_deblocking_filter_override_present_flag and pic_deblocking_filter_override_flag with a pic_deblocking_filter_override_idc codeword specifying whether deblocking filter parameters are not overridden in picture header or slice header, are overridden in picture header or may be overridden in slice header.

### Proposed syntax and semantics

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flagidc | u(12) |
| if( pic_deblocking_filter_override_flagidc == 1 ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( deblocking_filter_override_enabled_flag && !pic_deblocking_filter_override_flagidc == 2 ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| … |  |
| } |  |

pic_deblocking_filter_override_present_flag equal to 1 specifies that pic_deblocking_filter_override_flag is present in the PH. pic_deblocking_filter_override_present_flag equal to 0 specifies that pic_deblocking_filter_override_flag is not present in the PH. When pic_deblocking_filter_override_present_flag is not present, it is inferred to be equal to 0.

pic_deblocking_filter_override_flagidc equal to 0 specifies that deblocking parameters are not present in the PH or any slice headers associated with the PH. pic_deblocking_filter_override_idc equal to 1 specifies that deblocking parameters are present in the PH. pic_deblocking_filter_override_flagidc equal to 02 specifies that deblocking parameters are not present in the PH but may be present for one, or more, or all slices associated with the PH. When not present, the value of pic_pic_deblocking_filter_override_flagidc is inferred to be equal to 0. pic_deblocking_filter_override_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC.

## SAO enabled in picture header

It is proposed to replace pic_sao_enabled_present_flag and pic_sao_luma_enabled_flag with a pic_sao_enabled_idc codeword specifying whether SAO is disabled for the whole picture, SAO is enabled for the whole picture or SAO may be enabled per slice.

### Proposed syntax and semantics

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flagidc | u(12) |
| if(ChromaArrayType != 0 && (pic_sao_enabled_idc == 1 ¦¦ pic_sao_enabled_idc == 2)) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_sao_enabled_flag && !pic_sao_enabled_present_flagidc == 3 ) { |  |
| slice_sao_luma_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| slice_sao_chroma_flag | u(1) |
| } |  |
| … |  |
| } |  |

pic_sao_enabled_present_flag equal to 1 specifies that pic_sao_luma_flag and pic_sao_chroma_flag are present in the PH. pic_sao_enabled_present_flag equal to 0 specifies that pic_sao_luma_flag and pic_sao_chroma_flag are not present in the PH. When pic_sao_enabled_present_flag is not present, it is inferred to be equal to 0.

pic_sao_luma_flagenabled_idc equal to 0 specifies that SAO is not enabled for any slice associated with the PH. pic_sao_enabled_idc equal to 1 specifies that SAO is enabled for the luma component in all slices associated with the PH; pic_sao_enabled_idc equal to 2 specifies that SAO is disabled for the luma component in all slices associated with the PH. pic_sao_luma_flagenabled_idc equal to 03 specifies that SAO for the luma component may be disabled for one, or more, or all slices associated with the PH. When pic_sao_luma_flagenabled_idc is not present, it is inferred to be equal to 0.

pic_sao_chroma_enabled_flag equal to 1 specifies that SAO is enabled for the chroma component in all slices associated with the PH; pic_sao_chroma_enabled_flag equal to 0 specifies that SAO for chroma component may be disabled for one, or more, or all slices associated with the PH. When pic_sao_chroma_enabled_flag is not present, it is inferred to be equal to 0.

## ALF enabled in picture header

It is proposed to replace pic_alf_enabled_present_flag and pic_alf_enabled_flag with a pic_alf_enabled_idc codeword specifying whether ALF is disabled for the whole picture, ALF is enabled for the whole picture or if ALF may be enabled per slice.

### Proposed syntax and semantics

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flagidc | u(12) |
| if( pic_alf_enabled_ flagidc == 1 ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_flagidc == 2 ) { |  |  |
| slice_alf_enabled_flag | u(1) |  |
| if( slice_alf_enabled_flag ) { |  |  |
| slice_num_alf_aps_ids_luma | u(3) |  |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |  |
| slice_alf_aps_id_luma[ i ] | u(3) |  |
| if( ChromaArrayType != 0 ) |  |  |
| slice_alf_chroma_idc | u(2) |  |
| if( slice_alf_chroma_idc ) |  |  |
| slice_alf_aps_id_chroma | u(3) |  |
| } |  |  |
| } |  |  |
| … |  |  |
| } |  |  |

pic_alf_enabled_present_flag equal to 1 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, and pic_alf_aps_id_chroma are present in the PH. pic_alf_enabled_present_flag equal to 0 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, and pic_alf_aps_id_chroma are not present in the PH. When pic_alf_enabled_present_flag is not present, it is inferred to be equal to 0.

pic_alf_enabled_flagidc equal to 0 specifies that adaptive loop filter is not enabled for any slice associated with the PH. pic_alf_enabled_idc equal to 1 specifies that adaptive loop filter is enabled for all slices associated with the PH and may be applied to Y, Cb, or Cr colour component in the  slices. pic_alf_enabled_flagidc equal to 02 specifies that adaptive loop filter may be disabled for one, or more, or all slices associated with the PH. When not present, pic_alf_enabled_flagidc is inferred to be equal to 0. pic_alf_enabled_idc equal to 3 is reserved for future use by ITU-T | ISO/IEC.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).