| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0272-v3 |
| --- | --- |

| Title: | AHG9: Compact Region-Wise Packing SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Martin Pettersson Rickard Sjöberg Mitra Damghanian | Email: | martin.m.pettersson@ericsson.com rickard.sjoberg@ericsson.com mitra.damghanian@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to update the region-wise packing (RWP) SEI message in the draft VUI and SEI for coded video bitstreams specification to, according to the proponents, a more bit efficient representation. The proponents claim that the SEI functionality is kept the same and that only the size of the coded SEI message is affected. The proposed solution introduces three methods to compress the data sent in the RWP SEI message:

- Scaling method: Signal 16 bit scale factor value and 8 bit, instead of 16 or 32 bit, scaled values for the width, height, top and left offsets for each of the projected and packed regions in the RWP SEI message. To reproduce the width, height, top offset and left offset of each region, the scaled versions are multiplied with the scale factor. A scale_factor_flag is used to indicate if scaling is to be used or not.

- Copy method: Signal the region width and height only for the first region if all regions have the same size. A flag is used for each of the projected and packed picture to indicate whether all regions have the same size or not. If the flag is set, the width and height of the remaining regions are copied from the first region.

- Raster scan order method: Do not signal the top and left offsets of the regions if all regions are ordered in raster scan order. The offsets can be reconstructed on the receiver side using the picture width and the widths and heights of the regions. A flag is used for each of the projected and packed picture to indicate whether all regions are ordered in raster scan order.

These methods are claimed to be complementary to each other and each have its benefit in various scenarios.

The authors claim that in a scenario with 24 regions in the RWP, 4608 bits are spent on signaling the size and locations of the regions and in a scenario with 96 regions the corresponding bit cost is 18432 bits. It is further claimed that 75% to over 99% of the bits spent on coding the size and location of the regions in RWP can be saved using the proposed methods.

Proposed specification text relative to JVET-P2007-v3 is provided with the contribution.

In version 2 of this contribution a flag compress_regions_flag is added to not have to use up all 5 reserved zero bits. The enable flags for the proposed methods are only signaled if compress_regions_flag is equal to 1, otherwise the enable flags for the proposed methods are inferred to be 0. Presentation slides are also provided with this version.

In version 3 of this contribution an extension of the number of packed regions was added as suggested during the discussion in track A.

# Introduction

The region-wise packing SEI message was added to draft VUI and SEI for coded video bitstreams document (JVET-P2007) at the 16th JVET meeting in Geneva. The SEI message contains information of how to unpack and project each region in a packed picture onto a projected picture. The structure supports up to 255 regions for each of the projected picture and packed picture. The width, height, top offset and left offset of each region is coded with 32 bits for the projected picture and 16 bits for the packed picture. This adds up to a substantial number of bits when the number of regions is increased.

We believe that the RWP SEI message will be used a lot for VVC compared to how it has been used for HEVC so far and we therefore believe it is important to signal it in an efficient way.

The compact region-wise packing (CRWP) method as proposed in this contribution was proposed to be included in OMAF v2 at the 123rd MPEG meeting in Ljubljana. It was adopted to the OMAF v2 Technologies under Considerations (TuC) document, N18826, with a comment that the syntax may not be backwards compatible [with OMAF v1]. An OMAF v2 player would be able to use either an OMAF v1 RWP box or an OMAF v2 CRWP box, and an OMAF v1 player would be able to use either an RWP box or a CRWP box with compression disabled, but would not be able to use an OMAF v2 CRWP box with compression enabled.

However, since an OMAF v1 player would not be able to decode an OMAF v2 bitstream encoded with VVC, the backwards compatibility is anyway broken and it would be useful to use an updated more efficient version of the RWP message for VVC and other future codecs.

# Observations on regions

The size and locations of regions in a picture often tend to be aligned with a common devisor, e.g. the CTU size. The width, height, top offset and left offset of each region could then be expressed with a factor equal to the the common devisor.

Regions in the same picture often have the same size, which then makes it redundant to signal the region width and height for each region of a picture.

Moreover, the order of the regions in either one or both of the projected and packed picture may often be ordered in raster scan order. If the order of the regions are known, the top and left offsets could be derived instead of explicitly signaled.

# Proposal

It is proposed to update the region-wise packing SEI message in the draft VUI and SEI for coded video bitstreams document with functionality to compress the signaling of the SEI message. To differentiate it from the region-wise packing SEI message in HEVC it is proposed to call it compact region wise packing (CRWP) SEI message.

Three different methods, to be used individually or in combination, to more efficiently signal the RWP parameters in an CRWP SEI message are proposed:

- Scaling method: Signal a 16 bit scale factor value and 8 bit, instead of 16 or 32 bit, scaled values for the width, height, top and left offsets for each of the projected and packed regions in the RWP SEI message. To reproduce the width, height, top offset and left offset of each region, the scaled versions are multiplied with the scale factor. A scale_factor_flag is used to indicate if scaling is to be used or not.

- Copy method: Signal the region width and height only for the first region if all regions have the same size. A flag is used for each of the projected and packed picture to indicate whether all regions have the same size or not. If the flag is set, the width and height of the remaining regions are copied from the first region.

- Raster scan order method: Do not signal the top and left offsets of the regions if all regions are ordered in raster scan order. The offsets can be reconstructed on the receiver side using the picture width and the widths and heights of the regions. A flag is used for each of the projected and packed picture to indicate whether all regions are ordered in raster scan order.

These methods are complementary to each other and each have its benefit in various scenarios.

It is also proposed to enable extending the number of regions beyond 256, by deriving NumPackedRegions as ( num_packed_regions_msb << 8 ) + num_packed_regions, where num_packed_regions_msb is signaled in the SEI message if an extend_num_packed_regions_flag is equal to 1.

## Proposed syntax

Proposed syntax changes on top of the draft VUI and SEI for coded video bitstreams document (JVET-P2007-v3) are marked in yellow with removed text in red strike-through.

The proposed semantics can be found in the attached specification text word document.

| compact_regionwise_packing( payloadSize ) { | Descriptor |
| --- | --- |
| rwp_cancel_flag | u(1) |
| if( !rwp_cancel_flag ) { |  |
| rwp_persistence_flag | u(1) |
| constituent_picture_matching_flag | u(1) |
| compress_regions_flag | u(1) |
| rwp_reserved_zero_45bits | u(45) |
| if ( compress_regions_flag ) { |  |
| extend_num_packed_regions_flag | u(1) |
| scale_factor_flag | u(1) |
| proj_regions_copy_size_flag | u(1) |
| packed_regions_copy_size_flag | u(1) |
| proj_regions_raster_scan_flag | u(1) |
| packed_regions_raster_scan_flag | u(1) |
| rwp_reserved_zero_2bits | u(2) |
| if ( extend_num_packed_regions_flag ) |  |
| num_packed_regions_msb | u(8) |
| if ( scale_factor_flag ) { |  |
| proj_regions_scale_factor | u(16) |
| packed_regions_scale_factor | u(16) |
| } |  |
| } |  |
| num_packed_regions | u(8) |
| proj_picture_width | u(32) |
| proj_picture_height | u(32) |
| packed_picture_width | u(16) |
| packed_picture_height | u(16) |
| for( i = 0; i < NumPackedRegionsnum_packed_regions; i++ ) { |  |
| rwp_reserved_zero_4bits[ i ] | u(4) |
| rwp_transform_type[ i ] | u(3) |
| rwp_guard_band_flag[ i ] | u(1) |
| if ( i == 0 ¦¦ !proj_regions_copy_size_flag ) { |  |
| proj_region_width[ i ] | u(n32) |
| proj_region_height[ i ] | u(n32) |
| } |  |
| if ( !proj_regions_raster_scan_flag ) { |  |
| proj_region_top[ i ] | u(n32) |
| proj_region_left[ i ] | u(n32) |
| } |  |
| if ( i == 0 ¦¦ !packed_regions_copy_size_flag ) { |  |
| packed_region_width[ i ] | u(n16) |
| packed_region_height[ i ] | u(n16) |
| } |  |
| if ( !packed_regions_raster_scan_flag ) { |  |
| packed_region_top[ i ] | u(n16) |
| packed_region_left[ i ] | u(n16) |
| } |  |
| if( rwp_guard_band_flag[ i ] ) { |  |
| rwp_left_guard_band_width[ i ] | u(8) |
| rwp_right_guard_band_width[ i ] | u(8) |
| rwp_top_guard_band_height[ i ] | u(8) |
| rwp_bottom_guard_band_height[ i ] | u(8) |
| rwp_guard_band_not_used_for_pred_flag[ i ] | u(1) |
| for( j = 0; j < 4; j++ ) |  |
| rwp_guard_band_type[ i ][ j ] | u(3) |
| rwp_guard_band_reserved_zero_3bits[ i ] | u(3) |
| } |  |
| } |  |
| } |  |
| } |  |

# Bit cost examples

Some bit cost examples are provided to show the potential gains of the proposed methods. In a first example 24 equal size regions are coded with both the projected and packed picture in the region-wise packing in raster scan order. Table 1 shows a bit cost comparison between RWP and compact RWP for 24 equal size regions with regions in raster scan order in both projected and packed picture. The letters s, c, and r in the table stands for the proposed scaling, copy and raster scan order CRWP methods respectively.

|  | Video with 24 equal size regions in raster scan order in both projected and packed picture |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RWP | CRWP-s | CRWP-c | CRWP-r | CRWP-sr | CRWP-cr | CRWP-scr |
| bits proj_region_width | 768 | 192 | 32 | 768 | 192 | 32 | 8 |
| bits proj_region_height | 768 | 192 | 32 | 768 | 192 | 32 | 8 |
| bits packed_region_width | 384 | 192 | 16 | 384 | 192 | 16 | 8 |
| bits packed_region_height | 384 | 192 | 16 | 384 | 192 | 16 | 8 |
| bits proj_region_top | 768 | 192 | 768 | 0 | 0 | 0 | 0 |
| bits proj_region_left | 768 | 192 | 768 | 0 | 0 | 0 | 0 |
| bits packed_region_top | 384 | 192 | 384 | 0 | 0 | 0 | 0 |
| bits packed_region_left | 384 | 192 | 384 | 0 | 0 | 0 | 0 |
| bits compress regions flags |  | 8 | 8 | 8 | 8 | 8 | 8 |
| bits proj_regions_scale_factor |  | 16 |  |  | 16 |  | 16 |
| bits packed_regions_scale_factor |  | 16 |  |  | 16 |  | 16 |
| Tot bits | 4608 | 1576 | 2408 | 2312 | 808 | 104 | 72 |
| Tot bytes | 576 | 197 | 301 | 289 | 101 | 13 | 9 |

Table 1	Bit cost comparisons between RWP and compact RWP for 24 equal size regions with regions in raster scan order in both projected and packed picture

In a second example 96 equal size regions, such as for very high-resolution cube maps, are coded with both the projected and packed picture in the region-wise packing in raster scan order. Table 2 shows the bit cost comparison between RWP and compact RWP for 96 equal size regions with regions in raster scan order in both projected and packed picture.

|  | Video with 96 equal size regions in raster scan order in both projected and packed picture |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RWP |  | CRWP-s |  | CRWP-c |  | CRWP-r |  | CRWP-sr |  | CRWP-cr |  | CRWP-scr |
| bits proj_region_width | 3072 |  | 768 |  | 32 |  | 3072 |  | 768 |  | 32 |  | 8 |
| bits proj_region_height | 3072 |  | 768 |  | 32 |  | 3072 |  | 768 |  | 32 |  | 8 |
| bits packed_region_width | 1536 |  | 768 |  | 16 |  | 1536 |  | 768 |  | 16 |  | 8 |
| bits packed_region_height | 1536 |  | 768 |  | 16 |  | 1536 |  | 768 |  | 16 |  | 8 |
| bits proj_region_top | 3072 |  | 768 |  | 3072 |  | 0 |  | 0 |  | 0 |  | 0 |
| bits proj_region_left | 3072 |  | 768 |  | 3072 |  | 0 |  | 0 |  | 0 |  | 0 |
| bits packed_region_top | 1536 |  | 768 |  | 1536 |  | 0 |  | 0 |  | 0 |  | 0 |
| bits packed_region_left | 1536 |  | 768 |  | 1536 |  | 0 |  | 0 |  | 0 |  | 0 |
| bits compress regions flags |  | 8 |  | 8 |  | 8 |  | 8 |  | 8 |  | 8 |  |
| bits proj_regions_scale_factor |  |  | 16 |  |  |  |  |  | 16 |  |  |  | 16 |
| bits packed_regions_scale_factor |  |  | 16 |  |  |  |  |  | 16 |  |  |  | 16 |
| Tot bits | 18432 |  | 6184 |  | 9320 |  | 9224 |  | 3112 |  | 104 |  | 72 |
| Tot bytes | 2304 |  | 773 |  | 1172 |  | 1153 |  | 389 |  | 13 |  | 9 |

Table 2	Bit cost comparisons between RWP and compact RWP for 96 equal size regions with regions in raster scan order in both projected and packed picture

The third example is a frame packing example from OMAF. The example is illustrated in the figure below where regions from two different resolutions have been packed into one packed picture.

Figure 1 OMAF frame packing example.

Table 3 shows the bit cost comparison between RWP and compact RWP for the OMAF example having projected and packed picture with 24 non-equal size regions and the regions in raster scan order in the projected picture only.

|  | OMAF example: Projected and packed picture with 24 non-equal size regions, raster scan order in projected picture only |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | RWP | CRWP-s | CRWP-c | CRWP-r | CRWP-sr | CRWP-cr | CRWP-scr |
| bits proj_region_width | 768 | 192 | N/A | 768 | 192 | N/A | N/A |
| bits proj_region_height | 768 | 192 | N/A | 768 | 192 | N/A | N/A |
| bits packed_region_width | 384 | 192 | N/A | 384 | 192 | N/A | N/A |
| bits packed_region_height | 384 | 192 | N/A | 384 | 192 | N/A | N/A |
| bits proj_region_top | 768 | 192 | N/A | 0 | 0 | N/A | N/A |
| bits proj_region_left | 768 | 192 | N/A | 0 | 0 | N/A | N/A |
| bits packed_region_top | 384 | 192 | N/A | 384 | 192 | N/A | N/A |
| bits packed_region_left | 384 | 192 | N/A | 384 | 192 | N/A | N/A |
| bits compress regions flags |  | 8 | N/A | 8 | 8 | N/A | N/A |
| bits proj_regions_scale_factor |  | 16 |  |  | 16 |  | N/A |
| bits packed_regions_scale_factor |  | 16 |  |  | 16 |  | N/A |
| Tot bits | 4608 | 1576 | N/A | 3080 | 1192 | N/A | N/A |
| Tot bytes | 576 | 197 | N/A | 385 | 149 | N/A | N/A |

Table 3	cost comparisons between RWP and compact RWP for the OMAF example.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).