| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0285 |
| --- | --- |

| Title: | AHG9: On adaptation parameter set identification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution contains two proposals:

Proposal-1: Method for identification of APSs associated with a subpicture for bitstream extraction and merge, by signaling the list of associated subpicture IDs in each APS.

Proposal-2: Replacement of u(5) with ue(v), to code adaptation_parameter_set_id, for future extensibility.

Proposal-1: Identification of APSs associated with a subpicture

In the current VVC specification draft JVET-P2001 (editorially updated by JVET-Q0041), the subpicture partitioning is supported. When a bitstream consists of one or more motion-constrained subpictures (subpic_treated_as_pic_flag == 1), the subpictures can be extracted and independently decoded. When the subbitstream of the subpictures is extracted, APSs that are not referenced by the subpictures need to be removed. However, it is complicated to figure out which APSs are referenced by a specific sub-picture or which sub-pictures refers to a specific APS. After parsing APS IDs referenced by all PHs and SHs associated with each sub-picture, the APSs referenced by a specific sub-picture can be identified. Hence, it is computationally heavy to remove unreferenced APS from the sub-bitstream of sub-pictures. Because the bit size of APS is not negligible, it is desired to develop a practical method to identify the APSs associated with each sub-picture. Also, it deserves consideration how merge two different bitstreams into one, without conflict of APS ID values. When each sub-bitstream has its own APSs with a separate value space of APS ID, APS IDs values have to be completely rewritten in the merged bitstream to avoid a conflict.

In order to address the above issues, it is proposed to signal a list of sub-picture IDs aps_subpic_id[] in APS. The aps_subpic_id[] values, which specifies the subpicture identifier of the i-th subpicture that contains a slice that refers to the APS. An APS NAL unit with a list of aps_subpic_id[ i ] can be shared within the subpictures with a subpicture identifier equal to one of the aps_subpic_id[] values, when aps_subpic_id_list_present_flag is equal to 1. Thus, a slice in the subpicture with the subpicture identifier that is not listed in the APS shall not refer to the APS. It means that aps_subpic_id[] lists are used as additional identifiers to identify the referenced APSs. For example, when two APS NAL units have the same values of adaptation_parameter_set_id and aps_params_type, and each APS NAL unit has a different aps_subpic_id[] list from each other, the APS referenced by a specific subpicture is identified by the value of aps_subpic_id[].

The proposed APS identification method can be used for subbitstream extraction proposed by JVET-0283.

- Prosed spec text:

- Adaptation parameter set syntax

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| aps_subpic_id_list_present_flag | u(1) |
| if( aps_subpic_id_list_present_flag ) { |  |
| aps_num_subpics_minus1 | ue(v) |
| aps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= aps_num_subpic_id_minus1; i++ ) |  |
| aps_subpic_id[ i ] | u(v) |
| } |  |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 6.

Table 6 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3..7 | Reserved | Reserved |

NOTE 1 – Each type of APSs uses a separate value space for adaptation_parameter_set_id.

NOTE 2 – An APS NAL unit (with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type) can be shared across pictures, and different slices within a picture can refer to different ALF APSs.

NOTE 3 – A suffix APS NAL unit associated with a particular VCL NAL unit (this VCL NAL unit precedes the suffix APS NAL unit in decoding order) is not for use by the particular VCL NAL unit, but for use by VCL NAL units following the suffix APS NAL unit in decoding order.

aps_subpic_id_list_present_flag equal to 1 specifies that a slice contained in the subpicture with the subpicture identifier equal to aps_subpic_id[ i ] refers to the APS and a slice in the subpicture with the subpicture identifier that is not listed in the APS shall not refer to the APS. aps_subpic_id_list_present_flag equal to 0 specifies that the APS are shared across all subpictures, when subpictures are present. When sps_subpic_id_signalling_present_flag is equal to 0 and pps_subpic_id_signalling_present_flag is equal to 0, it is a requirement of bitstream conformance that aps_subpic_id_list_present_flag shall be equal to 0.

aps_num_subpics_minus1 plus 1 specifies the number of subpictures referring to the APS.

aps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element aps_subpic_id[ i ]. The value of aps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

aps_subpic_id[ i ] specifies the subpicture identifier of the i-th subpicture that contains a slice that refers to the APS. The length of the aps_subpic_id[ i ] syntax element is aps_subpic_id_len_minus1 + 1 bits.

NOTE 1 – An APS NAL unit with a list of aps_subpic_id[ i ] can be shared within the subpictures with a subpicture identifier equal to one of the aps_subpic_id[] values, when aps_subpic_id_list_present_flag is equal to 1.

NOTE 2 – When two APS NAL units have the same values of adaptation_parameter_set_id and aps_params_type, and each APS NAL unit has a different aps_subpic_id[] list from each other, the APS referenced by a specific subpicture is identified by the value of aps_subpic_id[].

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

Proposal-2: Extensible APS ID

An APS NAL unit with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type can be shared across pictures and layers. While the same value space of adaptation_parameter_set_id is shared across layers, adaptation_parameter_set_id is coded as u(5) in the current VVC spec. When aps_params_type is equal to ALF_APS or SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive. When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

The range of APS IDs may be related to the resource (e.g. memory) used for supporting the maximum number of APSs. It is not desired to increase the implementation burden of VVC V1. Considering futuristic immersive media use cases that potentially need tens layers, however, it is proposed to code adaptation_parameter_set_id as ue(v), without changes of the value ranges. The ue(v) coding is extensible for future use cases in the next versions of VVC.

- Prosed spec text:

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5)ue(v) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).