| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0298 |
| --- | --- |

| Title: | Reports on level mapping off versus level mapping on |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jungah Choi Sunmi Yoo Jin Heo Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 jungah.choi@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution investigates coding performance impact from the usage of level mapping. According to simulation results, disabling level mapping does not cause any coding loss for camera-captured contents, but some coding loss to sequences from Class F and TGM. In addition, when level mapping is disabled for the chroma blocks some coding gains are observed to sequences except TGM. This contribution proposes four methods regarding how to handle level mapping as follow.

- Method 1: disabling level mapping for all blocks

- Method 2: disabling level mapping for chroma blocks

- Method 3/4: SPS flag to indicate level mapping usage for all blocks/chroma blocks

Disabling level mapping:

Overall: 0.00% / -0.01% / 0.00% (AI), -0.02% / 0.00% / -0.04% (RA), -0.02% / -0.10% / -0.06% (LD)

Class F: 0.37% / 0.35% / 0.53% (AI), 0.24% / 0.43% / 0.32% (RA), -0.02% / 0.14% / 0.37% (LD)

TGM: 0.78% / 0.93% / 0.87% (AI), 0.51% / 0.66% / 0.68% (RA), 0.37% / 0.43% / 0.40% (LD)

Disabling level mapping (Low QP):

Overall: -0.03% / -0.05% / -0.06% (AI), -0.02% / -0.02% / -0.02% (RA), -0.01% / -0.01% / -0.01% (LD)

Class F: 0.11% / 0.17% / 0.18% (AI), 0.06% / 0.11% / 0.03% (RA), -0.06% / 0.06% / 0.05% (LD)

TGM: 1.48% / 1.26% / 1.34% (AI), 1.15% / 1.14% / 1.13% (RA), 0.93% / 1.01% / 1.02% (LD)

Disabling level mapping for chroma blocks:

Overall: 0.00% / -0.01% / 0.00% (AI), 0.00% / 0.03% / 0.01% (RA), -0.01% / -0.16% / -0.04% (LD)

Class F: 0.00% / 0.04% / 0.05% (AI), -0.04% / 0.02% / 0.03% (RA), -0.01% / -0.87% / 0.36% (LD)

TGM: 0.09% / 0.12% / 0.12% (AI), 0.03% / -0.01% / 0.05% (RA), -0.01% / -0.11% / -0.09% (LD)

Disabling level mapping for chroma blocks (Low QP):

Overall: -0.01% / -0.02% / -0.03% (AI), 0.00% / -0.01% / -0.02% (RA), 0.00% / -0.01% / 0.04% (LD)

Class F: 0.00% / -0.05% / -0.03% (AI), -0.05% / -0.01% / 0.03% (RA), 0.09% / 0.10% / -0.08% (LD)

TGM: 0.46% / 0.39% / 0.46% (AI), 0.22% / 0.26% / 0.33% (RA), 0.10% / 0.17% / 0.13% (LD)

Problem statements

In VVC7, a level mapping mechanism is applied to transform skip residual coding. The level mapping performs prediction of the abs value. Level mapping uses the top and left neighboring coefficient levels to predict the current coefficient level in order to reduce signaling cost. In this contribution, the performance impact of level mapping off versus level mapping on was studied to determine whether it is worthwhile to do the pixel-wise level prediction for the level mapping.

Proposed methods

Disabling level mapping for all blocks

In the proposed method 1, the level mapping is disabled. Table 1 shows the part of the transform skip residual coding syntax table of the proposed method.

Table 1. Part of syntax table of the proposed method 1

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| (…) |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && coded_sub_block_flag[ xS ][ yS ] ) ) |  |
| abs_remainder[ n ] | ae(v) |
| if( n <= lastScanPosPass2 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else if(n <= lastScanPosPass1 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else { /* bypass |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |
| if( abs_remainder[ n ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] ) : 0 |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] ) : 0 |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |
| AbsLevel[ xC ][ yC ] −= 1 |  |
| } |  |
| } |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |
| } |  |
| } |  |

Disabling level mapping for chroma blocks

In the proposed method 2, the level mapping is disabled for only chroma blocks. Table 2 shows the part of the transform skip residual coding syntax table of the proposed method.

Table 2. Part of syntax table of the proposed method 2

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| (…) |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && coded_sub_block_flag[ xS ][ yS ] ) ) |  |
| abs_remainder[ n ] | ae(v) |
| if( n <= lastScanPosPass2 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else if(n <= lastScanPosPass1 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else { /* bypass |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |
| if( abs_remainder[ n ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( cIdx == 0 && BdpcmFlag[ x0 ][ y0 ][ 0 ] = = 0 && n <= lastScanPosPass1 ) { |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] ) : 0 |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] ) : 0 |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |
| AbsLevel[ xC ][ yC ] −= 1 |  |
| } |  |
| } |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |
| } |  |
| } |  |

SPS level signaling to enable/disable level mapping

In this method, we introduce an SPS flag, sps_level_mapping_enable_flag, to enable/disable the level mapping for luma and chroma blocks. This flag is signaled when sps_transform_skip_enabled_flag is 1. Table 3 shows the part of the SPS syntax of the proposed method.

Table 3. Part of syntax table of the proposed method 3

| sps_transform_skip_enabled_flag | u(1) |
| --- | --- |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| sps_level_mapping_enabled_flag | u(1) |

The following is the semantic of the sps_level_mapping_enable_flag. Table 4 shows the part of the transform skip residual coding syntax of the proposed method.

sps_level_mapping_enabled_flag equal to 1 specifies that the level mapping is enabled. sps_level_mapping_enabled_flag equal to 0 specifies that the level mapping is disabled. When not present, the value of sps_level_mapping_enabled_flag is inferred to be equal to 0.

Table 4. Part of transform skip residual coding syntax table of the proposed method 3

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| (…) |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && coded_sub_block_flag[ xS ][ yS ] ) ) |  |
| abs_remainder[ n ] | ae(v) |
| if( n <= lastScanPosPass2 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else if(n <= lastScanPosPass1 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else { /* bypass |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |
| if( abs_remainder[ n ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if(sps_level_mapping_enabled_flag == 1 && BdpcmFlag[ x0 ][ y0 ][ cIdx ] = = 0 && n <= lastScanPosPass1 ) { |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] ) : 0 |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] ) : 0 |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |
| AbsLevel[ xC ][ yC ] −= 1 |  |
| } |  |
| } |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |
| } |  |
| } |  |

SPS level signaling to enable/disable chroma level mapping

In this method, we introduce an SPS flag, sps_level_mapping_chroma_enable_flag, to enable/disable the level mapping for chroma blocks. Table 5 shows the part of the SPS syntax of the proposed method.

Table 5. Syntax table of the proposed method 4

| sps_transform_skip_enabled_flag | u(1) |
| --- | --- |
| if( sps_transform_skip_enabled_flag ) |  |
| sps_bdpcm_enabled_flag | u(1) |
| sps_ level_mapping_chroma_enabled_flag | u(1) |

The following is the semantic of the sps_level_mapping_chroma_enable_flag. Table 6 shows the part of the transform skip residual coding syntax of the proposed method.

sps_level_mapping_chroma_enabled_flag equal to 1 specifies that the level mapping is enabled for chroma blocks. sps_level_mapping_chroma_enabled_flag equal to 0 specifies that the level mapping is disabled for chroma blocks. When not present, the value of sps_level_mapping_chroma_enabled_flag is inferred to be equal to 0.

Table 6. Part of transform skip residual coding syntax table of the proposed method 4

| residual_ts_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |
| --- | --- |
| (…) |  |
| /* remainder scan pass */ |  |
| for( n = 0; n <= numSbCoeff − 1; n++ ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( ( n <= lastScanPosPass2 && AbsLevelPass2[ xC ][ yC ] >= 10 ) ¦ ¦ ( n > lastScanPosPass2 && n <= lastScanPosPass1 && AbsLevelPass1[ xC ][ yC ] >= 2 ) ¦ ¦ ( n > lastScanPosPass1 && coded_sub_block_flag[ xS ][ yS ] ) ) |  |
| abs_remainder[ n ] | ae(v) |
| if( n <= lastScanPosPass2 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass2[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else if(n <= lastScanPosPass1 ) |  |
| AbsLevel[ xC ][ yC ] = AbsLevelPass1[ xC ][ yC ] + 2 * abs_remainder[ n ] |  |
| else { /* bypass |  |
| AbsLevel[ xC ][ yC ] = abs_remainder[ n ] |  |
| if( abs_remainder[ n ] ) |  |
| coeff_sign_flag[ n ] | ae(v) |
| } |  |
| if( (cIdx =! 0 && sps_level_mapping_chroma_enaled_flag == 1) && BdpcmFlag[ x0 ][ y0 ][ 0 ] = = 0 && n <= lastScanPosPass1 ) { |  |
| absLeftCoeff = xC > 0 ? AbsLevel[ xC − 1 ][ yC ] ) : 0 |  |
| absAboveCoeff = yC > 0 ? AbsLevel[ xC ][ yC − 1 ] ) : 0 |  |
| predCoeff = Max( absLeftCoeff, absAboveCoeff ) |  |
| if( AbsLevel[ xC ][ yC ] = = 1 && predCoeff > 0 ) |  |
| AbsLevel[ xC ][ yC ] = predCoeff |  |
| else if( AbsLevel[ xC ][ yC ] > 0 && AbsLevel[ xC ][ yC ] <= predCoeff ) |  |
| AbsLevel[ xC ][ yC ] −= 1 |  |
| } |  |
| } |  |
| TransCoeffLevel[ x0 ][ y0 ][ cIdx ][ xC ][ yC ] = ( 1 − 2 * coeff_sign_flag[ n ] ) * AbsLevel[ xC ][ yC ] |  |
| } |  |
| } |  |

Experimental results

Table 7 and Table 8 show the simulation results of the proposed method 1 and 2. These simulations are conducted using VTM-7.0 under common test conditions [1] with standard QPs and low QPs. The SPS level signaling methods (method 3 and method 4) are high level syntax changes only and practically can achieve identical performance as method 1 and method 2, respectively.

Table 7. Experimental results of method 1

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.04% | 0.00% | 101% | 100% | -0.03% | -0.03% | -0.04% | 99% | 100% |
| Class A2 | 0.00% | -0.02% | -0.03% | 101% | 100% | 0.00% | -0.01% | 0.00% | 99% | 101% |
| Class B | 0.00% | -0.01% | -0.07% | 100% | 99% | -0.02% | -0.03% | -0.04% | 100% | 98% |
| Class C | -0.02% | 0.05% | 0.14% | 100% | 100% | -0.03% | -0.06% | -0.04% | 100% | 100% |
| Class E | 0.01% | -0.05% | -0.03% | 101% | 99% | -0.09% | -0.16% | -0.21% | 100% | 99% |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 100% | -0.03% | -0.05% | -0.06% | 100% | 99% |
| Class D | -0.01% | 0.00% | -0.04% | 101% | 100% | 0.01% | -0.07% | -0.04% | 100% | 102% |
| Class F | 0.37% | 0.35% | 0.53% | 100% | 98% | 0.11% | 0.17% | 0.18% | 99% | 99% |
| TGM | 0.78% | 0.93% | 0.87% | 98% | 102% | 1.48% | 1.26% | 1.34% | 97% | 97% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.06% | 0.03% | 100% | 101% | -0.04% | -0.03% | -0.02% | 100% | 98% |
| Class A2 | -0.03% | -0.08% | 0.04% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.03% | -0.10% | 100% | 100% | -0.01% | 0.00% | -0.01% | 102% | 103% |
| Class C | -0.03% | -0.01% | -0.08% | 100% | 100% | -0.02% | -0.06% | -0.04% | 100% | 101% |
| Class E | - | - | - | - | - | - | - | - |  |  |
| Overall | -0.02% | 0.00% | -0.04% | 100% | 100% | -0.02% | -0.02% | -0.02% | 100% | 101% |
| Class D | -0.06% | -0.22% | -0.23% | 100% | 100% | -0.05% | -0.05% | -0.14% | 99% | 100% |
| Class F | 0.24% | 0.43% | 0.32% | 99% | 100% | 0.06% | 0.11% | 0.03% | 100% | 102% |
| TGM | 0.51% | 0.66% | 0.68% | 98% | 100% | 1.15% | 1.14% | 1.13% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - | - |
| Class B | -0.01% | -0.45% | -0.01% | 100% | 99% | -0.01% | -0.02% | 0.00% | 100% | 98% |
| Class C | -0.01% | -0.35% | -0.01% | 100% | 99% | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Class E | -0.06% | 0.83% | -0.21% | 100% | 99% | -0.01% | -0.01% | -0.01% | 100% | 101% |
| Overall | -0.02% | -0.10% | -0.06% | 100% | 99% | -0.01% | -0.01% | -0.01% | 100% | 100% |
| Class D | -0.04% | 0.45% | -0.27% | 100% | 99% | -0.08% | -0.04% | -0.15% | 99% | 99% |
| Class F | 0.14% | 0.24% | 1.07% | 100% | 99% | -0.06% | 0.06% | 0.05% | 99% | 98% |
| TGM | 0.37% | 0.43% | 0.40% | 99% | 100% | 0.93% | 1.01% | 1.02% | 99% | 100% |

Table 8. Experimental results of method 2

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.04% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.01% | -0.02% | 99% | 100% | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class B | 0.00% | -0.03% | -0.02% | 99% | 99% | -0.02% | -0.01% | -0.02% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.05% | 99% | 101% | -0.01% | -0.02% | -0.01% | 100% | 89% |
| Class E | 0.01% | -0.01% | -0.05% | 99% | 101% | -0.03% | -0.07% | -0.12% | 102% | 106% |
| Overall | 0.00% | -0.01% | 0.00% | 99% | 100% | -0.01% | -0.02% | -0.03% | 100% | 98% |
| Class D | 0.00% | -0.03% | -0.02% | 99% | 99% | 0.01% | -0.04% | -0.04% | 101% | 101% |
| Class F | 0.00% | 0.04% | 0.05% | 98% | 98% | 0.01% | -0.05% | -0.03% | 100% | 104% |
| TGM | 0.09% | 0.12% | 0.12% | 99% | 100% | 0.46% | 0.39% | 0.46% | 100% | 106% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 101% | 100% | 0.00% | 0.01% | -0.02% | 102% | 99% |
| Class A2 | -0.01% | -0.02% | 0.04% | 101% | 100% | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class B | 0.01% | 0.09% | 0.04% | 100% | 99% | 0.00% | 0.00% | -0.02% | 102% | 103% |
| Class C | 0.00% | 0.02% | -0.05% | 99% | 99% | -0.01% | -0.02% | -0.03% | 100% | 101% |
| Class E | - | - | - | - | - | - | - | - |  |  |
| Overall | 0.00% | 0.03% | 0.01% | 100% | 99% | 0.00% | -0.01% | -0.02% | 101% | 101% |
| Class D | -0.02% | -0.08% | -0.10% | 99% | 99% | 0.01% | 0.02% | -0.04% | 100% | 98% |
| Class F | -0.04% | 0.02% | 0.03% | 99% | 100% | -0.05% | -0.01% | 0.03% | 101% | 102% |
| TGM | 0.03% | -0.01% | 0.05% | 100% | 99% | 0.22% | 0.26% | 0.33% | 99% | 103% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with Standard QPs |  |  |  |  | Over VTM-7.0 with Low QPs |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | - | - | - | - | - | - | - | - | - | - |
| Class A2 | - | - | - | - | - | - | - | - | - | - |
| Class B | -0.02% | -0.28% | -0.18% | 98% | 99% | 0.01% | 0.00% | 0.01% | 98% | 97% |
| Class C | 0.05% | -0.35% | 0.07% | 98% | 96% | 0.00% | 0.00% | 0.02% | 99% | 97% |
| Class E | -0.06% | 0.27% | 0.03% | 99% | 98% | 0.00% | -0.04% | 0.11% | 97% | 94% |
| Overall | -0.01% | -0.16% | -0.04% | 98% | 98% | 0.00% | -0.01% | 0.04% | 98% | 97% |
| Class D | -0.01% | 0.08% | 0.39% | 100% | 100% | 0.00% | 0.00% | -0.12% | 100% | 99% |
| Class F | -0.01% | -0.87% | 0.36% | 99% | 97% | 0.09% | 0.10% | -0.08% | 99% | 97% |
| TGM | -0.01% | -0.11% | -0.09% | 97% | 96% | 0.10% | 0.17% | 0.13% | 99% | 99% |

Conclusion

In order to reduce the complexity of transform skip residual coding, this contribution proposes to disable level mapping for (i) all (ii) chroma blocks. It also proposes to add an SPS flag to enable/disable level mapping for (iii) all (iv) chroma blocks. It is observed that removing level mapping affects only Class F and TGM, and does not cause any loss in camera-captured contents. Rather, BD-rate decreases are found in most camera-captured sequences.

References

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N1010, Geneva, CH, 19–27 March 2019.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).