| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0389 |
| --- | --- |

| Title: | Non-CE4: SbTMVP harmonization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Christopher Hollmann, Ruoyang Yu Torshamnsgatan 23 16480 Stockholm Sweden | Email: | {du.liu, christopher.hollmann, ruoyang.yu} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-7.0, the SbTMVP uses the motion information of a neighboring candidate block to derive the motion information of sub-CUs. In the current algorithm, the spatial neigboring block A1 is used to derive the motion information. On the other hand, the order of the spatial neighboring candidates is B1, A1, B0, A0, and B2. The primary candidate of the spatial merge mode is not the primary candidate for SbTMVP. To align with the spatial neighboring candidate list, this contribution proposes to use the first candidate B1 from the spatial candidate list in SbTMVP to derive the motion information. The proposed method was reportedly tested under VTM-7.0 random access (RA) and low delay B (LDB) common test configuration. The BD-rate impact is reported to be 0.01% for RA and -0.01% for LDB.

# Proposal

Subblock-based Temporal Motion Vector Prediction (SbTMVP) divides a current coding unit (CU) into multiple 8x8 sub-CUs and uses the motion field in the collocated picture to improve motion vector prediction and merge mode. The current SbTMVP uses the motion information of a neighboring candidate block to derive the motion information of sub-CUs. The position of candidates is shown in Figure 1. In the current algorithm the candidate block A1 is used to derive the motion information.

Figure 1. Spatial merge candidates

The order of candidates in the spatial candidate derivation is B1, A1, B0, A0 and B2 [1]. This was changed due to the adoption of JVET-P0325 [2] at the last meeting. This indicates that the primary candidate of the spatial merge mode is not the primary candidate for SbTMVP.

This contribution proposes to use B1 as the spatial neighbor to derive the motion information for sub-CUs in SbTMVP to align with the spatial neighboring candidate list. The result is reported to have negligible impact on BD-rate performance.

Also, it is noted that the algorithm description in JVET-P2002 assigns different names in Figure 21 and Figure 29 to the neighboring blocks. It is recommended to produce an updated version of the algorithm description using the same names for the same blocks as the specification.

# Simulation results

The proposed method was implemented on top of VTM-7.0. The simulations were run using the VTM common test configurations [3].

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.06% | 0.04% | 99% | 103% |
| Class A2 | 0.00% | 0.00% | 0.02% | 99% | 100% |
| Class B | 0.00% | 0.05% | 0.02% | 99% | 101% |
| Class C | 0.01% | 0.02% | 0.06% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.03% | 0.03% | 99% | 101% |
| Class D | -0.02% | -0.03% | -0.04% | 99% | 98% |
| Class F | 0.01% | -0.04% | 0.00% | 103% | 98% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.15% | 0.10% | 97% | 93% |
| Class C | -0.03% | -0.14% | -0.06% | 98% | 102% |
| Class E | 0.01% | 0.16% | -0.72% | 98% | 101% |
| Overall | -0.01% | -0.07% | -0.16% | 98% | 98% |
| Class D | -0.07% | 0.78% | 0.25% | 99% | 100% |
| Class F | 0.04% | -0.37% | -0.12% | 97% | 99% |

# Conclusions

To align with the spatial neighboring candidate list, this contribution proposes to use the first candidate B1 in the list as the spatial neighbor to derive the motion information in SbTMVP. The result is reported to have negligible impact on BD-rate performance with 0.01% for RA and -0.01% for LDB.

We recommend adopting the proposed method into the next VTM software and VVC working draft.

# References

- J. Chen et al. Algorithm description for Versatile Video Coding and Test Model 7 (VTM 7). JVET-P2002, Geneva, Switzerland, October 2019

- Z. Wang et al. Non-CE4: Construction of spatial merge candidates. JVET-P0325, Geneva, Switzerland, October 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring. JVET common test conditions and software reference configurations for SDR video. JVET-N1010, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec text modification

The change of Spec text is as follows:

8.5.5.3 Derivation process for subblock-based temporal merging candidates Inputs to this process are: - a luma location ( xCb, yCb ) of the top-left sample of the current luma coding block relative to the top-left luma sample of the current picture, - a variable cbWidth specifying the width of the current coding block in luma samples, - a variable cbHeight specifying the height of the current coding block in luma samples. - the availability flag availableFlagBA1 of the neighbouring coding unit, - the reference index refIdxLXBA1 of the neighbouring coding unit with X being 0 or 1, - the prediction list utilization flag predFlagLXBA1 of the neighbouring coding unit with X being 0 or 1, - the motion vector in 1/16 fractional-sample accuracy mvLXBA1 of the neighbouring coding unit with X being 0 or 1. Outputs of this process are: - the availability flag availableFlagSbCol, - the number of luma coding subblocks in horizontal direction numSbX and in vertical direction numSbY, - the reference indices refIdxL0SbCol and refIdxL1SbCol, - the luma motion vectors in 1/16 fractional-sample accuracy mvL0SbCol[ xSbIdx ][ ySbIdx ] and mvL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1, - the prediction list utilization flags predFlagL0SbCol[ xSbIdx ][ ySbIdx ] and predFlagL1SbCol[ xSbIdx ][ ySbIdx ] with xSbIdx = 0..numSbX − 1, ySbIdx = 0 .. numSbY − 1. The availability flag availableFlagSbCol is derived as follows. - If one or more of the following conditions is true, availableFlagSbCol is set equal to 0. - pic_temporal_mvp_enabled_flag is equal to 0. - sps_sbtmvp_enabled_flag is equal to 0. - cbWidth is less than 8. - cbHeight is less than 8. - Otherwise, the following ordered steps apply: - The location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block and the location ( xCtr, yCtr ) of the below-right center sample of the current luma coding block are derived as follows: xCtb = ( xCb >> CtuLog2Size ) << CtuLog2Size (8-529) yCtb = ( yCb >> CtuLog2Size ) << CtuLog2Size (8-530) xCtr = xCb + ( cbWidth / 2 ) (8-531) yCtr = yCb + ( cbHeight / 2 ) (8-532) - The luma location ( xColCtrCb, yColCtrCb ) is set equal to the top-left sample of the collocated luma coding block covering the location given by ( xCtr, yCtr ) inside ColPic relative to the top-left luma sample of the collocated picture specified by ColPic. - The derivation process for subblock-based temporal merging base motion data as specified in clause 8.5.5.4 is invoked with the location ( xCtb, yCtb ), the location ( xColCtrCb, yColCtrCb ), the availability flag availableFlagBA1, and the prediction list utilization flag predFlagLXBA1, and the reference index refIdxLXBA1, and the motion vector mvLXBA1, with X being 0 and 1 as inputs and the motion vectors ctrMvLX, and the prediction list utilization flags ctrPredFlagLX of the collocated block, with X being 0 and 1, and the temporal motion vector tempMv as outputs. - The variable availableFlagSbCol is derived as follows: - If both ctrPredFlagL0 and ctrPredFlagL1 are equal to 0, availableFlagSbCol is set equal to 0. - Otherwise, availableFlagSbCol is set equal to 1. When availableFlagSbCol is equal to 1, the following applies: - The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows: numSbX = cbWidth >> 3 (8-533) numSbY = cbHeight >> 3 (8-534) sbWidth = cbWidth / numSbX (8-535) sbHeight = cbHeight / numSbY (8-536) refIdxLXSbCol = 0 (8-537) - For xSbIdx = 0..numSbX − 1 and ySbIdx = 0 .. numSbY − 1, the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows: - The luma location ( xSb, ySb ) specifying the below-right center sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows: xSb = xCb + xSbIdx * sbWidth + sbWidth / 2 (8-538) ySb = yCb + ySbIdx * sbHeight + sbHeight / 2 (8-539) - The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows. - The following applies: yColSb = Clip3( yCtb, Min( pic_height_in_luma_samples − 1, yCtb + ( 1 << CtbLog2SizeY ) − 1 ), (8-540) ySb + tempMv[1] ) - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies: xColSb = Clip3( xCtb, Min( SubPicRightBoundaryPos, xCtb + ( 1 << CtbLog2SizeY ) + 3 ), (8-541) xSb + tempMv[0] ) - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies: xColSb = Clip3( xCtb, Min( pic_width_in_luma_samples − 1, xCtb + ( 1 << CtbLog2SizeY ) + 3 ), (8-542) xSb + tempMv[0] ) - The variable currCb specifies the luma coding block covering the current coding subblock inside the current picture. - The variable colCb specifies the luma coding block covering the modified location given by ( ( xColSb >> 3 ) << 3, ( yColSb >> 3 ) << 3 ) inside the ColPic. - The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic. - The derivation process for collocated motion vectors as specified in clause 8.5.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxL0 set equal to 0 and sbFlag set equal to 1 as inputs and the output being assigned to the motion vector of the subblock mvL0SbCol[ xSbIdx ][ ySbIdx ] and availableFlagL0SbCol. - The derivation process for collocated motion vectors as specified in clause 8.5.2.12 is invoked with currCb, colCb, ( xColCb, yColCb ), refIdxL1 set equal to 0 and sbFlag set equal to 1 as inputs and the output being assigned to the motion vector of the subblock mvL1SbCol[ xSbIdx ][ ySbIdx ] and availableFlagL1SbCol. - When availableFlagL0SbCol and availableFlagL1SbCol are both equal to 0, the following applies for X being 0 and 1: mvLXSbCol[ xSbIdx ][ ySbIdx ] = ctrMvLX (8-543) predFlagLXSbCol[ xSbIdx ][ ySbIdx ] = ctrPredFlagLX (8-544) 8.5.5.4 Derivation process for subblock-based temporal merging base motion data Inputs to this process are: - the location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block, - the location ( xColCtrCb, yColCtrCb ) of the top-left sample of the collocated luma coding block that covers the below-right center sample. - the availability flag availableFlagBA1 of the neighbouring coding unit, - the reference index refIdxLXBA1 of the neighbouring coding unit, - the prediction list utilization flag predFlagLXBA1 of the neighbouring coding unit, - the motion vector in 1/16 fractional-sample accuracy mvLXBA1 of the neighbouring coding unit. Outputs of this process are: - the motion vectors ctrMvL0 and ctrMvL1, - the prediction list utilization flags ctrPredFlagL0 and ctrPredFlagL1, - the temporal motion vector tempMv. The variable tempMv is set as follows: tempMv[ 0 ] = 0 (8-545) tempMv[ 1 ] = 0 (8-546) The variable currPic specifies the current picture. When availableFlagBA1 is equal to TRUE, the following applies: - If all of the following conditions are true, tempMv is set equal to mvL0BA1: - predFlagL0BA1 is equal to 1, - DiffPicOrderCnt(ColPic, RefPicList[ 0 ][refIdxL0BA1]) is equal to 0, - Otherwise, if all of the following conditions are true, tempMv is set equal to mvL1BA1: - slice_type is equal to B, - predFlagL1BA1 is equal to 1, - DiffPicOrderCnt(ColPic, RefPicList[ 1 ][refIdxL1BA1]) is equal to 0. The rounding process for motion vectors as specified in clause 8.5.2.14 is invoked with mvX set equal to tempMv, rightShift set equal to 4, and leftShift set equal to 0 as inputs and the rounded tempMv as output. The location ( xColCb, yColCb ) of the collocated block inside ColPic is derived as follows. - The following applies: yColCb = Clip3( yCtb, Min( pic_height_in_luma_samples − 1, yCtb + ( 1 << CtbLog2SizeY ) − 1 ), (8-547) yColCtrCb + tempMv[1] ) - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies: xColCb = Clip3( xCtb, Min( SubPicRightBoundaryPos, xCtb + ( 1 << CtbLog2SizeY ) + 3 ), (8-548) xColCtrCb + tempMv[0] ) - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to o, the following applies: xColCb = Clip3( xCtb, Min( pic_width_in_luma_samples − 1, xCtb + ( 1 << CtbLog2SizeY ) + 3 ), (8-549) xColCtrCb + tempMv[0] ) The array colPredMode is set equal to the prediction mode array CuPredMode[ 0 ] of the collocated picture specified by ColPic. The motion vectors ctrMvL0 and ctrMvL1, and the prediction list utilization flags ctrPredFlagL0 and ctrPredFlagL1 are derived as follows: - If colPredMode[xColCb][yColCb] is equal to MODE_INTER, the following applies: - The variable currCb specifies the luma coding block covering ( xCtrCb ,yCtrCb ) inside the current picture. - The variable colCb specifies the luma coding block covering the modified location given by ( ( xColCb >> 3 ) << 3, ( yColCb >> 3 ) << 3 ) inside the ColPic. - The luma location ( xColCb, yColCb ) is set equal to the top-left sample of the collocated luma coding block specified by colCb relative to the top-left luma sample of the collocated picture specified by ColPic. - The derivation process for collocated motion vectors specified in clause 8.5.2.12 is invoked with currCb, colCb, (xColCb, yColCb), refIdxL0 set equal to 0, and sbFlag set equal to 1 as inputs and the output being assigned to ctrMvL0 and ctrPredFlagL0. - The derivation process for collocated motion vectors specified in clause 8.5.2.12 is invoked with currCb, colCb, (xColCb, yColCb), refIdxL1 set equal to 0, and sbFlag set equal to 1 as inputs and the output being assigned to ctrMvL1 and ctrPredFlagL1. - Otherwise, the following applies: ctrPredFlagL0 = 0 (8-550) ctrPredFlagL1 = 0 (8-551)