| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0390 |
| --- | --- |

| Title: | Non-CE4: SbTMVP simplification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Christopher Hollmann, Ruoyang Yu Torshamnsgatan 23 16480 Stockholm Sweden | Email: | {du.liu, christopher.hollmann, ruoyang.yu} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In VTM-7.0, the SbTMVP uses the motion information of a neighboring candidate block to derive the motion information of sub-CUs. Currently, the 8-tap interpolation filter is used in SbTMVP, whereas the 6-tap filter is used for 4x4 CUs and the subblocks of affine CUs. This contribution proposes to use the 6-tap interpolation filter in SbTMVP. This aligns all the subblocks to use the 6-tap interpolation filter and reduces the memory bandwidth of SbTMVP by 25%. The BD-rate impact is reported to be 0.02% for RA and 0.04% for LDB.

# Proposal

In VTM-7.0, the Subblock-based Temporary Motion Vector Prediction (SbTMVP) uses the motion information of a neighboring candidate block to derive the motion information of sub-CUs. Currently, the 6-tap filter is used for 4x4 CUs and the subblocks of affine CUs and the 8-tap interpolation filter is used in SbTMVP. This contribution proposes to use the 6-tap interpolation filter in SbTMVP. This aligns all the subblocks to use the 6-tap interpolation filter and reduces the memory bandwidth of SbTMVP.

SbTMVP divides a CU of size WxH into multiple 8x8 blocks. For the 8-tap interpolation filter with uni-prediction, extra samples are fetched to compute the interpolation. The memory bandwidth (i.e., the required samples to be fetched) is (8 + 8 – 1) * (8 + 8 – 1) / (8 * 8) ≈ 3.52 per sample, and the memory bandwidth for a CU is 3.52 * W * H. If the 6-tap interpolation filter is used, the memory bandwidth per sample becomes (8 + 6 – 1) * (8 + 6 – 1) / (8 * 8) ≈ 2.64, and the memory bandwidth for a CU is 2.64 * W * H, which reduces the memory bandwidth to 2.64 / 3.52 = 75%, i.e., a reduction of 25%. In the worst case when a CU has a size of 128x128, the memory bandwidth requirement is given in Table 1.

Table 1. The memory bandwidth for a 128x128 CU.

| Filter & prediction | Memory bandwith per sample | Memory bandwidth per CU |
| --- | --- | --- |
| 8-tap uni | 3.52 | 57,600 |
| 6-tap uni | 2.64 | 43,264 |
| 8-tap bi | 7.03 | 115,200 |
| 6-tap bi | 5.28 | 86,528 |

# Results

The proposed method was implemented on top VTM-7.0 [1]. The simulations were run using the VTM common test configurations [2].

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.02% | -0.01% | 102% | 108% |
| Class A2 | -0.01% | -0.02% | 0.04% | 100% | 100% |
| Class B | 0.02% | 0.03% | 0.01% | 100% | 98% |
| Class C | 0.05% | 0.01% | 0.06% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.01% | 0.02% | 100% | 101% |
| Class D | 0.05% | 0.00% | 0.12% | 99% | 97% |
| Class F | 0.00% | 0.00% | -0.01% | 101% | 96% |

|  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.17% | -0.09% | 100% | 98% |
| Class C | 0.02% | -0.15% | 0.00% | 103% | 109% |
| Class E | 0.10% | 0.48% | -0.29% | 101% | 104% |
| Overall | 0.04% | 0.00% | -0.11% | 101% | 103% |
| Class D | 0.02% | 0.84% | 0.17% | 102% | 105% |
| Class F | 0.09% | -0.51% | 0.06% | 97% | 100% |

# Conclusions

This contribution proposes to use the 6-tap interpolation filter in SbTMVP. This aligns all the subblocks to use the 6-tap interpolation filter and reduces the memory bandwidth of SbTMVP by 25%. The BD-rate impact is reported to be 0.02% for RA and 0.04% for LDB.

We recommend adopting the proposed method into the next VTM software and VVC working draft.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang. "Versatile Video Coding (Draft 7)", Output document approved by JVET, document number JVET-P2001.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring. JVET common test conditions and software reference configurations for SDR video. JVET-N1010, Geneva, Switzerland, March 2019.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Spec changes

Luma sample interpolation filtering process Inputs to this process are: - a luma location in full-sample units ( xIntL, yIntL ), - a luma location in fractional-sample units ( xFracL, yFracL ), - a luma location in full-sample units ( xSbIntL, ySbIntL ) specifying the top-left sample of the bounding block for reference sample padding relative to the top-left luma sample of the reference picture, - the luma reference sample array refPicLXL, - the half sample interpolation filter index hpelIfIdx, - a variable sbWidth specifying the width of the current subblock, - a variable sbHeight specifying the height of the current subblock, - a variable merge_subblock_flag[ xCb ][ yCb ] specifying if the current block is in subblock merge mode, - a fixedpoint representation of the horizontal scaling factor scalingRatio[ 0 ], - a fixedpoint representation of the vertical scaling factor scalingRatio[ 1 ], - a luma location ( xSb, ySb ) specifying the top-left sample of the current subblock relative to the top-left luma sample of the current picture. Output of this process is a predicted luma sample value predSampleLXL The variables shift1, shift2 and shift3 are derived as follows: - The variable shift1 is set equal to Min( 4, BitDepth − 8 ), the variable shift2 is set equal to 6 and the variable shift3 is set equal to Max( 2, 14 − BitDepth ). - The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples. The horizontal luma interpolation filter coefficients fLH[ p ] for each 1/16 fractional sample position p equal to xFracL or yFracL are derived as follows: - If one or more of the following conditions is true, MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLH[ p ] are specified in Table 8-14. - MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, - merge_subblock_flag[ xCb ][ yCb ] is equal to 1, - Otherwise, if scalingRatio[ 0 ] is greater than 28672, luma interpolation filter coefficients fLH[ p ] are specified in Table 8-12. - Otherwise, if scalingRatio[ 0 ] is greater than 20480, luma interpolation filter coefficients fLH[ p ] are specified in Table 8-13. - Otherwise, the luma interpolation filter coefficients fLH[ p ] are specified in Table 8-11 depending on hpelIfIdx. The vertical luma interpolation filter coefficients fLV[ p ] for each 1/16 fractional sample position p equal to yFracL are derived as follows: - If one or more of the following conditions is true, MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLV[ p ] are specified in Table 8-14. - MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, - merge_subblock_flag[ xCb ][ yCb ] is equal to 1, - Otherwise, if scalingRatio[ 1 ] is greater than 28672, luma interpolation filter coefficients fLV[ p ] are specified in Table 8-12. - Otherwise, if scalingRatio[ 1 ] is greater than 20480, luma interpolation filter coefficients fLV[ p ] are specified in Table 8-13. - Otherwise, the luma interpolation filter coefficients fLV[ p ] are specified in Table 8-11 depending on hpelIfIdx. The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7: – If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies: xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i − 3 ) (8-760) yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 ) (8-761) – Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies: xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ? ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) : (8-762) xIntL + i − 3 ) yInti = Clip3( 0, picH − 1, yIntL + i − 3 ) (8-763) The luma locations in full-sample units are further modified as follows for i = 0..7: xInti = Clip3( xSbIntL − 3, xSbIntL + sbWidth + 4, xInti ) (8-764) yInti = Clip3( ySbIntL − 3, ySbIntL + sbHeight + 4, yInti ) (8-765) The predicted luma sample value predSampleLXL is derived as follows: - If both xFracLand yFracL are equal to 0, and both scalingRatio[ 0 ] and scalingRatio[ 1 ] are less than 20481, the value of predSampleLXL is derived as follows: predSampleLXL = refPicLXL[ xInt3 ][ yInt3 ] << shift3 (8-766) - Otherwise, if yFracL is equal to 0 and scalingRatio[ 1 ] is less than 20481, the value of predSampleLXL is derived as follows: predSampleLXL = i=07fLH[ xFracL ][ i ] * refPicLXL[ xInti ][ yInt3 ] >> shift1 (8-767) - Otherwise, if xFracL is equal to 0 and scalingRatio[ 0 ] is less than 20481, the value of predSampleLXL is derived as follows: predSampleLXL = i=07fLV[ yFracL ][ i ] * refPicLXL[ xInt3 ][ yInti ] >> shift1 (8-768) - Otherwise, the value of predSampleLXL is derived as follows: - The sample array temp[ n ] with n = 0..7, is derived as follows: temp[ n ] = i=07fLH[ xFracL ][ i ] * refPicLXL[ xInti ][ yIntn ] >> shift1 (8-769) - The predicted luma sample value predSampleLXL is derived as follows: predSampleLXL = i=07fLV[ yFracL ][ i ] * temp[ i ] >> shift2 (8-770) … Table 8-14 – Specification of the luma interpolation filter coefficients fL[ p ] for each 1/16 fractional sample position p for affine motion mode. Fractional sample position p; interpolation filter coefficients. fL[ p ][ 0 ]; fL[ p ][ 1 ]; fL[ p ][ 2 ]; fL[ p ][ 3 ]; fL[ p ][ 4 ]; fL[ p ][ 5 ]; fL[ p ][ 6 ]; fL[ p ][ 7 ]. 1; 0; 1; −3; 63; 4; −2; 1; 0. 2; 0; 1; −5; 62; 8; −3; 1; 0. 3; 0; 2; −8; 60; 13; −4; 1; 0. 4; 0; 3; −10; 58; 17; −5; 1; 0. 5; 0; 3; −11; 52; 26; −8; 2; 0. 6; 0; 2; −9; 47; 31; −10; 3; 0. 7; 0; 3; −11; 45; 34; −10; 3; 0. 8; 0; 3; −11; 40; 40; −11; 3; 0. 9; 0; 3; −10; 34; 45; −11; 3; 0. 10; 0; 3; −10; 31; 47; −9; 2; 0. 11; 0; 2; −8; 26; 52; −11; 3; 0. 12; 0; 1; −5; 17; 58; −10; 3; 0. 13; 0; 1; −4; 13; 60; −8; 2; 0. 14; 0; 1; −3; 8; 62; −5; 1; 0. 15; 0; 1; −2; 4; 63; −3; 1; 0.