| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0395 |
| --- | --- |

| Title: | AHG12: On subpicture level information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution addresses the subpicture level information SEI message and consists of three parts:

- CpbSize: The derivation of SubPicCpbSizeVcl and SubPicCpbSizeNal in the subpicture level information SEI message is based on MaxCPB of an indicated level as specified in clause A.4.2. This contribution asserts that when the bitstream contains an exact CpbSize indication, it should be used for the derivation instead MaxCPB. Otherwise, the inference rule for ref_level_fraction_minus1[ i ][ j ] (infer to be equal to area ratio) will likely result in very large values of SubPicCpbSizeVcl and SubPicCpbSizeNal.

- BitRate: The derivation of BitRate related limits is currently missing from the SEI message. It is proposed to add the derivation of SubPicBitRateVcl and SubPicBitRateNal to the semantics in an aligned fashion

- Number of tiles: It is proposed to add a constraint regarding the number of tiles

In addition, minor typos are fixed.

# Introduction

The current VVC draft specification includes signaling means for subpicture conformance. However, there are still some parts missing parts to provide a full solution that covers the use cases motivating scenarios of subpicture extraction and merging.

Currently, the subpicture level information SEI message specifies syntax elements that allow to derive level related information of bitstreams resulting from a sub-bitstream extraction process. For that purpose, constraints are specified for SubPicSizeY[ i ], subpic_width_minus1[ i ], subpic_height_minus1[ i ], SubPicNumTileCols[ j ], SubPicNumTileRows[ j ] and SubPicCpbSizeNal/Vcl[ i ][ j ].

The SubPicCpbSizeNal/Vcl[ i ][ j ] are derived as follows and based on the MaxCPB values of the indicated reference level.

SubPicCpbSizeVcl[ i ][ j ] = Floor( CpbVclFactor * MaxCPB * RefLevelFraction[ i ][ j ] ÷ 256)	(D-6)

SubPicCpbSizeNal[ i ][ j ] = Floor( CpbNalFactor * MaxCPB * RefLevelFraction[ i ][ j ] ÷ 256)	(D-7)

However, as MaxCPB of the level definitions is a maximum worst-case value, it is assumed that the inferred value of RefLevelFraction (ratio of subpicture area to picture area) will in most cases not be ideal and lead to a SubPicCpbSize that is larger than necessary or explicit signalling of ref_level_fraction in such cases. Using the actual CpbSize of the bitstream when it indicated in the HRD parameters for the above derivation may be beneficial and avoid the need to explicitly signal the ref_level_fraction depending on the encoder setting.

The current VVC draft specification also misses the bitrate derivation which can be carried out according to the above.

In addition, SubPicNumTileCols[ j ] and SubPicNumTileRows[ j ] fulfil constraints separately while the contribution of those to a level would be also be in terms of number of tiles, i.e. SubPicNumTileCols[ j ] multiplied by SubPicNumTileRows[ j ]. Therefore, a constrain related to the number of tiles in a subpicture is proposed.

In addition, minor typos are fixed.

# Proposal

## Updates to subpicture level information SEI message semantics for CpbSize, Bitrate and Number of tiles

The variable RefLevelFraction[ i ][ j ] is set equal to ref_level_fraction_minus1[ i ][ j ] + 1.

The variables SubPicNumTileCols[ j ] and SubPicNumTileRows[ j ] are derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++) {
	SubPicNumTileCols[ i ] = 1
	SubPicNumTileRows[ i ] = 1
	for( ctbAddrRs = subpic_ctu_top_left_x[ i ] + 1; ctbAddrRs  <=				subpic_ctu_top_left_x[ i ] + subpic_width_minus1[ i ]; ctbAddrRs++ )
		if( CtbToTileColBd[ ctbAddrRs ]  !=  CtbToTileColBd[ ctbAddrRs − 1 ] )
			SubPicNumTileCols[ i ]++		(D.5)
	for( ctbAddrRs = ( subpic_ctu_top_left_y[ i ] + 1 ) * PicWidthInCtbsY;
			ctbAddrRs  <=  ( subpic_ctu_top_left_y[ i ] + subpic_height_minus1[ i ] ) * PicWidthInCtbsY;
			ctbAddrRs  +=  PicWidthInCtbsY )
		if( CtbToTileRowBd[ ctbAddrRs ]  !=  CtbToTileRowBd[ ctbAddrRs − PicWidthInCtbsY ] )
			SubPicNumTileRows[ i ]++
}

The variables SubPicCpbSizeVcl[ i ][ j ] and SubPicCpbSizeNal[ i ][ j ] are derived as follows:

When ref_level_idc[ i ] is equal to general_level_idc in the profile_tier_level( ) syntax structure of the active SPS, the value of ValCPB is set to:

ValCPB = ( cpb_size_value_minus1[ Htid ][ cpbIdx ] + 1) * 2( 4 + cpb_size_scale )

wherein Htid is the index of the highest sub-layer and cpbIdx is the index of the selected CPB.

Otherwise, the value of ValCPB is set to MaxCPB derived from ref_level_idc[ i ] as specified in clause A.4.2.

SubPicCpbSizeVcl[ i ][ j ] = Floor( CpbVclFactor * MaxValCPB * RefLevelFraction[ i ][ j ] ÷ 256)	(D-6)

SubPicCpbSizeNal[ i ][ j ] = Floor( CpbNalFactor * MaxValCPB * RefLevelFraction[ i ][ j ] ÷ 256)	(D-7)

with MaxCPB derived from ref_level_idc[ i ] as specified in clause A.4.2.

The variables SubPicBitRateVcl[ i ][ j ] and SubPicBitrateNal[ i ][ j ] are derived as follows:

When ref_level_idc[ i ] is equal to general_level_idc in the profile_tier_level( ) syntax structure of the active SPS the value of ValBR is set to:

ValBR = ( bit_rate_value_minus1[ Htid ][ j ] + 1) * 2( 6 + bit_rate_scale )

wherein Htid is the index of the highest sub-layer and cpbIdx is the index of the selected CPB.

Otherwise, the value of ValBR is set to MaxBR derived from ref_level_idc[ i ] as specified in clause A.4.2.

SubPicBitRateVcl[ i ][ j ] = Floor( CpbVclFactor * MaxValBR * RefLevelFraction[ i ][ j ] ÷ 256)	(D-8)

SubPicBitRateNal[ i ][ j ] = Floor( CpbNalFactor * MaxValBR * RefLevelFraction[ i ][ j ] ÷ 256)	(D-9)

NOTE 1 – When a subpicture is extracted, the resulting bitstream has a CpbSize (either indicated in the SPS or inferred) that is greater than or equal to SubPicCpbSizeVcl[ i ][ j ] and SubPicCpbSizeNal[ i ][ j ] and a BitRate (either indicated in the SPS or inferred) that is greater than or equal to SubPicBitRateVcl[ i ][ j ] and SubPicBitRateNal[ i ][ j ].

It is a requirement of bitstream conformance that the bitstreams resulting from extracting the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1, inclusive, and conforming to a profile with general_tier_flag equal to 0 and level equal to ref_level_idc[ i ] for i in the range of 0 to num_ref_level_minus1, inclusive, shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	Ceil( 256 * SubPicSizeY[ i ] ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1.

–	The value of Ceil( 256 * ( subpic_width_minus1[ i ] + 1 ) ÷ RefLevelFraction[ i ][ j] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of Ceil( 256 * ( subpic_height_minus1[ i ] + 1 ) ÷ RefLevelFraction[ i ][ j ] ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of SubPicNumTileCols[ j ] shall be less than or equal to MaxTileCols and of SubPicNumTileRows[ j ] shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1.

–	The value of SubPicNumTileCols[ j ] * SubPicNumTileRows[ j ] shall be less than or equal to MaxTileCols * MaxTileRows * RefLevelFraction[ i ][ j ], where MaxTileCols and MaxTileRows are specified in Table A.1.

For any subpicture set containing one or more subpictures and consisting of a list of subpicture indices SubPicSetIndices and a number of subpictures in the subpicture set NumSubPicInSet, the level information of the subpicture set is derived.

The variable SubPicSetAccLevelFraction[ i ] for the total level fraction with respect to the reference level ref_level_idc[ i ], and the variables SubPicSetCpbSizeVcl[ i ][ j ] and SubPicSetCpbSizeNal[ i ][ j ], SubPicSetBitRateVcl[ i ] and SubPicSetBitRateNal[ i ] of the subpicture set, are derived as follows:

for (i = 0; i  <=  num_ref_level_minus1; i ++) {
	SubPicSetAccLevelFraction[ i ] = 0
	SubPicSetCpbSizeVcl[ i ] = 0
	SubPicSetCpbSizeNal[ i ] = 0
	SubPicSetNumTiles[ i ] = 0
	for (j = 0; j < NumSubPicInSet; j ++) {
		SubPicIdx = SubPicSetIndices[ j ]
		SubPicSetAccLevelFraction[ i ]  +=  RefLevelFraction[ i ][ SubPicIdx ]	(D.8)
		SubPicSetCpbSizeVcl[ i ]  +=  SubPicSetCpbSizeVcl[ i ][ SubPicIdx ]
		SubPicSetCpbSizeNal[ i ]  +=  SubPicSetCpbSizeNal[ i ][ SubPicIdx ]
		SubPicSetBitRateVcl[ i ]  +=  SubPicSetBitRateVcl[ i ][ SubPicIdx ]
		SubPicSetBitRateNal[ i ]  +=  SubPicSetBitRateNal[ i ][ SubPicIdx ]
		SubPicSetNumTiles[ i ]  +=  SubPicNumTileCols[ SubPicIdx ] * 
			SubPicNumTileRow[ SubPicIdx ]
	}
}

The value of the subpicture set sequence level indicator, SubPicSetLevelIdc, is derived as follows:

SubPicSetLevelIdc = general_level_idc
for (i = num_ref_level_minus1; i  >=  0; i− − )
	if( SubPicSetNumTiles[ i ]  <=  ( MaxTileCols * MaxTileRows )  &&	(D.9)
			SubPicSetAccLevelFraction[ i ]  <=  256 )
		SubPicSetLevelIdc = ref_level_idc[ i ]

where MaxTileCols and MaxTileRows are specified in Table A.1 for ref_level_idc[ i ].

The subpicture set bitstream conforming to a profile with general_tier_flag equal to 0 and a level equal to SubPicSetLevelIdc shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- For the VCL HRD parameters, SubPicSetCpbSizeVcl[ i ] shall be less than or equal to CpbVclFactor * MaxCPB, where CpbVclFactor is specified in Table A.3 and MaxCPB is specified in Table Table A.1 in units of CpbVclFactor bits.

- For the NAL HRD parameters, SubPicSetCpbSizeNalVcl[ i ] shall be less than or equal to CpbNalFactor * MaxCPB, where CpbNalFactor is specified in Table A.3, and MaxCPB is specified in Table Table A.1 in units of CpbNalFactor bits.MaxCPB

- For the VCL HRD parameters, SubPicSetBitRateVcl[ i ] shall be less than or equal to CpbVclFactor * MaxBR, where CpbVclFactor is specified in Table A.3 and MaxBR is specified in Table Table A.1 in units of CpbVclFactor bits.

- For the NAL HRD parameters, SubPicSetCpbSizeNal[ i ] shall be less than or equal to CpbNalFactor * MaxCR, where CpbNalFactor is specified in Table A.3, and MaxBR is specified in Table Table A.1 in units of CpbNalFactor bits.

NOTE 2 – When a subpicture set is extracted, the resulting bitstream has a CpbSize (either indicated in the SPS  or inferred) that is greater than or equal to SubPicSetCpbSizeVcl[ i ][ j ] and SubPicSetCpbSizeNal[ i ][ j ] and a BitRate (either indicated in the SPS  or inferred) that is greater than or equal to SubPicSetBitRateVcl[ i ][ j ] and SubPicSetBitRateNal[ i ][ j ].

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)