| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0494_r1 |
| --- | --- |

| Title: | CE5-related: Joint clip operation for CCALF and chroma ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma, Xiaoyu Xiu, Yi-Wen Chen, Hong-Jheng Jhu, Xianglin Wang 8910 University Center Dr., Suite 400, San Diego, CA 92122 | Tel: Email: | 1-310-567-2937 {tsung-chuanma, xiaoyuxiu, yiwenchen, jhuhong-jheng, xianglinwang}@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

According to Core Experiment (CE5) [1] common base software, when both chroma ALF and CCALF are enabled, range clipping operations are performed on modified chroma samples separately in each of the two modules to prevent overflow or underflow. In this contribution, it is proposed to perform only one range-clipping operation after chroma samples processed by ALF and CCALF. The proposed change is implemented based on CE5 [1] software. Simulation results show that the proposed change results in a BD-rate change (YUV) of 0.00%, 0.00%, 0.00% for AI, 0.00%, 0.00%, 0.00% for RA, XXX%, XXX%, XXX% for LDB, respectively.

# Introduction

In CE5 common base [2], CCALF is proposed as a candidate for adoption for VVC [3]. As shown in Figure 1, range clipping operations are performed on modified chroma samples separately after chroma ALF and CCALF to prevent overflow or underflow. When both chroma ALF and CCALF are enabled, chroma samples from chroma ALF module are range-clipped first, then added with the output from CCALF and range-slipped again. Such a double-clipping on chroma samples does not seem necessary.

We propose to remove the first range-clipping performed right after chroma ALF module, as shown in Figure 2.

Figure 1. Current of CCALF and chroma ALF implementation

Figure 2. Proposed modification for CCALF and chroma ALF

# Simulation results

The proposed changes were implemented on top of CE5 common base [1] software and tested based on the conditions specified in CE5 description [1].

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5-Anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 101% |

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5-Anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.02% | 101% | 101% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 99% | 100% |

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE5-Anchor |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | XXX% | XXX% | XXX% | XXX% | XXX% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Overall | XXX% | XXX% | XXX% | XXX% | XXX% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 102% |

# Conclusions

Based on the proposed method in this contribution, only one range-clipping operation is needed for each chroma sample processed by ALF and CCALF, instead of two. Simulation results show that the proposed change results in negligible BD-rate impact. We recommend that if CCALF is adopted into VVC, the proposed change is to be adopted together.

# Reference

- [1] C.-Y. Chen, A. Segall, “Description of Core Experiment 5 (CE5): Cross-component adaptive loop filtering”, JVET-P2025, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

- [2] K. Misra, F. Bossen, P. Cowan, A. Segall (Sharp Labs of America), N. Hu, J. Dong, V. Seregin, M. Karczewicz (Qualcomm), P. Onno, C. Gisquet, G. Laroche (Canon), J. Li, C.S. Lim, C.-W. Kuo (Panasonic), J. Nam, J. Choi, J. Lim, S. Kim (LGE), O. Chubach, C.-Y. Lai, C.-Y. Chen, T.-D. Chuang, Y.-W. Huang, S.-M. Lei (MediaTek), “CE5 Common Base: Cross Component Adaptive Loop Filter”, JVET-Q0058, ISO/IEC JTC1/SC 28/WG 11 Joint Video Exploration Team (JVET) 17th Meeting, Brussels, BE, 7–17 January 2020

- [3] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).