| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0649 |
| --- | --- |

| Title: | Cross-check of JVET-Q0186 (CE3-related: TB-level residual coding selection for lossless coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Tsung-Chuan Ma | Tel: Email: | tsung-chuanma@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This is a crosscheck for JVET-Q0186. In this contribution, transform block (TB) level switch between TSRC and RRC for lossless coding is proposed to further improve lossless coding gain.

Two aspects are confirmed by cross-checkers, source code is identical to description. Simulation results all match proponent’s results.

# Introduction

In this contribution, transform block (TB) level switch between TSRC and RRC for lossless coding is proposed to further improve lossless coding gain. First, a new “slice transform disabled flag” is signalled or inferred at slice level, and it is used to indicate whether transform is disabled for the slice. When the slice transform disabled flag is true, the transform process and quantization process are the same as those when TS is applied, and the TS flag is still signalled at TB level but used to select TSRC or RRC. Second, since in VTM7.0 block-wise differential pulse code modulation (BDPCM) blocks are always combined with TS and TSRC, to remove this combination so as to make BDPCM blocks be able to choose between TSRC and RRC, a new “slice BDPCM infer transform skip disabled flag” is signalled or inferred at slice level, and it is used to indicate whether TS and TSRC are combined with BDPCM blocks. If the slice BDPCM infer transform skip disabled flag is true, the TS flag is still signalled for each BDPCM block, and TS is still applied to all BDCM blocks, but the TS flag is used to select TSRC or RRC.

Proposed aspect is confirmed by cross-checkers. Source code does match description and spec is correct.

# Results

Results 	are proposed over the VTM7.0 anchor. Runtimes are not reliable.

Table 1. Proposed method with BDPCM disabled

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -5.67% | 2.2 | 2.4 | -6.47% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.99% | 1.7 | 1.9 | -8.25% |  |  |  |
| Class B | 2.2 | 2.3 | -4.22% | 2.3 | 2.4 | -3.95% | 2.3 | 2.4 | -3.95% |
| Class C | 1.9 | 2.0 | -5.20% | 2.4 | 2.6 | -5.52% | 2.4 | 2.6 | -5.43% |
| Class D | 1.9 | 2.0 | -4.90% | 2.8 | 2.9 | -5.31% | 2.7 | 2.9 | -5.27% |
| Class E | 2.8 | 2.9 | -4.10% |  |  |  | 3.1 | 3.3 | -6.43% |
| Class F | 5.3 | 5.5 | -3.59% | 33.7 | 35.0 | -4.34% | 50.7 | 52.3 | -4.20% |
| TGM | 11.8 | 12.3 | -3.88% | 107.1 | 112.4 | -4.52% | 124.9 | 131.3 | -4.52% |
| Overall | 2.1 | 2.3 | -5.46% | 2.2 | 2.4 | -5.73% | 2.6 | 2.7 | -5.06% |
| Enc Time[%] | 118% |  |  | 112% |  |  | 115% |  |  |
| Dec Time[%] | 97% |  |  | 96% |  |  | 99% |  |  |

Table 2. Proposed method with luma and chroma BDPCM enabled

|  | All Intra |  |  | Random Access |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | ratio |  | bit-rate savings | ratio |  | bit-rate savings | ratio |  | bit-rate savings |
|  | VTM7.0 | TEST |  | VTM7.0 | TEST |  | VTM7.0 | TEST |  |
| Class A1 | 2.2 | 2.4 | -6.81% | 2.2 | 2.4 | -7.21% |  |  |  |
| Class A2 | 1.6 | 1.8 | -10.56% | 1.7 | 1.9 | -8.59% |  |  |  |
| Class B | 2.2 | 2.3 | -6.42% | 2.3 | 2.5 | -4.63% | 2.3 | 2.4 | -4.64% |
| Class C | 1.9 | 2.1 | -7.30% | 2.4 | 2.6 | -6.10% | 2.4 | 2.6 | -6.00% |
| Class D | 1.9 | 2.1 | -9.06% | 2.8 | 2.9 | -6.09% | 2.7 | 2.9 | -5.86% |
| Class E | 2.8 | 3.1 | -9.35% |  |  |  | 3.1 | 3.4 | -7.93% |
| Class F | 5.3 | 5.9 | -8.66% | 33.7 | 36.2 | -6.53% | 50.7 | 53.4 | -5.85% |
| TGM | 11.8 | 13.0 | -8.50% | 107.1 | 114.7 | -6.37% | 124.9 | 132.1 | -5.44% |
| Overall | 2.1 | 2.3 | -7.86% | 2.2 | 2.4 | -6.33% | 2.6 | 2.7 | -5.91% |
| Enc Time[%] | 131% |  |  | 117% |  |  | 111% |  |  |
| Dec Time[%] | 95% |  |  | 101% |  |  | 98% |  |  |