| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0681-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-Q0437 (Non-CE4: Disabling MTS for Geo) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Xin Zhao | Tel: Email: | xinzzhao@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution reports the crosscheck results of JVET-Q0437, which proposes to disable MTS for Geometric Inter Prediction. The software implementation is provided by the proponent of JVET-Q0437, and simulations results are reportedly matching with the results provided by proponent of JVET-Q0437.

# Crosscheck report

In CE4, a new inter prediction tool named Geometric Inter Prediction (Geo) was studied on top of VTM-7. With Geo, a coding block can be partitioned as two parts using a straight line with flexible angles and displacements. It is reported in JVET-Q0437 that, residuals of Geo predicted blocks may be less homogeneous compared to regular one and it is therefore proposed that only DCT-2 is allowed for Geo prediction residuals.

The software provided by the proponent of JVET-Q0437 has been studied, and it is reported that the software changes on top of the CE4 common codebase include the followings:

1. An encoder change.

2. Both Transform Skip mode and MTS is disabled for Geo predicted residual blocks.

Simulated are performed using common test condition (CTC) [1] with --MTS=3, simulation results under RA and LDB configuration are reported in this crosscheck. The anchor is CE4 common codebase, and the test is the CE4 common codebase with proposed changes. Below tables summarize the results.

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | 0.03% | -0.08% | 99% | 100% |
| Class B | 0.01% | -0.04% | 0.04% | 99% | 100% |
| Class C | 0.00% | -0.17% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | 0.00% | 99% | 100% |
| Class D | 0.04% | 0.10% | 0.03% | 99% | 100% |
| Class F | 0.02% | 0.00% | 0.01% | 99% | 99% |

|  |  |  | Low delay B |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.15% | 0.06% | 99% | 100% |
| Class C | -0.04% | -0.09% | -0.23% | 98% | 99% |
| Class E | -0.12% | -0.33% | -0.18% | 99% | 99% |
| Overall | -0.02% | -0.18% | -0.10% | 99% | 99% |
| Class D | 0.07% | -0.23% | -0.34% | 98% | 98% |
| Class F | -0.02% | -0.19% | -0.48% | 99% | 100% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, 14th Meeting: Geneva, CH, 19–27 March 2019.