| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0021-v3 |
| --- | --- |

| Title: | Deployment status of the VVC standard |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Gary J. Sullivan | Email: | g-j-sullivan@outlook.com |
| Source: | SC 29 Chair & Q6/16 VCEG Rapporteur |  |  |

_____________________________

# Abstract

This contribution contains a survey of deployed products and services, publicly available software source code, related tools, and formal specifications supporting the VVC standard (Rec. ITU-T H.266 | ISO/IEC 23090-3).

Revision marking is included to show changes relative to JVET-AC0021-v4 of January 2023.

# Introduction to the VVC standard

The VVC standard (formalized as Rec. ITU-T H.266 and ISO/IEC 23090-3) [1] was finalized in July 2020. In ITU-T its first edition was approved in August 2020 and formally published “in force” in November 2020. In ISO/IEC, the final approval ballot for the first edition was completed in January 2021 and it was published in February 2021. A journal article with an overview of the new standard and a historical perspective on the evolution of its features has been published in the Proceedings of the IEEE [2], and another journal article with an overview of the standard and discussion of its applications and early implementations has been published in the IEEE Transactions on Circuits and Systems for Video Technology [3]. A video tutorial [4] and an overview slide presentation deck [5] are also available. Web sites with further information about the new standard are found at [6][7][8], and a White Paper with further information is also available [9].

The compelling advantage of VVC is that it can achieve roughly double the compression efficiency of the previous flagship ITU-T and ISO/IEC High Efficiency Video Coding (HEVC) standard (Rec. ITU-T H.265 and ISO/IEC 23008-2) and about four times the compression efficiency of the previously-dominant H.264/MPEG-4 AVC standard (Rec. ITU-T H.264 and ISO/IEC 14496-10) – enabling, e.g., storage of more video content in the same storage space with the same video quality, sending more channels in a broadcasting service, reducing the bit rate needed for sending video on a network, or alternatively providing a major increase in video frame quality, frame rate, or picture resolution (when using a sufficiently powerful encoder). The need for maximal compression efficiency for video is pressing and urgent, since about 80% of the data transmitted on world-wide networks is video, and that percentage has been steadily growing and is projected to continue to grow further [10][11]. The introduction of ultra high definition (UHD), high dynamic range (HDR), wide color gamut (WCG), and high frame rate (HFR) video services have increased the challenge with their use of larger frame sizes, faster refresh rates for progressive-scan video, and greater pixel bit depth. For example, by 2023, it is projected that about two-thirds of installed flat-panel television will support UHD resolution (up from one third in 2018) [12].

The complexity requirements for implementation of VVC decoders are relatively modest (less than twice the complexity of a comparable HEVC decoder and less than four times the complexity of a comparable AVC decoder), and VVC encoders, while being somewhat more of a challenge, are also quite feasible and are vendor-customizable and generally less cost-sensitive.

In addition to providing a large improvement in compression efficiency capability, the VVC standard has been designed to address a very broad range of applications, including new and emerging use cases, and hence the inclusion of “versatile” in the name of the standard. The capabilities built into version 1 of the VVC standard include features designed to address high dynamic range (HDR), wide colour gamut (WCG), resolution-adaptive multi-rate video streaming services, 360-degree immersive video, bitstream extraction and merging, temporal scalability, gradual decoder refresh, and multi-layer coding to deliver layered video content to support application features such as multiview, alpha maps, depth maps, and spatial and quality scalability.

Along with the VVC standard itself, an accompanying standard known as Versatile Supplemental Enhancement Information (VSEI) has also been developed (formalized as Rec. ITU-T H.274 and ISO/IEC 23002-7) [13]. The VSEI specification defines supplemental metadata in a generalized way that can be used with VVC and also with systems specifications and possibly with other video coding standards as well. Like VVC, the VSEI standard was finalized in July 2020, and its final approval and publication had a very similar timeline. Both of these standards were produced by a joint collaborative team containing experts of ITU-T Study Group 16 and ISO/IEC JTC 1/SC 29 that is known as the Joint Video Experts Team (JVET). The JVET document archive is publicly available online at [14].

# Deployment statistics

- Bitmovin reported the following statistics in December 2022 from a video developer survey of June to September 2022 with 424 respondents from over 80 countries [15]:

  - For live coding:

    - 19% of developers “currently using” VVC in production

    - 20% of developers “planning to implement” VVC within 12–24 months

  - For video-on-demand coding:

    - 15% of developers currently using VVC

    - 29% of developers “planning to implement” VVC within 12–24 months

# Publicly available software source code

- JVET has developed the VVC Test Model (VTM) as its reference software encoder and decoder codebase [7]. It is intended primarily to demonstrate coding efficiency capability and proper interpretation of the syntax and decoding process specified in the standard (but not as a speed-optimized implementation), and is intended to be usable as a starting basis for product implementations. The software is available under a BSD copyright licence. The reference software was developed by JVET as H.266.2 (ex H.VVC.2) and ISO/IEC 23090-16. In ITU-T, its first edition was approved in April 2022 and published in July 2022. In ISO/IEC it reached the CD stage of the approval process in October 2020, the DIS stage in January 2021, the FDIS stage in January 2022 (with a ballot issued 2022-07-24 and closed on 2022-09-18), and was published in October 2022 (on 2022-10-23). A description of the encoding algorithms in the test model is also maintained and made available by JVET [16].

- InterDigital developed a multi-threaded VTM decoder, and reported 6–10× speed-up relative to the single-threaded reference software in October 2020 [17]. It was intended to support all features of the current VTM. The software was later placed in an accessible repository, and it is available under the same BSD copyright licence as the VTM software [18]. A further update to correspond with the newer VTM and improve speed by about 10% using various scheduling and low-level changes was reported in April 2021 [19].

- Fraunhofer HHI announced the VVenC encoder and VVdeC decoder open-source software (release 0.1) in September 2020 [20][21][22][23][24].

  - Release 0.1 of September 2020 included support for multithreading operation, single-pass rate control, perceptual QP adaptation, and motion-compensated temporal filtering (MCTF). The software had four defined presets for quality/speed tradeoff (called “slow”, “medium”, “fast”, and “faster”). Subjective testing reported in October 2020 indicated that the VVenC encoder had about the same or better subjective compression performance as the VTM encoder when operating in its “medium” speed configuration (operating with MCTF and QP adaptation disabled in the VTM and enabled in VVenC and with rate control disabled in both) with encoding speed more than 100× that of the VTM, for 4K UHD SDR video content [24][25].

  - As of December 2020, a “slower” preset was added, along with an improved single-pass rate control and a new two-pass rate control [26]. The “slower” preset mode reportedly achieved approximately the same BD-rate coding efficiency as the VTM while providing a speedup of more than 8.6× for UHD and 5.2× for HD sequences relative to the VTM. As of December 2020, with release 0.2, the software was available under a BSD copyright licence.

  - Release 0.3 of March 2021 included substantial further speed and multithreading improvements [27].

  - Release 1.0.0 of VVenC was declared in May 2021 with further coding efficiency and speed enhancements, encoding support for additional features, frame-level parallelism, and improved rate control [28]. The coding efficiency of the two-pass rate control was reportedly equivalent to that of fixed-QP operation.

  - Release 1.1.2 of VVdeC was issued in July 2021 with support of subpictures (including support of nested SEI messages), improved conformance testing by checking of MD5 checksums, correction of picture output order and cropping [28].

  - Version 1.1.0 of VVenC was released in August 2021 with faster presets (20% for “faster”, 15% for “fast”, 7% for “medium”), bi-prediction with CU-level weights (BCW) added as new coding tool, improved two-pass rate control (RC), screen content coding, and various bugfixes and improvements [29].

  - Version 1.2.0 of VVdeC was released in September 2021 with WebAssembly to enable decoding and playback in web browsers, various fixes and improvements, improvements of SSE41 SIMD usage, minor improvements to ALF, DMVR and LMCS processing, and an added option to remove padding from YUV on output [29].

  - Version 1.3.1 of VVenC was released in December 2021 [30]. Some of its changes included 10% speed-up of the “slower” mode, coding efficiency improvement of the “faster” mode (5% for HD and 3% for UHD), rate control improvements, improved QP adaptation for subjective quality, tile parallelism support, and MCTF improvement (ported from VTM).

  - Version 1.3.0 of VVdeC was released in December 2021 [30]. The main changes reported for VVdeC included improved tune-in behaviour, 3× memory reduction, MinGW support, general cleanups, and WebAssembly improvements (support for 32-bit browsers and theoretically UHD decoding). In addition, an example implementation of a web-based player was released to GitHub that enabled the use of VVdeC for VVC playback in a web browser [30].

  - Version 1.5.0 of VVdeC was released in April 2022 and version 1.5.0 of VVenC was released in July 2022 [31]. The main changes for VVenC v1.5.0 since version 1.3.1 included a change of license to an unmodified “Clear BSD” and adding the authors as copyright holders, GOP-based scene-cut detection, addition of header reading functionality, ability to specify bit rate using common suffixes (Mbps, M, Kbps, K, and bps), integrated Y4M video input format support, speedups (~10% for “faster”, “fast”, “medium”, ~7% for “slow” and “slower”), improved rate-control stability for noisy input and scene changes and miscellaneous general cleanups [31]. The main changes for VVdeC since version 1.3.0 included a similar change of license, support for arbitrary architectures using extensive SIMD including arm64, riscv64 and more, ARM support that enabled native playback on Android and M1 Macs, and miscellaneous general cleanups [31].

  - Version 1.6.1 of VVenC and version 1.6.0 of VVdeC were released in September 2022 [32]. The main changes for VVenC v1.6.1 since version 1.5.0 included faster presets (~15% for “faster”, “fast”, and “medium”, ~5% for “slow” and ~18% for “slower”) and miscellaneous general cleanups [32]. The main changes for VVenC v1.6.0 since version 1.5.0 were miscellaneous general cleanups [32].

  - Version 1.7.0 of VVenC was released in December 2022 [33]. The main changes for VVenC v1.7.0 since version 1.6.1 included faster presets (~20% for “faster”, ~5% for other presets), coding efficiency improvement by adding block importance mapping from the VTM (~1% for “faster”, ~2% for other presets), improved support for ARM processors with SIMDe, content-adaptive placement of random-access points, the possibility to add a logo overlay to encoded video, and some other features and cleanups [33].

  - Version 1.8.0 of VVenC was released in April 2023 and version 1.6.1 of VVdeC was released in January 2023 [34]. The main changes for VVenC v1.8.0 since version 1.7.0 included faster presets (~10% for “faster”, ~5% for “fast” and “slow” presets), improved coding efficiency for multi-threaded operation (using temporal ALF APS prediction), extended block importance mapping, and various bugfixes and cleanups [34].

- Friedrich–Alexander University Erlangen–Nürnberg released an open-source bitstream analyser as an add-on for the VTM decoder [35][36]. The analyser counts the occurrence of coding tools and coding modes used in a decoded bitstream and can be used for evaluating the decoding energy and time demands of VVC features. The software was made available under a BSD copyright licence.

- Bitmovin released a VVC video player called vvDecPlayer or BitvvDecPlayer that works with the Fraunhofer HHI VVdeC for VVC bitstream playback [37][38][39]. The player operates with four threads to perform 1) HTTP download of video segments, 2) bitstream parsing, 3) decoding to raw YUV frames, and 4) conversion to RGB for display.

# Software decoders and media players

- Sharp announced a real-time software decoder in June 2020 and issued a corresponding press release in December 2020 [40][41]. As of June 2020, it was reportedly capable of decoding 4K common test conditions (CTC) UHD bitstreams at up to 40 Mbps at more than 60 fps. In an IEEE journal paper of October 2021, the Sharp decoder was reported to be capable of decoding 8K bitstreams at up to 60 Mbps at more than 110 fps [42].

- Tencent announced its O266dec software decoder with SIMD and multithreading support and an associated FFmpeg/VLC-based video player in October 2020 [24][43][44]. As of December 2020, it was reportedly more than 3× the speed of the VTM reference software decoder when tested under VVC CTC in single-threaded operation and about 20× the VTM decoder speed in 8-thread operation. It could reportedly decode UHD video at more than 60 fps at up to 40 Mbps and decode full HD video at more than 200 fps. In December 2020, a version with mobile platform support based on Arm Neon technology was reported. On an Apple A14 processor (iPhone 12pro) in single-threaded operation, it could reportedly decode 8-bit 1080p CTC bitstreams at more than 50 fps, and in multi-threaded operation it could decode such bitstreams at more than 100 fps and could decode 8-bit 4K UHD bitstreams at more than 30 fps in the random access (RA) example configuration [45]. Although 8-bit operation was more optimized, the decoder also supported 10-bit operation. In July 2021, Tencent Cloud reported that its players and other products had begun supporting VVC decoding [46]. (For information about Tencent Cloud encoding, transcoding and related media processing applications, see item 24).

- Alibaba announced its Ali266 decoder for mobile devices (e.g., Android and iPhone) in December 2020 [47] (for information about a later release of a corresponding encoder, see below in item 23). The Ali266 decoder included optimizations for multi-threading, ARM assembly, cache efficiency, and memory usage, particularly for 8 bit video content. Real-time speed was reported for 8 bit 720p, 1080p (using 2–4 threads for up to 60 fps) and 4K (up to 7 Mbps) video content with the ALF feature disabled. In April 2021, updated information was provided with both ALF and CCALF enabled [48]. This version of the Ali266 decoder reached 15× speed-up compared to VTM-11.0 on Android P40 with 8 threads on the CTC bitstreams of class A (4K) and class B (1080p), and performed real-time decoding of tested 720p e-commerce content on most phones with 3 threads. In January 2022, a successful trial deployment of the Ali266 decoder was reported in Alibaba’s Youku video streaming service for mobile app usage [49]. The trial used movies that were encoded by the Ali266 encoder (see item 23) at 1080p and 720p at bit rates 35% to 55% below that of HEVC encoding, reportedly with satisfactory decoding time and stability metrics.

- Kwai announced K266Dec in April 2020 [50]. The decoder was designed from scratch following the VVC specification and could support all coding tools specified in the VVC Main profile. For the Android platform, K266Dec could decode 2K 8-bit CTC bitstreams at 33 fps with single thread on Huawei P40, which was 4.11× of VTM-11.0 decoder. In addition, CTC bitstreams and UGC videos were tested on various Android phones based on the 50B+ daily playback data from Kwai App. For an iOS platform, K266Dec could decode 2K 8-bit CTC bitstreams at 94 fps with a single thread on an iPhone 12 Pro Max, which was 4.78× compared to VTM-11.0 reference decoder.

- Bytedance announced its BVC decoder in April 2021 [51]. The BVC decoder supported various platforms such as Android, iOS, Linux/MacOS and Windows. It was optimized especially for mobile platforms with different decoding capabilities. For decoding of 4K 8-bit CTC bitstreams on iPhone 12 (core A14), it was reported that the BVC decoder achieved 22 fps on average with a single thread, which was 10× faster than the VTM-11.0 reference decoder, and that 55 fps could be achieved with 6 threads. For decoding of 2K 8-bit CTC bitstreams, it was reported that the BVC decoder achieved 86 fps on average with a single thread, which was 8.8× faster than the VTM-11.0 reference decoder. Updated information about the BVC decoder was provided in January 2022 [52], reporting that the decoder had been optimized for heterogeneous CPU plus GPU systems, operating on the Apple AArch64 platform with GPU acceleration of the VVC decoding process. For decoding of 4K 8-bit CTC bitstreams on a Mac mini (with the Apple M1 chip), it was reported that the updated decoder achieved 39 fps on average with a single thread, which was 10x faster than the VTM-11.0 reference decoder, and that 72 fps could be achieved with two threads. For decoding of 4K 8-bit CTC bitstreams on an iPhone 13 (with the Apple A15 chip), it was reported that the BVC decoder achieved 35 fps on average with a single thread, which was 15x faster than the VTM-11.0 reference decoder.

- Spin Digital announced a real-time software decoder and a media player called Spin Player VVC in June 2021 [53]. It included support of the VVC Scalable Main 10 profile and had 8K capability at 60 and 120 fps on recent-generation CPUs.

- AstroDesign announced a media player application called the SP-5020 VVC Viewer supporting VVC in November 2021 [54]. The AstroDesign player was based on the Sharp software decoder (discussed in item 7). This included support of the Main 10 profile and 4K capability at 60 fps on a recent-generation, 8-core CPU.

# Hardware decoders and televisions

- MediaTek announced the Pentonic 2000 chipset for televisions in November 2021, featuring VVC support with 8K resolution and up to 120 Hz frame rate [55][56][57]. The Pentonic 700 chipset for 4K televisions announced in August 2022 targets 4K televisions and supports frame rates up to 144 Hz [58]. The Pentonic 1000 chipset announced in November 2022 (for availability in the first quarter of 2023) also supports VVC with similar capabilities to the Pentatonic 700 [59].

  - Philips and Sony were reported in January 2023 to be expected to use the Pentonic chipsets in televisions shipping in 2023, with a formal announcement for 4K Philips OLED televisions with the Pentonic 1000 chipset expected in the following month [60][61][62].

- Realtek announced its RTD1319D SoC for set-top boxes in August 2022, featuring VVC support with 4K resolution [63].

- LG included VVC capability in its webOS TV 22 Developer Specifications for 8K Ultra HD (“7680x4320@60P Main/Main10@L6.1 Maximum 60Mbps”) as of January 2023 [62][64]. As of April 2023, the LG QNED99 “Quantum Dot NanoCell” 75-inch and 86-inch 8K television models include VVC support in their listed specifications (“Up to 3840x2160 @120p / 7680x4320 @ 60p”) [65].

# Devices with integrated playback capability

- Skyworth announced the A63 television around the beginning of 2023, with 4K 188 Hz HDR-10 VVC playback capability [66].

# Services and encoding products

- Bytedance announced its BVC encoder in April 2021 [67]. The majority of the coding tools and features introduced in VVC had been integrated and optimized in the BVC encoder. In addition, it also included real-world encoder features like scene change detection, rate control and multi-threading. When compared to the HM and VTM, the corresponding global BD-rate savings and speedup factors under the RA configuration are summarized as follows: “fast” preset (6 threads): 34.13% (vs. HM), 17× (HM), 131× (VTM); “slow” preset (6 threads): 41.63% (vs. HM), 1× (HM), 8× (VTM).

- KDDI Research announced a real-time VVC encoder with 4K@60 fps capability in September 2020 [68]. In December 2020, a demonstration of live 4K@60 fps transmission over a 4G mobile network was reported [69]. In April 2021, additional capability was reported, including a real-time VVC encoder with 8K@30 fps capability along with a demonstration of 8K@30 fps real-time transmission over network lines [70].

- Ateme launched support for VVC in its Titan family of products, and demonstrated the technology in an OTT channel launched in November 2020 [71].

- Bitmovin, in partnership with Fraunhofer HHI based on VVencC (as described in item 4), announced support of VVC in its video encoding platform in November 2020 [72] and announced a next-generation VOD encoder with VVC support in March 2022 [73].

- MX Player and MX TakaTak, a streaming service in India, began OTT services using VVC in June 2021 [74].

- Alibaba released information about its Ali266 encoder in July 2021 [75] (see also information above about the previously reported Ali266 decoder in item 9). For CTC classs A1 (4K), A2 (4K), B (1080p) and C (720p), when compared to the VVC VTM 13.0 reference software, its “slow” preset reportedly achieved BD rates for {Y, U, V, YUV} of {10.95%, −6.11%, −4.50%, 7.11%} under the RA configuration with an average speedup factor of 227 times. For internal VOD content, Ali266 “fast” preset was compared to the x265 open source HEVC software encoder “medium” preset, and reportedly achieved BD rates for {Y, U, V, YUV} of {−32.87%, −46.07%, −42.70%, −35.35%} under the RA configuration. Ali266 “fast” preset was aimed at real-time VVC encoding, and achieved average encoding speeds of 19.3 fps for classes 1920×1080/1080×1920 and 37.7 fps for classes 1280×720/720×1080. Updated information about the encoder was provided in October 2021 [76]. It was reported that a 1.9x encoding speedup had been achieved in the previous three months for video sequences at 1920×1080 or 1080×1920 resolution. With the “fast” preset, the encoder reportedly achieved 36.3 fps for 1920×1080/1080×1920 video and 63.4 fps for 1280×720/720×1080 video. The compression performance for the “fast” preset had also reportedly improved. For the internal VOD content, the updated Ali266 at “fast” preset achieved BD rates for {Y, U, V, YUV} of {−36.25%, −45.73%, −43.02%, −38.10%} over the widely used open source HEVC software encoder x265 at its “medium” preset, which was better than the results previously reported in July over the same anchor. Further updated information was provided in January 2022 [77]. The Ali266 encoder was enhanced with four predefined quality/speed presets called “slow”, “medium”, “fast”, and “faster”, to meet different application needs. With the “faster” preset, 10-bit real time encoding for 1080p video-on-demand (VOD) content was reported and the coding efficiency of this preset had also been improved as well. Experiments reportedly demonstrated BD rates for {YUV-PSNR, YUV-SSIM} of {−42.01%, −42.90%} over the widely used open-source HEVC software encoder x265 at its “medium” preset, compared to the previously reported results of {−38.10%, −41.10%} over the same anchor.

- Tencent announced VVC support in its RT-ONE transcoding and related media processing applications for the Tencent Cloud in July 2021 [46]. The company said its RT-ONE video infrastructure network was available worldwide, and that Tencent Cloud has reached 90% of the audio and video customers in China. The announced encoder was able to support all major tools in the VVC Main 10 Profile at the same time, and the company specifically mentioned applications such as screen content, screen sharing, cloud collaboration and cloud gaming with specialized encoding tools, facilitating computer-generated content.

- Spin Digital announced a real-time VVC encoder designed for live 4K 60 fps and 8K 30 fps UHD streaming and broadcasting in August 2022 [78]. The encoder was CPU-based and had been optimized for Intel Xeon Scalable / Ice Lake processors. It included SIMD processing using Intel AVX-512 and Intel Deep Learning Boost instructions, supported advanced multithreading for systems with tens of CPU cores, and had been integrated into a live streaming framework. In October 2022, Spin Digital provided current information to JVET describing their real-time encoder [79]. The encoder was reported to achieve 17.82%, 17.34%, and 18.70% BD-rate savings based on PSNR, MS-SSIM and VMAF, respectively, with an encoding time ratio of 1.71, compared to Spin Digital’s HEVC real-time encoder and was reportedly able to process 4Kp60 and 8Kp30 HDR 10-bit video in real-time using a single server with a dual-socket CPU architecture with 76 CPU cores. A more favourable tradeoff of fidelity with complexity was reported relative to various other encoders (see [79], Figures 3–6 and 11). This encoder had been integrated into a framework for live applications that included SDI capture and HTTP (HLS and DASH) and TS over IP (RTP, SRT, RIST) streaming capabilities. The encoder was able to reduce the company’s recommended bit rates for UHD live video from the 16–17 Mbps needed by the company’s HEVC encoder to 13–14 Mbps for the VVC encoder. The improved capability had reportedly been verified by subjective testing, with 8 out of 11 one-minute test sequences rated “excellent” at 14 Mbps and none rated below “good” quality. Further information and analysis were released in February 2023 [80][81][82][83].

- MainConcept (a subsidiary of Endeavor Streaming) announced a real-time VVC encoder and SDK with real-time 8K 10-bit capability for a public Beta release in August 2022 [84]. The official non-Beta release of the live encoder application and SDK were announced in April 2023 [85].

# Adoption into non-ITU/ISO/IEC specifications

- SBTVD Forum in Brazil, following its phase 2 testing conducted during July–December 2021, announced in January 2022 the decision for VVC to be used for the main video stream for the upcoming next-generation TV 3.0 broadcasting standard in Brazil, both for over-the-air and over-the-Internet transmission [86].

- DVB announced the adoption of VVC into its DVB-AVC TS 101 154 Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications as of v2.8.1 of 22 February 2022. “The revised DVB-AVC specification includes four conformance points for VVC, the minimum requirement being a baseline receiver capable of supporting resolutions up to 4K (3840×2160) with HDR. The three additional conformance points cover the support of high frame rates (HFR), and resolutions up to 8K (7680×4320),” as described by DVB in its press release [87][88].

- SCTE issued its specifications SCTE 281-1 VVC Video Constraints for Cable Television Part 1 – Coding [89] and SCTE 281-2 VVC Video Constraints for Cable Television Part 2 – Transport [90] in March 2023. Formats up to 8K (7680×4320) 10 bit and frame rates up to 120 fps are specified, based on the Main 10 profile (Main tier up to level 6.2).

# Bitstream analyser products

- Elecard announced support for VVC in its StreamEye and StreamAnalyzer products in April 2020 [91].

- ViCueSoft supported VVC in its VQ Analyzer bitstream analysis product, as of late 2020 [92].

- Jongbel Media Solutions released its VVC/H.266 Video ES Viewer in 2022 for high-level syntax parsing and analysis, with limited bitstream editing features [93].

- Interra Systems supports VVC in its VEGA Media Analyzer as of early 2023 [94][95].

# Conformance test sets

- A conformance test set was developed by JVET as H.266.1 (ex H.VVC.1) and ISO/IEC 23090-15 [96]. In ITU-T, its first edition was approved in April 2022 and published in July 2022. In ISO/IEC it reached the CD stage of the approval process in October 2020, the DIS stage in January 2021, the FDIS stage in October 2021 (with an FDIS ballot issued 2022-09-08 and closed on 2022-11-03), and was published in November 2022 (on 2022-11-24).

- Allegro DVT began offering a conformance test set for VVC as early as January 2020 (i.e., initially in preliminary form before the finalization of the standard) [97][98][99].

# References

- ITU-T and ISO/IEC, Versatile Video Coding, July 2020 (with subsequent approval and publication). Available from ITU-T at https://www.itu.int/rec/T-REC-H.266 and from ISO/IEC at https://www.iso.org/standard/73022.html.

- Benjamin Bross, Jianle Chen, Jens-Rainer Ohm, Gary J. Sullivan, and Ye-Kui Wang, “Developments in International Video Coding Standardization After AVC, With an Overview of Versatile Video Coding (VVC)”, Proceedings of the IEEE, Vol. 109, Issue 9, pp. 1463–1493, September 2021 (open access publication), DOI: 10.1109/JPROC.2020.3043399, available at https://ieeexplore.ieee.org/abstract/document/9328514.

- Benjamin Bross, Ye-Kui Wang, Yan Ye, Shan Liu, Gary J. Sullivan, and Jens-Rainer Ohm, “Overview of the Versatile Video Coding (VVC) Standard and its Applications”, IEEE Trans. Circuits & Systs. for Video Technol., Vol. 31, Issue 10, pp. 3736–3764, DOI: 10.1109/TCSVT.2021.3101953, October 2021 (open access publication), available at https://ieeexplore.ieee.org/document/9503377.

- Mathias Wien and Benjamin Bross, “Versatile Video Coding – Algorithms and Specification”, video tutorial presentation for IEEE International Conference on Multimedia and Expo (ICME), virtual conference, 6–10 July 2020, available from IEEE at https://signalprocessingsociety.org/community-involvement/multimedia-signal-processing and on YouTube at https://www.youtube.com/watch?v=j045KuFU2QM.

- Gary J. Sullivan, "Versatile Video Coding (VVC) Delivers: Coding Efficiency and Beyond", tutorial slide deck for invited keynote talk at IEEE Data Compression Conference (DCC), 25 March 2021, available at http://cs.brandeis.edu/~dcc/Programs/Program2021KeynoteSlides-Sullivan.pdf.

- JVET – Joint Video Experts Team, website hosted by ITU-T at https://www.itu.int/en/ITU-T/studygroups/2022-2024/16/video/Pages/jvet.aspx.

- Versatile Video Coding (VVC), software and information repository website hosted by Fraunhofer HHI at https://jvet.hhi.fraunhofer.de/.

- VVC: Versatile Video Coding, website hosted by Media Coding Industry Forum at https://www.mc-if.org/versatile-video-coding/.

- Benjamin Bross and Gary J. Sullivan (Eds.), “White paper on Versatile Video Coding” doc. JVET-W2019 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 23rd meeting, doc. N 35 of ISO/IEC JTC 1/SC 29/AG 3, 4th meeting, MDS 20736: July 2021.

- Cisco Systems, “Cisco Visual Networking Index: Forecast and Trends, 2017–2022”, Cisco Systems White Paper, November 2018. (The publication link was later redirected to a newer report with different content; the original report is available at http://web.archive.org/web/20181213105003/https://www.cisco.com/c/en/us/solutions/collateral/service-provider/visual-networking-index-vni/white-paper-c11-741490.pdf.)

- Cisco Systems, “Cisco Predicts More IP Traffic in the Next Five Years Than in the History of the Internet” (press release), 27 November 2018.

- Cisco Systems, “Cisco Annual Internet Report (2018–2023) White Paper”, Cisco Systems White Paper, 9 March 2020.

- ITU-T and ISO/IEC, Versatile Supplemental Enhancement Information, July 2020 (with subsequent approval and publication). Available from ITU-T at https://www.itu.int/rec/T-REC-H.274 and from ISO/IEC at https://www.iso.org/standard/79112.html.

- JVET document management system, available at http://jvet-experts.org.

- Bitmovin, “The 6th Annual Bitmovin Video Developer Survey”, December 2022.

- Jianle Chen, Yan Ye, and Seung Hwan Kim (Eds.), “Algorithm description for Versatile Video Coding and Test Model 18 (VTM 18)”, doc. JVET-AB2002 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 26th meeting: April 2022.

- Srinivas Gudumasu, Tangi Poirier, Fabrice Urban, Franck Hiron, Remi Jullian, and Philippe de Lagrange (InterDigital), “Multi-threaded VTM decoder description and performance analysis”, doc. JVET-T0061 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Philipe de Lagrange (InterDigital), “Release of multi-threaded VTM decoder”, announcement to JVET of software availability at https://vcgit.hhi.fraunhofer.de/delagrangep/VVCSoftware_VTM/-/tree/VTM-10.2-MT, 9 December 2020.

- Fabrice Urban, Franck Hiron, Remi Jullian, and Philippe de Lagrange, “Multi-thread VTM decoder: information update”, doc. JVET-V0088 of of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- Fraunhofer HHI VVenC software repository. [Online]. Available: https://github.com/fraunhoferhhi/vvenc.

- Fraunhofer HHI, “H.266 / VVC”, website page, accessed 11 January 2021.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, and Detlev Marpe (Fraunhofer HHI), “Open optimized VVC encoder (VVenC) and decoder (VVdeC) implementations”, doc. JVET-T0099 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Christian Helmrich, Benjamin Bross, Jonathan Pfaff, Heiko Schwarz, Detlev Marpe, and Thomas Wiegand (Fraunhofer HHI), “Information on and analysis of the VVC encoders in the SDR UHD verification test”, doc. JVET-T0103 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Benjamin Bross, Ye-Kui Wang, Yan Ye, Shan Liu, Gary J. Sullivan, and Jens-Rainer Ohm, “Overview of the Versatile Video Coding (VVC) Standard and its Applications”, preprint submitted to IEEE Trans. Circuits and Systs. for Video Tech., December 2020.

- Mathias Wien and Vittorio Baroncini, VVC Verification Test Report for Ultra High Definition (UHD) Standard Dynamic Range (SDR) Video Content, doc. JVET-T2020 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, and Detlev Marpe (Fraunhofer HHI), “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-U0135 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 21st meeting: January 2021.

- Fraunhofer HHI, “VVenC Fraunhofer Versatile Video Encoder v0.3.0.0”, white paper, 3 March 2021.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, Detlev Marpe, and Thomas Schierl (Fraunhofer HHI), “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-W0184 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 23rd meeting: July 2021.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, Detlev Marpe, and Thomas Schierl (Fraunhofer HHI), “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-X0044 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 24th meeting: October 2021.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Ivan Zupancic, Benjamin Bross, Heiko Schwarz, Detlev Marpe, and Thomas Schierl (Fraunhofer HHI), “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-Y0136 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 25th meeting: January 2022.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Schierl, “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-AA0154 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 27th meeting: July 2022.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Schierl, “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-AB0044 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 28th meeting: October 2022.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Schierl, “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-AC0266 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 29th meeting: January 2023.

- Adam Wieckowski, Jens Brandenburg, Christian Bartnik, Valeri George, Jens Güther, Gabriel Hege, Christian Helmrich, Anastasia Henkel, Tobias Hinz, Christian Lehmann, Christian Stoffers, Benjamin Bross, Heiko Schwarz, Detlev Marpe, Thomas Schierl, “Update on open, optimized VVC implementations VVenC and VVdeC”, doc. JVET-AD0266 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 29th meeting: April 2023.

- Matthias Kränzler, Christian Herglotz, and André Kaup (FAU), “Bit Stream Feature Analyzer (BSFA) for Coding Tool Statistics based on VTM-10.0”, doc. JVET-T0067 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- FAU VTM Analyzer repository. [Online]. Available: https://gitlab.lms.tf.fau.de/LMS/vtm-analyzer.

- Christian Feldmann, “VVC Video Codec – The Next Generation Codec”, 3 February 2022.

- Bitmovin vvDecPlayer video player repository. [Online]. Available: https://github.com/bitmovin/vvDecPlayer.

- Christian Feldman and Adam Więckowski, “State of VVC/H.266 (ft. Fraunhofer HHI)”, Bitmovin, accessed April 2022.

- Frank Bossen (Sharp), Performance of a reasonably fast VVC software decoder, doc. JVET-S0224 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 19th meeting: June/July 2020.

- Sharp, “Sharp Develops 8K Real-time VVC Decoder, a World First”, 4 December 2020.

- Frank Bossen, Karsten Sühring, Adam Wieckowski, and Shan Liu, “VVC Complexity and Software Implementation Analysis," in IEEE Transactions on Circuits and Systems for Video Technology, Vol. 31, Issue 10, pp. 3765–3778, DOI: 10.1109/TCSVT.2021.3072204, October 2021 (open access publication), available at https://ieeexplore.ieee.org/document/9399488.

- Bin Zhu, Shan Liu, Xiaozhong Xu, Xianguo Zhang, Chenchen Gu, Liang Wang, and Weiwei Feng (Tencent), “Performance of a VVC software decoder”, doc. JVET-T0095 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 20th meeting: October 2020.

- Tencent O266 video player repository. [Online]. Available: https://github.com/tencentcloud/o266player.

- Yiming Li, Shan Liu, Yu Chen, Yushan Zheng, Sijia Chen, Bin Zhu, and Jian Lou (Tencent), “Performance of a VVC Software Decoder on Mobile Platform”, doc. JVET-U0071 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 21st meeting: January 2021.

- Tencent, “Tencent Cloud Becomes First Cloud Service Provider to Support H.266/VVC Standard”, press release, July 16, 2021.

- J. Chen, L. Wang, R.-L. Liao, and Y. Ye (Alibaba), “VVC software decoder implementation for mobile devices”, doc. JVET-U0088 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 21st meeting: January 2021.

- L. Yu, Z. Cui, L. Wang, J. Cheng, Z. Huang, and L. Xu (Alibaba), “VVC software decoder implementation for mobile devices”, doc. JVET-V0132 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- Y. Jia, Y. Zhang, F. Hu, M. Li, W. Jiang, Z. Huang, J. Liu, J. Chen, and Y. Ye (Alibaba), “Ali266 @ Youku: trial deployment of VVC for video streaming”, doc. JVET-Y0122 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 25th meeting: January 2022.

- W.-L. Feng, F.-L. Luo, Y.-S. He, Z.-H. Liu, S.-M. Meng, and X. Wen (Kwai Inc.), “VVC Software Decoder for Mobile Platforms”, doc. JVET-V0070 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- Y. He, L. Li, Y. Li, H. Yin, J. Zhang, L. Zhang, and Y. Zhang (Bytedance), “Performance of a VVC software decoder – BVC”, doc. JVET-V0128 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- L. Li, H. Yin, L. Zhang, and Y. Zhang (Bytedance), “Update on a VVC software decoder, BVC, for heterogeneous CPU plus GPU systems”, doc. JVET-Y0054 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 25th meeting: January 2022.

- Spin Digital, “Spin Digital Announces a VVC 8K Decoder and Media Player”, 21 June 2021.

- AstroDesign, “SP-5020 VVC Viewer”, website page and November 2021 product catalogue description, accessed 10 January 2022.

- MediaTek, “MediaTek Announces New Pentonic Smart TV Family with New Pentonic 2000 for Flagship 8K 120Hz TVs”, press release on PR Newswire, 19 November 2021.

- MediaTek, “8 Best Features of the MediaTek Pentonic 2000 for 8KTVs”, 20 November 2021.

- MediaTek and TSMC, “MediaTek and TSMC Unveil the World's First 7nm 8K Resolution Digital TV System-on-Chip”, press release in Design & Reuse, 22 November 2021.

- Roy, Avik, “MediaTek launches Pentonic 700 chip for 4K televisions”, TechnoSports, 20 August 2022.

- de Looper, Christian, “MediaTek wants to power next-generation TVs and Chromebooks”, BGR, 10 November 2022.

- Larsen, Rasmus, “Sony and Philips will be the first TV makers to use MediaTek Pentonic chips”, FlatPanelsHD, 9 January 2023.

- Wheatley, Mike, “Philips and Sony's new 2023 TVs to be powered by MediaTek's new Pentonic 1000 chip”, HDTVtest, 10 January 2023.

- Flanagan, Tommy, “VVC in 2023 TVs from LG, Philips – a year earlier than expected”, Rethink Research, 12 January 2023.

- Realtek, “Realtek Launches World’s First 4K UHD Set-top Box SoC (RTD1319D) Supports VVC/H.266 Video Decoding, GPU with 10-bit Graphics, Multiple CAS, and HDMI 2.1a”, 29 August 2022.

- LG, “AV Format on webOS TV 22”, webOS TV Developer website, Accessed 13 April 2023.

- LG, “LG QNED99 75 inch 8K Smart QNED TV” and “LG QNED99 86 inch 8K Smart QNED TV”, Accessed 17 April 2023.

- Skyworth, “Skyworth A63”, accessed 4 January 2023.

- J. Cui, Y. Fan, Y. He, X. Jiang, H. Liu, H. Shi, H. Yang, H. Yang, H. Yin, J. Zhang, and L. Zhang (Bytedance), “An optimized VVC encoder implementation”, doc. JVET-V0127 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 22nd meeting: April 2021.

- KDDI Research, “KDDI Research Develops the world’s first 4K H.266 | VVC real-time encoder”, 1 September 2020.

- KDDI Research, “World’s first 4K video live transmission experiment using real-time H.266|VVC codec”, 23 December 2020.

- KDDI Research, “World’s first 8K video live transmission experiment using real-time H.266|VVC codec”, 19 April 2021.

- Ateme, “ATEME and The Explorers to Launch the First OTT Channel Promoting VVC”, press release, 10 November 2020.

- Bitmovin, “Bitmovin Enables Innovation with New VVC Codec Feature”, press release, 18 November 2020.

- Bitmovin, “Bitmovin to Unveil Next-Generation VOD Encoder at the 2022 NAB Show”, press release, 30 March 2022.

- “MX Player becomes first OTT to deploy H.266, cuts down video streaming data consumption into half”, Best Media Info, 16 June 2021.

- S. Fang, J. Guo, Z. Huang, J. Liu, S. Xu, L. Yu, J. Chen, R.-L. Liao, and Y. Ye (Alibaba), “Ali266: an optimized VVC software encoder implementation”, doc. JVET-W0127 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 23rd meeting: July 2021.

- X. Dong, S. Fang, Z. Huang, J. Liu, S. Xu, R. Yang, L. Yu, J. Chen, R.-L. Liao, and Y. Ye (Alibaba), “Update on the progress of the optimized VVC encoder implementation, Ali266”, doc. JVET-X0104 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 24th meeting: October 2021.

- X. Dong, S. Fang, Z. Huang, J. Liu, S. Xu, R. Yang, L. Yu, J. Chen, R.-L. Liao, and Y. Ye (Alibaba), “Update on Ali266, the optimized VVC encoder implementation by Alibaba”, doc. JVET-Y0066 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 25th meeting: January 2022.

- Spin Digital, “Live VVC/H.266 UHD Encoder Announced by Spin Digital”, 2 August 2022.

- Sergio Sanz-Rodriguez, Mauricio Alvarez-Mesa, and Chi Ching Chi, “A VVC/H.266 real-time software encoder for UHD live video applications”, doc. JVET-AB0043 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 28th meeting: October 2022.

- Spin Digital, “Whitepaper: A Real-time VVC Software Encoder for UHD Live Video Applications v2.0”, 17 February 2023.

- Spin Digital, “Spin Digital Releases VVC/H.266 Encoder and SDK for 4K and 8K Live Video”, 20 February 2023.

- Spin Digital, “Products / Spin Enc Live”, accessed 4 March 2023.

- Spin Digital, “Products / Spin SDK”, accessed 4 March 2023.

- “IBC 2022: MainConcept Launches VVC/H.266 SDK Beta And Demos Cloud-Ready 8K Broadcast Technology”, TVNewsCheck, 31 August 2022.

- MainConcept, “MainConcept Launches VVC, Introduces Codec-Based Ad Tech and Next-Generation Broadcast at Upcoming NAB Show”, press release, 12 April 2023.

- SBTVD Forum, “TV 3.0 Project”, accessed 11 January 2022.

- DVB, “DVB adds VVC to its video coding toolbox”, press release 22 February 2022.

- DVB TS 101 154, “Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications”, version 2.8.1, DVB BlueBook A001r19, 22 February 2022.

- SCTE 281-1 “VVC Video Constraints for Cable Television Part 1 – Coding”, 1 March 2023.

- SCTE 281-2 “VVC Video Constraints for Cable Television Part 2 – Transport”, 1 March 2023.

- Elecard, “Elecard Video Analyzers Now Support VVC”, press release, 14 April 2020.

- ViCueSoft, “VQ Analyzer”, website accessed December 2020.

- Jongbel Media Solutions, “VVC/H.266 video bitstream viewer released”, Jongbel Media Solutions website, accessed 19 December 2022.

- Interra Systems, “VEGA Media Analyzer (VMA) – For Analyzing and Debugging Media Streams”, accessed 14 March 2023.

- Interra Systems, “Interra Systems to Highlight Latest Innovations in Automated Media QC, Monitoring, and Analysis at NAB Show 2023”, accessed 14 March 2023.

- Jill Boyce, Elena Alshina, Frank Bossen, Kei Kawamura, Iole Moccagatta, and Wade Wan (Editors), “Conformance testing for versatile video coding (Draft 6)”, doc. JVET-U2008 of ITU-T/ISO/IEC Joint Video Experts Team (JVET), 21st meeting: January 2021.

- Allegro DVT, “Allegro DVT Unveils the Industry's First VVC Compliance Test Bitstreams” press release in Design & Reuse, 21 January 2020.

- Allegro DVT, “Realtek Semiconductor Selects Allegro DVT’s H.266/VVC Compliance Streams”, press release, 29 October 2020.

- Allegro DVT, “H.266/VVC Elementary Streams”, Allegro DVT website, accessed 11 January 2021.

“Globally, IP video traffic will be 82 percent of all IP traffic (both business and consumer) by 2022, up from 75 percent in 2017.” … “The implications of video growth are difficult to overstate.” … “Because of video consumption patterns, the Internet now has a much busier busy hour. Because video has a higher peak-to-average ratio than data or file sharing, and because video is gaining traffic share, peak Internet traffic will grow faster than average traffic.” – Cisco Systems, November 2018 [10].