| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0044-v1 |
| --- | --- |

| Title: | AHG4: Volumetric video-based coding (V3C) test content |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sebastian Schwarz, Miska M. Hannuksela Nokia Technologies Munich, Germany | Email: | {sebastian.schwarz, miska.hannuksela}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document introduces seven visual volumetric video-based coding (V3C) video test sequences. Four sequences for V3C V-PCC applications, and three sequences for V3C MIV applications. Together they covering various V3C use cases and content scenarios. It is asserted that these test sequences allow for easy development and verification of video coding tools targeted at volumetric video applications. Therefore, it is proposed to include these sequences as optional test content.

# Introduction

AHG4 has the mandate to “identify and characterize missing types of video material, solicit contributions, collect, and make available a variety of video sequence test material.” As such, we would like to propose V3C video sequences as optional test sequences.

In the V3C framework visual volumetric assets are segmented into coherent subsets that are projected to 2D patches, which are packed into 2D video frames and are accompanied by metadata information. Traditional 2D video codecs are used to compress 2D video frames and provide the bulk of the compression efficiency. Detailed description of the V3C standards can be found in [1, 2].

At the last meeting, we presented presented the characteristics of V3C content [3]. This contribution at hand, proposes specific test sequences especially prepared for video coding tool development.

# Video-based point cloud compression (V-PCC) content

Background on V-PCC and its video content characteristics can be found in our previous contribution [3]. For this contribution we want to propse the four test sequences shown in Tab. 1 and Fig. 1 below.

Table 1: Proposed V3C V-PCC test sequences.

|  |  |  |  | atlas |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| sequence | points/frame | framerate | frames | number | width | height |
| basketball_player | ~ 2,800,000 | 30 fps | 64 | 1 | 2560 | 3536 |
| football | ~ 600,000 | 30 fps | 150 | 1 | 1280 | 1280 |
| levi | ~ 600,000 | 30 fps | 150 | 1 | 1280 | 1680 |
| longdress | ~ 800,000 | 30 fps | 150 | 1 | 1280 | 1584 |

| basketball_player_vox11 | football_vox10 | levi_vox10 | longdress_vox10 |
| --- | --- | --- | --- |

Figure 1. Examples of potential V-PCC test content.

Independent geometry, attribute, and occupancy videos for each sequence have been created using a specific branch of the MPEG TMC2 test model, configured to output individual video streams [4].

Furthermore, we used full resolution occupancy maps, and only a single layer (map per patch), by setting the following configuration parameters:

occupanyPrecision: 	1

layerCountMinus1:	0

Encoding performance for VTM19.0 and ECM8.0 are attached to this contribution.

# MPEG Immersive video (MIV) content

Background on V-PCC and its video content characteristics can be found in our previous contribution [3]. For this contribution we want to propse the three test sequences shown in Tab. 2 and Fig. 2 below.

Table 2: Proposed V3C MIV test sequences.

|  |  |  |  | atlas |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| sequence | framerate | frames |  | number | width | height |
| H – BabyUnicorn | 30 | 97 |  | 2 | 3712 | 2400 |
| N – Chess | 30 | 97 |  | 2 | 2048 | 4352 |
| S – CBAbasketball | 30 | 97 |  | 2 | 2048 | 4352 |
| H - BabyUnicorn |  |  | N - Chess |  |  |  |  |
| S - CBAbasketball |  |  |  |  |  |  |  |

Figure 2. Examples of potential MIV test content.

Independent geometry, and attribute videos for each sequence have been created using the MIV TMIV14 testmodel [6].

Furthermore, we used full resolution geometry atlases, by setting the following configuration parameter:

geometryScaleEnableFlag: 	false

Encoding performance for VTM19.0 and ECM8.0 are attached to this contribution.

# Availability

The offered sequences are available for viewing at the April 2023 meeting of JVET. People interested in viewing the sequence are requested to contact Sebastian Schwarz (sebastian.schwarz@nokia.com) during the JVET meeting days.

The overall package of test content includes:

- Four V3C V-PCC test sequences, each with

  - 1 attribute video sequence

  - 1 geometry video sequence

  - 1 occupancy video sequence

  - 1 V3C V-PCC atlas bitstream

- Three V3C MIV test sequences, each with

  - 2 attribute video sequences

  - 2 geometry video sequences

  - 1 V3C MIV atlas bitstream

The full package can be made available to JVET on short notice.

# Licenses

All test sequences provided in this contribution were generated from source material originally made available to MPEG royalty-free and under licenses at least allowing to execute, copy, modify, and create derivative works, for

(a) non-commercial internal testing for standard-setting purposes, internal demonstrations, and internal personal experimentation, and

(b) publication (or presenting papers or articles) of research results.

The same applies to the derivates of the original test sequences presented in this contribution.

# Summary & Conclusion

This contributions provides a variety of V3C test sequences covering various V3C codecs, use cases and content scenarios. Such test content allows for easy development of future video coding tools targeted at volumetric video applications, as well as verification of such and other tools in this respect. Therefore, we propose to include these sequences as optional test content.

Table 3 below provides an overview of the current performance of ECM8.0 on this content over VTM19.0. Full results are attached to this contribution. As reference, Table 4 provides the respective coding performances for current CTC test sequence classes. Comparing the performance of ECM8.0 in other classes, there seems to be room for improvement for V3C type of content. Further studies on this topic are now easily possible with the test sequences provided in this contribution.

For further references, examples of rendered viewports of the proposed sequences at various coding points are available at [5].

Table 3: V3C content coding performance overview ECM8.0 over VTM19.0

| Random Access Main10 | BD-Rate [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| V-PCC texture | -14.68% | -28.00% | -27.53% |  |  |
| V-PCC geometry | -13.78% |  |  |  |  |
| MIV texture | -14.43% | -28.87% | -25.80% |  |  |
| MIV geometry | -9.31% |  |  |  |  |
| Average | -12.81% | -27.33% | -25.34% | 677.63% | 493.67% |
| All Intra Main10 | BD-Rate [%] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| V-PCC texture | -5.60% | -24.88% | -23.73% |  |  |
| V-PCC geometry | -4.98% |  |  |  |  |
| MIV texture | -9.71% | -27.51% | -23.82% |  |  |
| MIV geometry | -10.43% |  |  |  |  |
| Average | -8.16% | -24.71% | -22.20% | 778.92% | 378.58% |

Table 4: ECM8.0 over VTM19.0 coding performance reference

| Random Access Main10 | BD-Rate [%] |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -19.67% | -22.55% | -29.57% | 665% | 678% |
| Class A2 | -22.49% | -29.28% | -32.71% | 632% | 849% |
| Class B | -18.25% | -28.98% | -26.92% | 624% | 741% |
| Class C | -19.62% | -23.28% | -23.05% | 639% | 763% |
| OVERALL | -19.75% | -26.23% | -27.58% | 637% | 754% |
| Class D | -20.33% | -24.39% | -24.43% | 647% | 736% |
| Class F | -26.01% | -32.73% | -33.01% | 571% | 459% |
| All Intra Main10 | BD-Rate [%] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -9.03% | -18.79% | -25.17% | 667% | 333% |
| Class A2 | -13.53% | -22.78% | -25.89% | 644% | 337% |
| Class B | -8.30% | -21.80% | -20.07% | 626% | 357% |
| Class C | -9.22% | -14.00% | -14.34% | 666% | 350% |
| Classe E | -10.49% | -18.86% | -17.70% | 629% | 375% |
| OVERALL | -9.86% | -19.24% | -20.22% | 645% | 351% |
| Class D | -7.50% | -11.48% | -11.53% | 673% | 354% |
| Class F | -23.25% | -31.04% | -30.57% | 471% | 329% |

# References

[1]	D. Graziosi, et al., "An overview of ongoing point cloud compression standardization activities: video-based (V-PCC) and geometry-based (G-PCC)", APSIPA Transactions on Signal and Information Processing: Vol. 9: No. 1, 2020.

[2]	V. K. M. Vadakital, et al., "The MPEG Immersive Video Standard—Current Status and Future Outlook," in IEEE MultiMedia, vol. 29, no. 3, pp. 101-111, 1 July-Sept. 2022.

[3]	S. Schwarz et al., “On visual volumetric video-based coding (V3C) testing conditions,” 
https://jvet-experts.org/doc_end_user/current_document.php?id=12226

[4] V-PCC TMC2 testmodel, https://mpeg.expert/software/MPEG/PCC/TM/mpeg-pcc-tmc2/-/tree/V3CBitstreamsWithoutVideoBitstreams

[5] https://mpegfs.int-evry.fr/mpegcontent/ws-mpegcontent (access through MPEG DMS) → Explorations → V3C → JVET-AC0041_viewports.zip

[6] Test Model 14 for MPEG immersive video, https://dms.mpeg.expert/doc_end_user/current_document.php?id=84070&id_meeting=191

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).