| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting Antalya, TR, 21–28 April 2023 | Document: JVET-AD0048-v2 |
| --- | --- |

| Title: | EE2-related: Cross-component merge mode with temporal candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hsin-Yi Tseng, Chia-Ming Tsai, Cheng-Yen Chuang, Chih-Wei Hsu, Ching-Yeh Chen, Tzu-Der Chuang, Olena Chubach, Yi-Wen Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {ivy-hy.tseng, chia-ming.tsai, cheng-yen.chuang, cw.hsu, chingyeh.chen, peter.chuang, olena.chubach, yiwen-yw.chen yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

In JVET-AC0315, a cross-component merge (CCMerge) mode was proposed to inherit cross-component model parameters from spatial adjacent and non-adjacent neighbors. In this contribution, a method of further inheriting cross-component model parameters from temporal candidates is proposed. The definitions of terms related to temporal candidates, such as inclusion order, position, and collocated picture, are the same as those in the ECM-8.0 inter merge mode.

On top of ECM-8.0, simulation results of including the proposed temporal candidates into CCMerge mode are reported as below:
RA: {Y BD-rate = -0.xx%, Cb BD-rate = -0.xx%, Cr BD-rate = -0.xx%, EncT = 10x%, DecT = 10x%}

Introduction

Cross-component prediction (CCP) which includes cross-component linear model (CCLM), gradient linear model (GLM), convolutional cross-component model (CCCM), and CCCM variants is adopted in ECM to exploit the cross-component correlation. In JVET-AC0315, a cross-component merge (CCMerge) mode is proposed for inheriting CCP parameters from neighboring positions of the current block. In this proposal, further inheriting CCP parameters from temporal candidates is explored.

Proposed method

In CCMerge mode, final cross-component model parameters of the current chroma block can be inherited from its spatial adjacent and non-adjacent neighbors, or default models. First, a merge list is created, which includes CCP models from the spatial adjacent and non-adjacent neighbors coded in CCLM, MMLM, CCCM, GLM, chroma fusion, and CCMerge modes. After including neighboring CCP models, default models are further included to fill the remaining empty positions in the merge list. To avoid including redundant CCP models in the merge list, pruning operation is applied.

To improve the coding efficiency of CCMerge mode, two types of candidates, temporal candidates and shifted temporal candidates, are additionally included into the merge list. Temporal candidates are added into the merge list after the spatial adjacent candidates. Shifted temporal candidates are added after the spatial non-adjacent candidates. Candidates in the merge list are reordered based on template SAD cost, which helps to further improve signaling efficiency. Details of the two types of candidates are described in the following sections.

Temporal candidates

Temporal candidates are selected from the collocated picture. Fig. 1 depicts positions and inclusion order of the temporal candidates, which are the same as those for temporal candidates in inter merge mode of ECM-8.0. The definition of a collocated picture is also the same as that in ECM-8.0.

Fig. 1.  Positions of the temporal candidates

An inclusion order of the temporal candidates is as follows: C01  C02  …  C010. If C0i is not available (i.e., the position of C0i is outside of picture/slice boundary, or outside of the current CTU row) and C1i is available, C1i will be used to replace C0i, where 1 ≤ i ≤ 10. Otherwise (both C0i and C1i are not available), the next inclusion position is checked.

Shifted temporal candidates

Shifted temporal candidates are also selected from the collocated picture. As depicted in Fig. 2, the position of the collocated block is shifted by a selected neighboring motion vector. Consequently, the positions of C0i and C1i are shifted by the same neighboring motion vector. The inclusion order of shifted temporal candidates is the same as that of temporal candidates. The neighboring motion vector is selected from the motion vectors of the neighboring blocks as depicted in Fig. 3. The checking order is as follows: L0B1  L1B1  L0A1  L1A1  L0B0  L1B0  L0A0  L1A0  L0B2  L1B2. The first motion vector which uses the collocated picture as the reference picture is selected. If no such motion vector is found, no shifted temporal candidate is added.

Fig. 2.  Positions for selecting the neighboring motion vector

Fig. 3.  Positions for selecting the neighboring motion vector

Experimental results

The proposed method is implemented on top of ECM-8.0 [1] and tested following the common test conditions [2]. The simulation results over EE2 Test-1.6a and ECM 8.0 are summarized in Table 1. Note that the results of Class A1 are estimated due to that “Tango2 QP22” result is not completed and is replaced by ECM 8.0 anchor result. The EncT and DecT are not accurate. Based on short simulation, there are almost no EncT and DecT changes compared with EE2 Test-1.6a.

Table 1.  Simulation results of CCMerge+proposal (temporal candidates & shifted temporal candidates) under random access (RA) condition (to be updated)

|  | Over ECM 8.0 |  |  |  |  | Over Test-1.6a |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | EncT | DecT | Y | Cb | Cr | EncT | DecT |
| Class A1 | -0.58% | -1.36% | -4.11% | 105% | 104% | -0.37% | -0.32% | -1.77% | 90% | 84% |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.00% | -2.22% | -2.22% | 103% | 102% | 0.03% | -0.52% | -0.72% | 87% | 82% |
| Class C | -0.05% | -0.55% | -0.41% | 102% | 101% | 0.00% | -0.26% | -0.35% | 88% | 81% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |
| Class D | 0.05% | -0.82% | -0.88% | 102% | 101% | 0.07% | -0.58% | -0.23% | 87% | 79% |
| Class F | -0.14% | -1.39% | -1.35% | 103% | 105% | 0.00% | -0.09% | -0.20% | 83% | 76% |

Conclusions

This contribution presents adding temporal candidates for a cross-component merge mode for chroma intra coding. The proposed method results in promising coding gains with affordable low computational complexity. It is recommended to further study this proposal in EE2.

References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).