| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0052-v1 |
| --- | --- |

| Title: | AHG9: NNPFC SEI extension mechanism to enable usages beyond user viewing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela Francesco Cricri Honglei Zhang Nokia Technologies, Tampere, Finland | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document proposes to provision the neural-network post-filter characteristics (NNPFC) SEI message so that machine analysis related indications can be added in the NNPFC SEI message in a future VSEI version. Two options are proposed:

- In this VSEI version, it is proposed to add the target usage (nnpfc_target_usage) syntax element as a ue(v)-coded bit field, where bit 0 is defined to indicate user viewing.

A machine analysis target usage can be added in a future VSEI version by defining another bit of nnpfc_target_usage.

- In this VSEI version, it is proposed to add an nnpfc_purpose bit to indicate whether the intended usages include user viewing.

A machine analysis bit for nnpfc_purpose can be added in a future VSEI version.

In the proponents' opinion, nnpfc_purpose indicates a filtering purpose as a format transfer, whereas the intended usage of the filtered pictures for user viewing or machine analysis is another dimension. The proponents therefore prefer option 1.

# Introduction

It is expected that neural-network post-processing filters (NNPFs) can be used for improving the performance of machine analysis tasks. Some examples of prior works are briefly described in Section 4.

Two input contributions in the JVET-AC meeting proposed to indicate that the intended usage of an NNPF is a machine analysis task, briefly summarized as follows:

- JVET-AC0076 [1] proposed to add the target usage in the NNPFC SEI message to indicate whether the filtered video is suitable for any usage, is intended for user viewing, or is expected to be provided as input to machine analysis.

- Among other things JVET-AC0136 [2] proposed to add an nnpfc_purpose value to indicate machine analysis and, conditioned by this nnpfc_purpose value, an additional syntax element (nnpfc_machine_vision_task_type) to further indicate the type of the machine vision task.

In response to JVET-AC0076, further study was recommended, including study of whether consideration of machine analysis is necessary to indicate specifically for the filtering process and if so, what kind of indication should be provided.

Since it might be difficult to conclude a stable design for machine analysis support for the NNPFC SEI message in the JVET-AD meeting, this contribution proposes to provision the NNPFC SEI message so that machine analysis support can be added in a future VSEI version.

Two options are proposed in this contribution:

- In this VSEI version, it is proposed to add the target usage (nnpfc_target_usage) syntax element as a ue(v)-coded bit field, where bit 0 is defined to indicate user viewing.

A machine analysis target usage can be added in a future VSEI version by defining another bit of nnpfc_target_usage.

- In this VSEI version, it is proposed to add an nnpfc_purpose bit to indicate whether the intended usages include user viewing.

A machine analysis bit for nnpfc_purpose can be added in a future VSEI version.

In the proponents' opinion, nnpfc_purpose indicates a filtering purpose as a format transfer, whereas the intended usage of the filtered pictures for user viewing or machine analysis is another dimension. The proponents therefore prefer option 1.

# Option 1: nnpfc_target_usage syntax element

## Overview

This option refines the proposal of JVET-AC0076 as follows:

- The target usage (nnpfc_target_usage) syntax element is indicated as a ue(v)-coded bit field, inspired by the design of nnpfc_purpose resulting from the JVET-AC meeting.

- Bit 0 is defined to indicate user viewing.

A machine analysis target usage can be added in a future VSEI amendment by defining another bit of nnpfc_target_usage.

## Proposed specification text changes

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_target_usage | ue(v) |
| nnpfc_id | ue(v) |
| … |  |

nnpfc_target_usage indicates the intended usages of the filtered output sample arrays resulting from the post-processing filter as specified in Table XX. The filtered output sample arrays may undergo further processing, such as colour space conversion, prior to their intended usage.

The value of nnpfc_target_usage shall be in the range of 0 to 1, inclusive, in bitstreams conforming to this edition of this document. Values in the range of 2 to 255, inclusive, for nnpfc_target_usage are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_target_usage in the range of 2 to 255, inclusive. Values of nnpfc_target_usage greater than 255 shall not be present in bitstreams conforming to this edition of this document and are not reserved for future use.

Table XX – Intended usages of the filtered output sample arrays resulting from the post-processing filter based on nnpfc_target_usage values

| Value | Interpretation |
| --- | --- |
| nnpfc_target_usage = = 0 | The intended usage is determined by the application. |
| ( nnpfc_target_usage & 0x01 ) != 0 | The intended usages include user viewing. |

## Example how machine analysis usage can be added in a future VSEI version

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_target_usage | ue(v) |
| nnpfc_id | ue(v) |
| … |  |
| if( ( nnpfc_target_usage & 0x02 ) != 0 ) { |  |
| // syntax elements further characterizing machine analysis |  |
| } |  |
| … |  |

Table XX – Intended usages of the filtered output sample arrays resulting from the post-processing filter based on nnpfc_target_usage values

| Value | Interpretation |
| --- | --- |
| nnpfc_target_usage = = 0 | The intended usage is determined by the application. |
| nnpfc_target_usage > 0 && ( nnpfc_target_usage & 0x01 ) = = 0 | The intended usages do not include user viewing. |
| ( nnpfc_target_usage & 0x01 ) != 0 | The intended usages include user viewing. |
| nnpfc_target_usage > 0 && ( nnpfc_target_usage & 0x02 ) = = 0 | The intended usages do not include machine analysis. |
| ( nnpfc_target_usage & 0x02 ) != 0 | The intended usages include machine analysis. |

# Option 2: nnpfc_purpose bits

## Overview

In this option, it is proposed to specify an nnpfc_purpose bit to indicate whether the intended usages include user viewing.

The nnpfc_purpose bit for user viewing is proposed in the present VSEI text (VSEI v3), because neither of the following interpretations of nnpfc_purpose without the user viewing bit would be satisfactory:

- If it were interpreted that the NNPFC SEI message defines a filter whose intended usage is unknown, a user viewing bit might be added in a future VSEI amendment but would not be understood by VSEI v3 parser, which would discard the entire NNPFC SEI message with the nnpfc_purpose user viewing bit equal to 1.

- If it were interpreted that the NNPFC SEI message defines a filter whose intended usage includes user viewing, it would not be possible to indicate in a future VSEI amendment that a filter is not suitable for user viewing.

## Proposed specification text changes

nnpfc_purpose indicates the purpose of the NNPF as specified in Table 20.

The value of nnpfc_purpose shall be in the range of 0 to 63127, inclusive, in bitstreams conforming to this edition of this document. Values of 64128 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64128 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| nnpfc_purpose = = 0 | May be used as determined by the application |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x01 ) = = 0 | No general visual quality improvement |
| ( nnpfc_purpose & 0x01 ) != 0 | With general visual quality improvement |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x02 ) = = 0 | No chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x02 ) != 0 | With chroma upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x04 ) = = 0 | No resolution upsampling (increasing the width or height) |
| ( nnpfc_purpose & 0x04 ) != 0 | With resolution upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x08 ) = = 0 | No picture rate upsampling |
| ( nnpfc_purpose & 0x08 ) != 0 | With picture rate upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x10 ) = = 0 | No bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| ( nnpfc_purpose & 0x10 ) != 0 | With bit depth upsampling |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x20 ) = = 0 | No colourization (from the 4:0:0 chroma format to the 4:2:0, 4:2:2, or 4:4:4 chroma format) |
| ( nnpfc_purpose & 0x20 ) != 0 | With colourization |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x40 ) = = 0 | The intended usages of the NNPF do not include user viewing. |
| ( nnpfc_purpose & 0x40 ) != 0 | The intended usages of the NNPF include user viewing. |

NOTE 1– When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value.

## Example how machine analysis usage can be added in a future VSEI version

Table 20 – Definition of nnpfc_purpose

| Value | Interpretation |
| --- | --- |
| … | … |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x40 ) = = 0 | The intended usages of the NNPF do not include user viewing. |
| ( nnpfc_purpose & 0x40 ) != 0 | The intended usages of the NNPF include user viewing. |
| nnpfc_purpose > 0 && ( nnpfc_purpose & 0x80 ) = = 0 | The intended usages of the NNPF do not include machine analysis. |
| ( nnpfc_purpose & 0x80 ) != 0 | The intended usages of the NNPF include machine analysis. |

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| nnpfc_purpose | u(16) |
| nnpfc_id | ue(v) |
| … |  |
| if( ( nnpfc_purpose & 0x80 ) != 0 ) { |  |
| // syntax elements further characterizing machine analysis |  |
| } |  |
| … |  |

# Background

Some neural-network post-processing filters may have been trained to improve machine analysis precision. Such filters may cause blurring of the background, changing of the colour mapping, and overemphasizing object edges, for instance.

Our work [3] studied neural-network post-filters aimed at improving machine analysis precision. We trained a task-agnostic enhancement (TAE) post-filter using a loss function combining both mean squared error (MSE) and a generic machine task loss. The trained post-filter was applied on images decoded by VVC. The TAE-filtered images were found superior in object detection and instance segmentation performance when compared to both the decoded images as such and the post-filtered images resulting from a post-filter trained with MSE only. Figure 1 reproduces an example from [3], where a decoded picture appears on top-left, the respective TAE-filtered picture on the top-right. It can be observed that the background is flatter and the colour mapping of the dogs appears smoother in the TAE-filtered video.

Figure 1. Examples of a decoded picture (top-left), the respective post-filtered picture (top-right), and their instance segmentation results.

JVET-AB0274 [4] includes another example of neural-network post-filters trained to improve machine analysis precision. JVET-AB0274 presents a VVC-based system targeted at machine analysis where video is pre-processed by downsampling prior to VVC-compliant encoding. Upsampling is performed as post-processing subsequent to VVC decoding. JVET-AB0274 uses several alternative pairs of neural networks for upsampling and downsampling, where some of the pre- and post-processing filters are jointly trained using a machine analysis loss.

# References

- M. M. Hannuksela, F. Cricri, J. I. Ahonen, H. Zhang, “AHG9/AHG15: On the NNPFC SEI message for machine analysis”, JVET-AC0076.

- C. Lin, Y.-K. Wang, K. Zhang, Y. Li, J. Li, L. Zhang, “AHG9: On additional NNPF purposes”, JVET-AC0136.

- J. I. Ahonen, R. G. Youvalari, N. Le, H. Zhang, F. Cricri, H. Rezazadegan Tavakoli, M. M. Hannuksela and E. Rahtu, “Learned Enhancement Filters for Image Coding for Machines”, IEEE International Symposium on Multimedia, Nov. – Dec. 2021.

- S. Wang, B. Li, Z. Wang, Y. Ye, S. Wang, “Encoder-only algorithms in Alibaba and City University of Hong Kong’s response to the Video Coding for Machines CfP”, JVET-AB0274.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).