| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0076-v3 |
| --- | --- |

| Title: | EE2-1.20a/b: Combination of EE2-1.10, EE2-1.11 and EE2-1.14 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang Fan Wang Yue Yu Haoping Yu Dong Wang | Tel: Email: | zhanglai@oppo.com wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of combination tests EE2-1.20a/b. EE2-1.20a is a combination test of EE2-1.10 and EE2-1.11. EE2-1.20b is a combination test of EE2-1.10, EE2-1.11 and EE2-1.14.

On top of ECM-8.0, simulation results are reported as follows:

EE2-1.20a

AI { -0.31% Y, -0.28% U, -0.32% V, 100% EncT, 100% DecT }

RA { -0.11% Y, -0.08% U, -0.12% V, xx% EncT, xx% DecT }

EE2-1.20b

AI { -0.69% Y, -0.72% U, -0.73% V, 100% EncT, 103% DecT }

RA { -0.25% Y, -0.25% U, -0.23% V, xx% EncT, xx% DecT }

Introduction

In EE2-1.10[1], a multi-candidate IntraTMP method is investigated. In the proposed method, a candidate list is constructed with the candidate BVs ranked in ascending order of their template matching costs, and the index of the selected candidate is signaled in the bit-stream.

In EE2-1.11[2], an IntraTMP fusion method is investigated. The proposed method derives multiple IntraTMP matched blocks and then fuses them to produce a better overall prediction.

In EE2-1.14[3], modifications to the IntraTMP search range are investigated. EE2-1.14a proposed to extend the adjacent search area of IntraTMP and EE2-1.14b proposed to enlarge the search range for small blocks.

Proposed method

The multi-candidate IntraTMP method introduces a block level syntax element, intra_tmp_idx, which indicates the index of the selected candidate. The IntraTMP fusion method introduces a block level syntax element, intra_tmp_fusion_flag, which indicates whether the intraTMP fusion method is used for the current block. In this combination test, if intra_tmp_fusion_flag is true, an intra_tmp_fusion_idx is signalled to indicate the index of the selected candidate set for the fusion process; if intra_tmp_fusion_flag is false, the intra_tmp_idx flag is signalled then for Multi-candidate IntraTMP.

The syntax change is as follows:

…

intra_tmp_flag

if(intra_tmp_flag) {

intra_tmp _fusion_flag

if(intra_tmp_fusion_flag) {

intra_tmp_fusion_idx

} else{

intra_tmp_idx

}

}

…

Specifically, the searching process is reused for Multi-candidate IntraTMP method and IntraTMP fusion method. A candidate list consisting of up to 19 BVs is constructed. The intra_tmp_idx, ranging from 0 to 18, is used to indicate a specific BV. The intra_tmp_fusion_idx, ranging from 0 to 2, is used to indicate one of the three candidate sets {BV0 to BV4}, {BV5 to BV9}, {BV10 to BV14}. To reduce complexity, the case of fusing a matched block with an intra predictor is removed.

In EE2-1.20b, EE2-1.20a is combined with EE2-1.14c.

Simulation Results

EE2-1.20a/b were implemented on top of the ECM-8.0 [4] software and tested based on the common test condition as specified in the Exploration Experiments 2 document (JVET-AC2024 [5]). The test results are summarized below.

Table 1. EE2-1.20a on top of ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.00% | -0.02% | 99% | 99% |
| Class A2 | -0.41% | -0.42% | -0.46% | 100% | 101% |
| Class B | -0.37% | -0.35% | -0.33% | 100% | 100% |
| Class C | -0.12% | -0.09% | -0.20% | 100% | 100% |
| Class E | -0.65% | -0.56% | -0.61% | 101% | 103% |
| Overall | -0.31% | -0.28% | -0.32% | 100% | 100% |
| Class D | -0.07% | -0.17% | 0.07% | 101% | 100% |
| Class F | 0.03% | -0.21% | 0.17% | 101% | 102% |
| Class TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.19% | -0.03% |  |  |
| Class A2 | -0.16% | -0.14% | -0.09% |  |  |
| Class B | -0.18% | -0.17% | -0.23% |  |  |
| Class C | -0.05% | -0.12% | -0.08% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.11% | -0.08% | -0.12% |  |  |
| Class D | 0.03% | 0.11% | -0.20% |  |  |
| Class F | -0.07% | -0.53% | -0.31% |  |  |
| Class TGM |  |  |  |  |  |

Table 2. EE2-1.20b on top of ECM-8.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.10% | -0.07% | -0.06% | 99% | 101% |
| Class A2 | -0.93% | -1.02% | -1.08% | 97% | 102% |
| Class B | -0.75% | -0.79% | -0.77% | 100% | 103% |
| Class C | -0.31% | -0.37% | -0.37% | 101% | 103% |
| Class E | -1.45% | -1.44% | -1.49% | 101% | 105% |
| Overall | -0.69% | -0.72% | -0.73% | 100% | 103% |
| Class D | -0.53% | -0.85% | -0.75% | 101% | 102% |
| Class F | -0.18% | -0.13% | -0.08% | 100% | 104% |
| Class TGM |  |  |  |  |  |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.10% | -0.11% |  |  |
| Class A2 | -0.47% | -0.50% | -0.39% |  |  |
| Class B | -0.34% | -0.23% | -0.22% |  |  |
| Class C | -0.13% | -0.20% | -0.20% |  |  |
| Class E |  |  |  |  |  |
| Overall | -0.25% | -0.25% | -0.23% |  |  |
| Class D | -0.11% | -0.06% | -0.36% |  |  |
| Class F | -0.19% | -0.61% | -0.59% |  |  |
| Class TGM |  |  |  |  |  |

Conclusions

According to EE2-1.20a, the gain from EE2-1.10 and EE2-1.11 is additive. According to EE2-1.20b, the enlarged search range in EE2-1.14c boosts the combination of Multi-candidate IntraTMP method and the IntraTMP fusion method in EE2-1.20a.

Reference

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “EE2-1.10 multi-candidate IntraTMP”, JVET-AD0073, April 2023.

- L. Zhang, F. Wang, Y. Yu, H. Yu, D. Wang, “EE2-1.11: Intra template matching prediction fusion”, JVET-AD0072, April 2023.

- Y. Ma, J. Huo, H. Du, H. Zhang, Z. Zhang, W. Qiao, F. Yang (Xidian), F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang (OPPO), “EE2-1.14c combination test of EE2-1.14a and EE2-1.14b”, JVET-AD0075, April 2023.

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0

- V. Seregin, J. Chen, R. Chernyak, K. Naser, J. Ström, F. Wang, M. Winken, X. Xiu, K. Zhang, “Exploration Experiment on Enhanced Compression beyond VVC capability (EE2),” JVET-AC2024, January 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).