| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0081-v3 |
| --- | --- |

| Title: | Non-EE2: An extrapolation filter-based intra prediction mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Yue Yu Haoping Yu Dong Wang | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes an extrapolation filter-based Intra Prediction (EIP) mode. First, the extrapolation filter coefficients are obtained from the neighboring reconstructed pixels of the current block with a pre-determined template. Then, the extrapolation is processed position by position from top-left to bottom-right within the current block to form a prediction block. The proposed method was implemented on top of ECM-8.0 software. The simulation results are summarized as follows,

AI: -0.27%/-0.19%/-0.20% Y/U/V, 108%/101% EncT/DecT,

RA:

Introduction

In ECM-8.0[1], the convolutional cross-component model (CCCM) [2] is applied to predict chroma samples from reconstructed luma samples in a similar spirit as done by the current CCLM modes.

The convolutional 7-tap filter consists of a 5-tap spatial component of plus sign shape, a nonlinear term, and a bias term. The input to the spatial 5-tap component of the filter consists of a center (C) luma sample which is collocated with the chroma sample to be predicted and its above/north (N), below/south (S), left/west (W) and right/east (E) neighbors as illustrated below.

Figure-1. The spatial part of the convolutional filter.

The filter coefficients ci are calculated by minimising MSE between predicted and reconstructed chroma samples in the reference area. Figure-2 illustrates the reference area which consists of 6 lines of chroma samples above and left of the PU. The reference area extends one PU width to the right and one PU height below the PU boundaries. The area is adjusted to include only available samples. The extensions to the area shown in blue are needed to support the “side samples” of the plus-shaped spatial filter and are padded when in unavailable areas.

Figure-2. Reference area (with its paddings) used to derive the filter coefficients.

The MSE minimization is performed by calculating the autocorrelation matrix for the luma input and a cross-correlation vector between the luma input and chroma output. The autocorrelation matrix is LDL decomposed and the final filter coefficients are calculated using back-substitution. The process follows roughly the calculation of the ALF filter coefficients in ECM, however, LDL decomposition was chosen instead of Cholesky decomposition to avoid using square root operations.

Proposed method

The proposed extrapolation filter-based intra prediction is processed in two steps. First, the extrapolation filter coefficients are obtained from the neighboring reconstructed pixels of the current block with a pre-determined template. Second, the extrapolation generates a predicted value position by position from top-left to bottom-right within the current block.

Searching mean, min, and max value

Similar to CCCM mode, a mean value should be removed when feeding the inputs to the EIP filter. The value of the DC mode for the current block is used as a mean value for EIP prediction. The min and max value are searched from reconstructed pixels in the reconstructed area with thirteen columns and thirteen rows.

Calculation of filter coefficients

Three types of reconstructed areas and three filter shapes are proposed, as shown in Figure-3. When the current block uses the proposed EIP mode for prediction, the decoder decodes the relevant syntax elements to determine the selected type of reconstructed area and filter shape for the current block.

Figure-3. The defined three types of reconstructed areas include thirteen columns or rows of reconstructed pixels.

Figure-4. The defined three types of filter shapes have fifteen inputs and generate one output.

The selected filter slides in the selected reconstructed area with a one-pixel step to collect input samples and output samples of EIP. The auto-correlation matrix and cross-correlation vector are constructed while removing the mean value from input samples and output samples. Then, the EIP coefficients are obtained by the same method in CCCM.

Prediction of current block

The EIP mode makes predictions for the current block position by position, as shown in Figure-5.

For the position located at top-left of the current block, the inputs to the EIP filter are reconstructed samples.

For the positions located along the boundaries of the current block, partial inputs to the EIP filter are reference samples, and partial inputs to the EIP filter are previously predicted samples.

For other positions in the current block, the inputs to the EIP filter are previously predicted samples.

| All inputs to EIP are reconstructed samples. | Partial inputs are reconstructed samples, partial inputs are predicted samples. | All inputs to EIP area predicted samples. |
| --- | --- | --- |

Figure-5. Examples of prediction for different positions in the current block

To reduce the prediction error, the searched min and max values are applied to restrict the output range of each predicted value,

predx,y=clip(min,max,(i=0n(ci×(tx-xoffset, y-yoffset-mean)))+mean)

predx,y is the predicted value at (x, y) in the current block,

min,  max are searched min and max values from the thirteen reconstructed columns and rows,

ci is the ith coefficient of the derived EIP filter,

tx-xoffset, y-yoffset is reconstructed or predicted value used for the current position’s prediction,

mean is a value calculated by the DC prediction mode.

Experimental Results

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Proposed method over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.16% | -0.10% | -0.04% | 107% | 100% |  |  |  |  |  |
| Class A2 | -0.25% | -0.16% | -0.18% | 107% | 101% |  |  |  |  |  |
| Class B | -0.22% | -0.19% | -0.12% | 109% | 102% | -0.08% | -0.04% | -0.07% | 100% | 100% |
| Class C | -0.25% | -0.13% | -0.23% | 107% | 101% | -0.08% | -0.06% | 0.01% | 101% | 101% |
| Class E | -0.53% | -0.40% | -0.48% | 109% | 101% |  |  |  |  |  |
| Overall | -0.27% | -0.19% | -0.20% | 108% | 101% |  |  |  |  |  |
| Class D | -0.14% | -0.19% | -0.09% | 107% | 101% | -0.11% | 0.19% | -0.10% | 101% | 99% |
| Class F | -0.10% | -0.04% | 0.03% | 106% | 102% | -0.07% | -0.29% | -0.28% | 100% | 100% |

Conclusions

The proposed method achieves -0.27%/-0.19%/-0.20% all-intra coding performance changes against ECM-8.0 with reasonable encoding and decoding time increase. It is proposed to further study in EE.

References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0.

- P. Astola, et, al., “EE2-1.1a: Convolutional cross-component intra prediction model”, JVET-AA0057, July, 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).