| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0082-v2 |
| --- | --- |

| Title: | [AHG7] A configuration option for TMRL mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Luhang Xu Jonathan Gan Yue Yu Haoping Yu Dong Wang | Tel: Email: | xuluhang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes a configuration option for the template-based multiple reference line (TMRL) intra-prediction mode. The configuration option is added on top of ECM-8.0 reference software. The simulation results of TMRL tool-off tests are summarized as follow,

AI: 0.11%/0.04%/0.07%, Y/U/V

RA:

Introduction

In ECM-8.0[1], the TMRL mode [2] constructs a candidate list by selecting 50 best template-matched combinations of intra prediction mode and extended reference line. The decoder decodes a TMRL candidate index and determines the intra prediction mode and reference line for making the current block’s intra prediction.

This proposal proposes a configuration option for TMRL, which was agreed in the AHG7 discussions on adding configuration options for all new coding tools adopted in ECM.

Experimental Results

The simulation results of the proposed configuration tool-off option are shown as follows, The simulation time is not accurate.

|  | All Intra Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | EE2-1.21a over ECM-8.0 |  |  |  |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | 0.06% | 0.00% | 92% | 99% |  |  |  |  |  |
| Class A2 | 0.10% | 0.02% | 0.12% | 91% | 96% |  |  |  |  |  |
| Class B | 0.09% | -0.01% | 0.04% | 91% | 96% | 0.06% | -0.13% | -0.01% | 108% | 106% |
| Class C | 0.14% | 0.08% | 0.09% | 90% | 92% | 0.05% | -0.09% | -0.12% | 105% | 99% |
| Class E | 0.16% | 0.09% | 0.10% | 90% | 93% |  |  |  |  |  |
| Overall | 0.11% | 0.04% | 0.07% | 91% | 95% |  |  |  |  |  |
| Class D | 0.09% | -0.04% | -0.11% | 91% | 96% | 0.09% | 0.33% | 0.06% | 106% | 104% |
| Class F | 0.19% | -0.06% | 0.09% | 88% | 98% | 0.00% | -0.57% | -0.36% | 110% | 101% |

References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0.

- L. Xu, et. al., “EE2-1.12: Combination of EE2-1.10 and EE2-1.11,” JVET-AB0157, October 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).