| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0083-v1 |
| --- | --- |

| Title: | Non-EE2: Improvement over IBC-LIC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhihuang Xie Yue Yu Haoping Yu Dong Wang | Tel: Email: | xiezhihuang@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

In this contribution, multiple new modes related to IBC-LIC are proposed to further improve the coding performance of IBC-LIC. In ECM8.0, IBC-LIC derives the linear model from both the top and left templates. Two new modes are proposed to allow only using the top or the left template, respectively. In addition, it is proposed to allow IBC-LIC to have multiple linear models in one CU, which is similar to MMLM.

The proposed method was implemented on top of ECM8.0 and the summarized test results are shown below:

AI: 		-0.50% (Y), -0.67% (U), -0.54% (V), 101% (EncT), 99% (DecT) for class F

-0.11% (Y), -0.10% (U), -0.07% (V), 102% (EncT), 101% (DecT) for class TGM

RA:	-0.xx% (Y), -0.xx% (U), -0.xx% (V), 100% (EncT), 100% (DecT) for class F

-0.xx% (Y), -0.xx% (U), -0.xx% (V), 100% (EncT), 100% (DecT) for class TGM

Introduction

In ECM8.0, Intra block copy with local illumination compensation (IBC-LIC)[1] is a coding tool which compensates the local illumination variation within a picture between the CU coded with IBC and its prediction block with a linear model. The parameters of the linear model are derived using the same method as LIC for inter prediction except that the reference template is generated using a block vector in IBC-LIC.

IBC-LIC can be applied to IBC AMVP mode and IBC merge mode. For IBC AMVP mode, an IBC-LIC flag is signalled to indicate the use of IBC-LIC. For IBC merge mode, the IBC-LIC flag is inferred from the merge candidate.

In ECM8.0, IBC-LIC only applies to the CU whose block size is larger than or equal to 32 and smaller than or equal to 256.

Proposed method

It is proposed to add three more modes for IBC-LIC to further improve the coding performance.

The first two modes are related to using different template shapes in the parameters’ derivation. In the current ECM, IBC-LIC uses both the top and left templates to derive the parameters. It is proposed to allow IBC-LIC to use the top-only, left-only, or top and left templates for deriving the model parameters.

In addition, it is proposed to extend the MMLM to IBC-LIC, which allows IBC-LIC to have two linear models in one CU.

Finally, it is proposed to remove the large block-size constraint for IBC-LIC. In this contribution, IBC-LIC and the proposed additional modes can be applied to the CU whose block size is larger than 32.

In the AMVP mode, the signalling of the proposed method is summarized in Table.1.

Table.1 IBC-LIC signalling in the proposed method

| Value | Mode |
| --- | --- |
| 0 | Not IBC-LIC |
| 100 | Default IBC-LIC |
| 101 | Multi-models IBC-LIC |
| 110 | Top-only IBC-LIC |
| 111 | Left-only IBC-LIC |

Experimental results

The proposed method is implemented on top of ECM-8.0[2] and tested under the common test conditions[3].

Table.2 Coding performance of the proposed method

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.50% | -0.67% | -0.54% | 101% | 99% |
| Class TGM | -0.11% | -0.10% | -0.07% | 102% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F |  |  |  |  |  |
| Class TGM |  |  |  |  |  |

Conclusion

In this contribution, it is proposed to extend IBC-LIC with different template-shape and multiple-models to further improve the coding performance. Considering the interesting coding gains and a minor encoding time increase, it is suggested to further study the proposed method in the new EE2.

Reference

[1] Y. Wang, K. Zhang, L. Zhang, N. Zhang, C. Ma, X. Xiu, W. Chen, H. Jhu, C. Kuo, N. Yan, X. Wang. EE2-3.6: IBC-CIIP, IBC-GPM, and IBC-LIC. JVET-AC0112, 29th Meeting, by teleconference, 11-20 Jan 2023.

[2] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).