| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0096 |
| --- | --- |

| Title: | EE2-3.4: Copy-Padding for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhangna.cynthia@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In IBC mode, the reference block cannot be overlapped with the current block. In test EE2-3.4a, copy-padding can be applied on the overlapped area. With copy-padding, an unreconstructed sample in the overlapped region can be padded by copying its prediction sample.

On top of the ECM-8.0, simulation results of test EE2-3.4a are reported as below:

AI: Class F 0.00%,100%,101%; Class TGM -0.22%,101%,101%

RA: Class F -0.08%,100%,101%; Class TGM -0.21%,101%,100%

On top of the ECM-8.0, simulation results of EE2-3.4b, which combines EE2-3-4a and EE2-3.3 presenting the temporal block vector prediction (TBVP), are reported as below:

AI: Class F 0.00%, 100%,101%; Class TGM -0.22%,101%,101%

RA: Class F -0.15%	,100%,99%; Class TGM -0.60%,101%,100%

# Introduction

In IBC mode, a block vector (BV) is used to indicate the displacement from the current block to a reference block, which must be fully reconstructed inside the current picture. The reference block cannot be overlapped with the current block.

# Proposed methods

In this contribution, copy-padding can be applied on the overlapped area. With copy-padding, an unreconstructed sample in the overlapped region can be padded by copying its prediction sample, as shown in Figure 1, in an equation way,

P’(x,y) = P(x+BVx, y+BVy)

wherein P’(x,y) is a padded sample at position (x, y), P(x+BVx, y+BVy) is a prediction sample, and (BVx, BVy) is the BV of current block.

Copy-padding is performed only if the horizontal BV component is smaller than or equal to 0 and the vertical BV component is smaller than or equal to 0.

Figure 1. Unreconstructed samples (shaded) in the reference block are padded by copying their prediction samples.

# Experimental Results

The proposed method has been implemented on top of the ECM-8.0 [1]. Testing results of EE2-3.4a and EE2-3.4b which is a combination of EE2-3.4a and EE2-3.3 of TBVP in [2] under AI/RA configurations are summarized in Table 1 and Table 2. The simulations were performed following the common test conditions [3]. More detailed results can be found in the attached spreadsheets.

Table 1: Coding performance of the proposed method.

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | -0.09% | 0.09% | 100% | 101% |
| Class TGM | -0.22% | -0.21% | -0.16% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.08% | -0.58% | -0.45% | 100% | 101% |
| Class TGM | -0.21% | -0.10% | -0.17% | 101% | 100% |

Table 2: Coding performance of the proposed method combined with TBVP in [2].

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.00% | -0.09% | 0.09% | 100% | 101% |
| Class TGM | -0.22% | -0.21% | -0.16% | 101% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.15% | -0.59% | -0.53% | 100% | 99% |
| Class TGM | -0.60% | -0.41% | -0.51% | 101% | 100% |

- Conclusions

In test EE2-3.4a, IBC prediction can be copy-padded for an overlapped area to improve the efficiency of IBC. The proposed method can bring some coding gain with almost no coding time change. Test EE2-3.4b shows that the gains of EE2-3.3 and EE2-3.4a are additive. It is recommended to adopt test EE2-3.4b into the next version of ECM.

# References

[1] https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0.

[2] N. Zhang, K. Zhang, L. Zhang, “EE2-3.3: Temporal block vector prediction,” JVET-AD0095, Apr. 2023.

[3] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).