| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0123-v1 |
| --- | --- |

| Title: | EE2-5.1: Reference frame padding for GDR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tangi Poirier Saurabh Puri | Email: | tangi.poirier@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE Test 5.1 which proposes, in case of Gradual Decoding Refresh (GDR), to pad the dirty area using samples from the clean area instead of setting unavailable samples to 2BD-1 for reference pictures used by CUs in the clean (refreshed) area.

Simulations were conducted with ECM-8.0 [1] commit dec5cc28 (15 March 2023), including fix for GDR (#40), fix for OBMC and fix for Virtual Boundary, under low-delay B configuration specified by AHG 7 [2]. The results show a BD-rate gain of X.X% overall, 0.2% in class C and 0.4% in class E.

# Introduction

A GDR implementation has been adopted at the 26th JVET meeting [3], CUs in the clean area use reference pictures where all reconstructed samples in the dirty area are set to 2BD-1, where BD is the bit depth.

At the 27th JVET meeting, the reference frame padding has been proposed in [4].

# Proposed solution

In this contribution, it is proposed to pad the dirty area using samples from the clean area for reference pictures used by CUs located in the clean area, as shown in Figure 1

Figure 1: proposed padding of the dirty area

# Experimental results

The proposed padding has been implemented on ECM-8.0, commit dec5cc28 (15 March 2023) and simulations were conducted over JVET test sequences using a low-delay B configuration with the following settings:

- The first picture is coded as an IDR picture,

- A GDR picture is inserted every second,

- The GOP interval (distance between GDR picture and associated recovery point picture) is set to the frame rate,

- Each inter picture refers to up to 4 immediately preceding pictures in decoding order,

- The same QP is used for all pictures.

The results are shown in Table 1. The overall BD rates gain of the proposed method are X.X%, 0.2% in class C and 0.4% in class E.

Table 1: Simulation results of proposed method in LDB

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-7.0-IDG |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class C | -0.16% | 0.11% | 0.08% | 101% | 101% |
| Class E | -0.41% | -0.45% | -0.40% | 101% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class D | -0.24% | -0.22% | -0.11% | 99% | 98% |
| Class F | -0.75% | -0.67% | -0.65% | 100% | 100% |

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- G. Martin-Cocher, S. Puri, T. Poirier, K. Naser, J. Xu, D. Nicholson, M. Sychev, L. Wang, S. Liu, W. Yang, J.M. Tiesse, and M. Karczewicz, “AHG-7, LLCC Scenarios and baseline configurations”, JVTE -Y0043, Jan. 2022.

- S. Hong, L. Wang, K. Panusopone, “AHG 7: GDR Implementation for ECM 4.0”, JVET-Z0118, Apr. 2022.

- T. Poirier, F. Aumont, “AHG 7: Reference frame padding for GDR”, JVET-AC0180, Jan. 2023

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).