| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0148-v1 |
| --- | --- |

| Title: | Non-EE2: SGPM combined with multiple IntraTMP predictors |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Cheng Fang, Shuang Peng, Dong Jiang，Jucai Lin, Xue Zhang, Heng Jin, Xiameng Shi, Fang Ye | Tel: Email: | jiang_dong@dahuatech.com lin_jucai@dahuatech.com |
| Source: | Dahua (ZheJiang Dahua Technology CO.,LTD.) |  |  |

_____________________________

- Abstract

This contribution introduces multiple IntraTMP predictors to the SGPM design. In this method, the two geometric blocks can not only use intra prediction modes, but also use IntraTMP mode. The proposed method was implemented on top of ECM-8.0, it reportedly provides {Y, U, V} BD-rate reduction with encoder and decoder runtimes as follows:

AI: -0.05%, -0.06%, -0.05%, 102% (EncT), 107% (DecT)

Introduction

In ECM-7.0[1], the Spatial GPM method was adopted to intra prediction. SGPM consists of one partition mode and two associated intra prediction modes for two geometric blocks. To express the necessary partition and prediction information more efficiently in the bit-stream, a candidate list is employed and only the candidate index is signaled in the bit-stream. Each candidate in the list can derive a combination of one partition mode and two intra prediction modes. The length of the candidate list of combinations is set equal to 16.

In SGPM, there are 26 partition modes. For each partition mode, an IPM list is derived for each geometric block. The IPM list size is 3. The two intra prediction modes are selected from the IPM list.

A one line template is used to generate this candidate list.  For each possible combination of one partition mode and two intra prediction modes from the IPM list, a prediction is generated for the template with the partitioning weight extended to the template. These combinations are ranked in ascending order of their SATD between the prediction and reconstruction of the template.

In the last meeting, JVET-AC0097[2] proposed to introduce one IntraTMP poredictor to the geometric block which has more corresponding template pixels.

Proposed method

In this contribution, we introduce 3 IntraTMP predictors to the SGPM design. By using the IntraTMP searching and SAD ranking process, the best 3 BV are selected as predictors. Both of the geometric blocks can use the 3 IntraTMP predictors to do the prediction, an example is shown in Fig.1. However, if a geometric block whose corresponding template pixel number is smaller than the one quarter of the full template pixel number, it is forbidden to use the IntraTMP predictor.

SGPM combined with IntraTMP

The IntraTMP predictors only join the SGPM template rank process, so the candidate list length is still 16, and no additional syntax elements are required.

Experimental results

The method was implemented on top of ECM-8.0 and tested for AI configurations for class A to class F. The following experimental results were obtained. The encoding and decoding running times are asserted to be inaccurate since the encoding of the anchor and the tests were run on machines with different capabilities.

Results of the proposed method on top of ECM-8.0 under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.12% | 0.04% | 102% | 111% |
| Class A2 | -0.06% | -0.07% | -0.06% | 102% | 111% |
| Class B | -0.05% | -0.14% | -0.08% | 102% | 102% |
| Class C | -0.04% | -0.06% | -0.01% | 102% | 106% |
| Class E | -0.09% | -0.12% | -0.14% | 102% | 105% |
| Overall | -0.05% | -0.06% | -0.05% | 102% | 107% |
| Class D | 0.00% | 0.03% | 0.01% | 102% | 105% |
| Class F |  |  |  |  |  |

Conclusions

This contribution introduces multiple IntraTMP predictors to the SGPM design. The results show that the proposed method can bring more coding gains.

References

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- Cheng Fang, Shuang Peng, Dong Jiang, Jucai Lin, Xue Zhang, “Non-EE2: SGPM combined with IntraTMP”, JVET-AC0097, Jan. 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

Patent rights declaration(s)

Dahua Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).