| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0160-v2 |
| --- | --- |

| Title: | EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jianghao Jia, Yuantong Zhang, Han Zhu, Zhenzhong Chen Wuhan University Zizheng Liu, Xiaozhong Xu, Shan Liu Tencent | Email: | {jiajh2021, yuantongzhang, zhuhanlyx, zzchen}@whu.edu.cn {zizhengliu, xiaozhongxu, shanl} @tencent.com |
| Source: | Wuhan University, Tencent |  |  |

_____________________________

- Abstract

This contribution reports the EE results of JVET-AC0114, which presented a deep reference frame generation (DRF) method for VVC inter prediction enhancement. Based on VTM-11.0_NNVC-4.0, the proposed method achieves {RA: -4.32%/-10.72%/-10.07%, LDB: -3.16%/-10.40%/-7.60%} bitrate savings for Y/U/V components, respectively.

In the v2 version, the LDB overall results are fixed with ClassE included.

Introduction

In JVET-AC0114 [1], we proposed a deep reference frame generation method to enhance VVC inter prediction. The method was introduced into EE in the 29th JVET meeting. In this contribution, the EE results are reported. We use the same solution as in JVET-AC0114 and test the method based on the latest NNVC software VTM-11.0_NNVC-4.0.

Proposed Method

Framework

Fig. 1: The framework of the proposed DFR method

The framework of the proposed DRF method is shown in Fig. 1. In this method, two reconstructed frames selected from DPB, together with the QP maps, are fed into the deep reference frame generation network to synthesize a new frame. This generated frame will be inserted into Reference List0 and List1 with the same POC of the current encoding frame as an alternative reference. DRF can be used for RA and LDB configurations with some differences in network architecture. It should be noted that the ME process is not modified, and no extra information is signaled into the bitstream. Other details can be found in our previous work.

Network architecture

Fig. 2: The architecture of the DRF network

Fig. 2 shows the architecture of the proposed DFR network. The reconstructed frames and their corresponding QP maps are fed into a three-level hierarchical structure to extract features in different granularities. An additional module PFRB is employed to explore the relationship between the features further. Besides, we utilize the IFRNet to estimate the optical flows among the intermediate and two input frames. The estimated optical flows will also be down-sampled by a similar pyramid structure as the extracted features. The three levels of features and optical flows are synthesized by optical-flow-based calculation. Finally, the GridNet and ResBlocks will output the generated reference frame with the synthesized features as input.

Network information in training stage

Table 1: Network information for the NN-based video coding tool testing in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GTX 1080ti x 8 x 12GB |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 8 |
|  | Epoch | 200 |
|  | Batch size | 3Kx24 (RA) 4Kx18 (LDB) |
|  | Loss function | L1 |
|  | Training time (for 1 model) | 168h |
|  | Training data information | Vimeo (compressed) |
|  | Training configurations for generating compressed training data (if different to VTM CTC) | QP = 22, 27, 32, 37, 42 |
| Optional | Number of iterations |  |
|  | Patch size | 256x256 |
|  | Learning rate | 1e-4 |
|  | Learning rate update strategy |  |
|  | Optimizer | ADAM |
|  | Preprocessing |  |
|  | Mini-batch selection process |  |
|  | Training data update strategy |  |
|  | Other information |  |

Network information in inference stage

Table 2: Network information for the NN-based video coding tool testing in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment |  |
|  | GPU Type | CPU only |
|  | Framework | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 7797K (RA) 11512K (LDB) |
|  | Total Number of Parameters (All Models) | 19309K |
|  | Parameter Precision (Bits) | 32 |
|  | Memory Parameter (MB) | 29.8 (RA) 44.0 (LDB) |
|  | Multiply Accumulate (kMAC/pixel) | 986 (RA) 1192 (LDB) |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size |  |
|  | Patch size | 512x512 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information |  |

Results

The method was implemented on the VTM-11.0_NNVC-4.0. We conducted the compression experiments according to the JVET NNVC CTC [2]. Table 3 and Table 4 show the experimental results of the proposed DRF method compared to the anchor results under RA and LDB configurations, respectively.

Table 3: Performance of the proposed method (RA, full length)

|  | Random access Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 | -3.54% | -7.83% | -8.50% | -4.15% | -9.36% | -9.89% | 159% | 67960% |
| Class A2 | -4.73% | -12.01% | -7.82% | -5.48% | -12.94% | -8.06% | 150% | 63442% |
| Class B | -3.62% | -10.18% | -10.17% | -4.16% | -11.74% | -12.02% | 162% | 82099% |
| Class C | -5.46% | -12.59% | -12.80% | -6.87% | -12.67% | -13.97% | 125% | 48678% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -4.32% | -10.72% | -10.07% | -5.15% | -11.75% | -11.32% | 148% | 65312% |
| Class D | -7.15% | -12.47% | -13.83% | -6.52% | -12.96% | -13.61% | 114% | 32417% |
| Class F | -1.71% | -4.56% | -4.10% | -2.57% | -6.63% | -7.14% | 210% | 91133% |

Table 4: Performance of the proposed method (LDB, full length)

|  | Low delay B Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | BD-rate Over VTM-11.0_nnvc-4.0 |  |  |  |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -2.57% | -9.83% | -7.27% | -3.25% | -10.74% | -9.55% | 177% | 101502% |
| Class C | -3.35% | -14.36% | -11.93% | -4.63% | -13.91% | -9.29% | 142% | 58944% |
| Class E | -3.88% | -6.08% | -2.37% | -4.65% | -3.90% | -0.88% | 317% | 159610% |
| Overall | -3.16% | -10.40% | -7.60% | -4.06% | -10.09% | -7.30% | 190% | 94829% |
| Class D | -3.65% | -9.01% | -9.65% | -4.25% | -1.69% | -4.40% | 128% | 38049% |
| Class F | -1.37% | -7.89% | -7.69% | -1.85% | -10.67% | -12.14% | 240% | 132884% |

Conclusions

This contribution reports the EE results of JVET-AC0114, which presented a deep reference frame generation (DRF) method for VVC inter prediction enhancement. The proposed method achieves 4.32%/10.72%/10.07% and 3.16%/10.40%/7.60% coding efficiency improvements under RA and LDB configurations, respectively.

Reference

[1] J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AC0114, 29th Meeting, by teleconference, 11–20 January 2023.

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11–20 January 2023.

Patent rights declaration(s)

Wuhan University does not have any current or pending patent rights relating to the technology described in this contribution.

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).