| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0163-v1 |
| --- | --- |

| Title: | AHG11: Updating NNPFC and NNPFA SEI messages for the NNPF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Maria Santamaria Miska M. Hannuksela Francesco Cricri Ruiying Yang Nokia Technologies, Tampere, Finland | Email: | {maria.santamaria_gomez, miska.hannuksela, francesco.cricri, ruiying.yang}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This document describes the usage of the most recent NNPFC and NNPFA SEI messages for the NNPF in NNVC 4.0. It is reported that the overall coding gains are -X.XX% (Y), -X.XX%  (Cb), -X.XX% (Cr).

# Introduction

The content-adaptive neural network post filter (NNPF) adopted to NNVC 4.0, JVET-AC0055 [1], makes use of outdated NNPFC and NNPFA SEI messages, JVET-Z2044 [2]. Hence, this contribution studies the most recent version of those, which are described in JVET-AC2032 [3].

For this study, the latest NNPFC and NNPFA SEI messages have been implemented on top of NNVC 4.0, and the characteristics signalled within the NNPFC SEI message are used to correctly apply the post-filter at inference stage.

In JVET-AC0055, four base NNPFCs and one updated NNPFC (content-adaptation) were signalled. Also, an NNPFA was signalled per picture when the NNPF was to be activated on a particular picture. The NNPFA was present for pictures that would benefit from the NNPF (the SSD would lower compare to not using the NNPF). In contrast, in this new scenario, a single base NNPFC is signalled along with an NNPFC that describes the update. Additionally, now a single NNPFA is signalled to activate the NNPFA for the whole CLVS.

# Simulation results

The baseline code is the NNVC-4.0 and the evaluation is done in the JVET CTC RA configuration. The reported encoding and decoding times are not reliable.

The following table presents the BD-rates when the anchor is NNVC-4.0 and the NNPF is OFF.

|  | BD-rate Over NNVC-4.0 with NNPF OFF |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | -5,11 % | -20,60 % | -16,94 % | -3,55 % | -21,00 % | -16,77 % | 108 % | 8093 % | 97 % |
| Class C | -4,09 % | -19,40 % | -16,53 % | -3,20 % | -18,13 % | -16,40 % | 105 % | 7842 % | 98 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -5,17 % | -19,71 % | -18,84 % | -2,25 % | -18,30 % | -17,44 % | 106 % | 7897 % | 98 % |
| Class F | -1,16 % | -6,18 % | 2,50 % | -1,55 % | -12,60 % | -10,75 % | 112 % | 9978 % | 97 % |
| Class H |  |  |  |  |  |  |  |  |  |

The following table shows the BD-rates when the anchor is NNVC-4.0 and the NNPF is ON.

The losses come mostly from activating the NNPF for all pictures, regardless of negative impacts on the reconstruction quality.

|  | BD-rate Over NNVC-4.0 NNPF ON |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y-PSNR | U-PSNR | V-PSNR | Y-MSIM | U-MSIM | V-MSIM | EncT | DecT CPU | PSNR Overlap |
| Class A1 |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |
| Class B | 0,31 % | 0,39 % | 0,35 % | 0,06 % | 0,28 % | 0,36 % | 63 % | 35 % | 99 % |
| Class C | 0,03 % | 0,02 % | -0,07 % | 0,01 % | 0,20 % | 0,18 % | 70 % | 35 % | 100 % |
| Class E |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |
| Class D | -0,12 % | -0,19 % | -0,26 % | -0,25 % | -0,01 % | -0,03 % | 69 % | 35 % | 100 % |
| Class F | 0,45 % | 3,29 % | 14,73 % | 0,06 % | 4,59 % | 3,09 % | 52 % | 71 % | 99 % |
| Class H |  |  |  |  |  |  |  |  |  |

# Recommendation

Update the NNVC software to match with the latest NNPFC and NNPFA SEI messages and the method of applying the base and updated filters as described in this document.

# References

- M. Santamaria, R. Yang, F. Cricri, J. Lainema, H. Zhang, R. G. Youvalari, M. M. Hannuksela. EE1-1.11: Content-adaptive post-filter. JVET-AC0055. January 2023.

- M. M. Hannuksela, M. Santamaria, F. Cricri, E. B. Aksu, H. R. Tavakoli, T. Chujoh, Y. Yasugi, T. Ikai, S. McCarthy, A. Arora, T. Shao, P. Yin, T. Lu, F. Pu, W. Husak. AHG9: NN post-filter SEI. JVET-Z0244. April 2022.

- S. McCarthy, S. Deshpande, M. M. Hannuksela, Hendry, G. J. Sullian, Y.-K. Yang. Improvements under consideration for the neural network pos filter SEI messages. JVET-AC2032. January 2023.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).