| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0167-v1 |
| --- | --- |

| Title: | EE1-1.7-related: Combination test for NNVC based on filter set #0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Chang Liqiang Wang Xiaozhong Xu Shan Liu | Email: | renjiechang@tencent.com liqiangwang@tencent.com xiaozhongxu@tencent.com shanl@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the test results of joint test for NNVC based on JVET-AC-EE1-1.7. In this contribution, NN-based intra prediction, filter set #0, encoder optimization for filter set #0 and temporal filter for filter set #0 are all enabled to evaluate better performance for NNVC software. Specifically, the test base models for filter set #0 are from JVET-AC-EE1-1.7 test b and test c. Based on NNVC-4.0, the test results are shown in order of RA and AI configurations as follows.

Test a (Joint test based on the base model from EE1-1.7.b):

RA: -xx.xx%	-xx.xx%	-xx.xx% 	EncT: xxx%	DecT: xxxxx%

AI : -xx.xx%	-xx.xx%	-xx.xx% 	EncT: xxx%	DecT: xxxxx%

Test b (Jonit test based on the base model from EE1-1.7.c):

RA: -xx.xx%	-xx.xx%	-xx.xx%	EncT: xxx%	DecT: xxxxx%

AI : -xx.xx%	-xx.xx%	-xx.xx%	EncT: xxx%	DecT: xxxxx%

Introduction

In this contribution, two joint tests are performed to evaluate better performance for NNVC software based on the based models from JVET-AC-EE1-1.7 []. Specifically, several NN tools including NN-based intra prediction, filter set #0, encoder optimization for filter set #0 and temporal filter for filter set #0 are enabled. The used base models for these two tests are from JVET-AC-EE1-1.7.b and JVET-AC-EE1-1.7.c, respectively.

Proposed method

The inference details of joint test are described in Section 2.1, and the network information for filter set #0 in the inference stage is provided in Section 2.2.

Inference details

In Test a, SADL [] is adopted as the inference library and all the models are quantized into int16 precision. In Test b, both SADL and Libtorch are adopted as the inference libraries and only the inference for base model of filter set #0 is based on float-point precision. Besides, the models of temporal filter for filter set #0 is from filter set #1.

Network information in inference stage

| - Network Information for filter set #0 in Inference Stage |  |  |  |
| --- | --- | --- | --- |
|  | Test | Test a | Test b |
| Mandatory | HW environment: |  |  |
|  | GPU Type | CPU only |  |
|  | Framework: | SADL | Libtorch |
|  | Number of GPUs per Task | 0 |  |
|  | Number of Parameters (Each Model) | 1.9M | 1.64M |
|  | Total Number of Parameters (All Models) | 1.9M | 1.64M |
|  | Parameter Precision (Bits) | int16 | float |
|  | Memory Parameter (MB) | 3.8MB/model, 1 model in all | 6.8MB/model, 1 model in all |
|  | Multiply Accumulate (kMAC/pixel) | 485K (w/o block extension) 615K (w/ block extension) | 418K (w/o block extension) 529K (w/ block extension) |
|  | Calculation Method | block basis |  |
| Optional |  |  |  |
|  | Total Conv. Layers | 101 | 131 |
|  | Total FC Layers | 0 |  |
|  | Total Memory (MB) |  |  |
|  | Batch size: | 1 |  |
|  | Patch size | 144x144 |  |
|  | Changes to network configuration or weights required to generate rate points |  |  |
|  | Peak Memory Usage (Total) |  |  |
|  | Peak Memory Usage (per Model) |  |  |
|  | Border handling |  |  |
|  | Other information: |  |  |

Results

All the experiments are performed based on the common test conditions in []. The results of Test a and Test b are provided in Section 3.1 and Section 3.2, respectively. The average results can also be summarized as follows.

Test a:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -xx.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |
| AI | -x.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |

Test b:

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| RA | -xx.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |
| AI | -x.xx% | -xx.xx% | -xx.xx% | xxx% | xxxxx% |

Results of Test a

Table 1. Performance of Test a

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -12.50% | -24.68% | -24.49% | 153% | 33588% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -14.23% | -24.59% | -25.82% | 148% | 30557% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -11.20% | -20.98% | -23.25% | 213% | 19652% |
| Class E | -14.88% | -22.52% | -23.47% | 244% | 31406% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -10.88% | -19.80% | -23.01% | 202% | 18517% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Results of Test b

Table 2. Performance of Test b

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -12.85% | -27.05% | -26.62% | 153% | 35317% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -14.43% | -28.39% | -28.14% | 149% | 33287% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -11.75% | -24.24% | -26.53% | 222% | 27992% |
| Class E | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -11.06% | -24.03% | -26.17% | 213% | 30945% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Conclusions

In this contribution, two joint tests based on JVET-AC-EE1-1.7 is performed to evaluate better performance for NNVC software. In there two joint tests, NN-based intra prediction, filter set #0, encoder optimization for filter set #0 and temporal filter for filter set #0 are all enabled. Compared with NNVC-4.0 anchor, xx.xx% BD-rate luma gain is observed under RA configuration.

References

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

[] R. Chang, L. Wang, X. Xu, S. Liu. EE1-1.7: Optimization of training and network for NNVC filter set 0. JVET-AD0166, 30th Meeting, Antalya, TR, 21–28 April 2023.

[] F. Galpin, T. Dumas, P. Bordes, E. François. AHG11: Small Ad-hoc Deep-Learning Library (SADL) update. JVET-AB0086, 28th Meeting, Mainz, DE, 20–28 October 2022.

[] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11–20 January 2023.