| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0171-v1 |
| --- | --- |

| Title: | EE2-3.1: TMP using Reconstruction-Reordered for screen content coding (RR-TMP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) D. Ruiz Coll, O. Patino, P. Andrivon | Tel: Email: | duizcoll@ofinno.com, opatino@ofinno.com, pandrivon@ofinno.com |
| Source: | Ofinno, LLC |  |  |

_____________________________

# Abstract

This contribution reports the simulation results of EE2-Test-3.1 of TMP using Reconstruction-Reordered (RR-TMP) for screen content coding. In Test 3.1, RR-TMP finds the reference block position for the IntraTMP by best matching the current block's horizontal and vertical flipped templates with the reference block's template in a predefined search area of the intraTMP. The prediction block uses the flipped reference block according to the flip type. RR-TMP block vector (BV) and the flip type are recorded to be used by the following blocks as a BVP candidate in the IBC Merge/AMVP list. Simulation results based on ECM-8.0 are reported below:

AI: Class F 0.04%, -0.23%, -0.22%, 102%, 103%; Class TGM -0.10%, -0.10%, -0.05%, 102%, 104%

RA: Class F -0.07%, -0.66%, -0.28%, 100%, 100%; Class TGM -0.07%, -0.14%, -0.21%, 100%, 101%

# Introduction

This contribution reports the simulation results of EE2-Test-3.1 [1]. Since symmetry is often observed in screen content, a Reconstruction-Reordered IBC (RR-IBC) mode [2] was adopted in ECM-6.0 to improve the coding efficiency of IBC further. In RR-IBC, a motion shift is added to a block vector candidate obtained from a neighboring block according to a symmetric rule in case the neighboring block is coded with RR-IBC.

In IntraTMP, the decoder side's prediction block is obtained by matching the current block's L-shaped template with another block in a predefined search area. Using IntraTMP BV of neighboring blocks was proposed in JVET-AB0061 [3], which employs IntraTMP BV of the neighbor blocks for the IBC Merge/AMVP candidate list.

# Proposal

In EE2-Test-3.1, Reconstruction-Reordered TMP is proposed for screen content coding. When RR-TMP is applied, the encoder and decoder search for the most similar template to the flipped current template in a predefined area of the search region. Only the current CTU (R1) and a portion of the left region (R4) are explored for the horizontal flip type. Otherwise, only the current CTU (R1) and a part of the above region (R3) are explored in the vertical flip mode.

The dimensions of search regions in R3 and R4 regions (SR) are five times of block dimension (CbHeight, CbWidth) as it is determined for the regular IntraTMP, as shown in Figure 1.

The flip type is selected among the regular TMP (non-flip), horizontally flipped, and vertical flipped TMP modes according to the lowest TM cost. Consequently, no additional signaling is required. The corresponding block is flipped according to flip type, horizontal or vertical flip, and used as a prediction block.

Figure 1. RR-TMP search region for (a) horizontal and (b) vertical flip.

To better utilize the symmetry property, the RR-TMP BV is stored as a neighbor candidate in the IBC list of the following blocks. If a current IBC block selects the RR-TMP BV of neighboring blocks as a candidate, it is adjusted like it is done for the TMP BVs.

As in RR-IBC, a flip-aware BV adjustment is applied to refine the block vector candidate according to two flip types: horizontal and vertical. Figure 2 shows the flip-aware BV adjustment in RR-TMP for a horizontal and vertical flip. (xnbr,ynbr) and (xcur,ycur) represent the coordinates of the center sample of the neighboring block and the current block, respectively, BVnbr and BVcur denotes the BV of the neighboring block and the current block, respectively.

For horizontal flipping, in case the neighboring block is coded with a horizontal flip of RR-TMP, the horizontal component of BVcur is calculated by adding a motion shift to the horizontal component of BVnbr (denoted as BVhnbr), and the vertical component remains the same:

- BVhcur=2xnbr-xcur+BVhnbr

- BVvcur=BVvnbr

Similarly, in case the neighboring block is coded with a vertical flip, the vertical component of BVcur is calculated by adding a motion shift to the vertical component of BVnbr (denoted as BVvnbr), and the horizontal component remains invariant:

- BVhcur=BVhnbr

- BVvcur=2ynbr-ycur+BVvnbr

Figure 2. Illustration of RR-TMP BV adjustment for (a) horizontal flip and (b) vertical flip.

# Experimental Results

The proposed method is tested on top of ECM-8.0 software [4] under JVET ECM common test conditions [5] in AI and RA configurations. The following results were obtained:

Table I. Experimental result of the proposed method under AI configuration

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0.04% | -0.23% | -0.22% | 102% | 103% |
| Class TGM | -0.10% | -0.10% | -0.05% | 102% | 104% |

Table II. Experimental result of the proposed method under RA configuration

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-8.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.07% | -0.66% | -0.28% | 100% | 100% |
| Class TGM | -0.07% | -0.14% | -0.21% | 100% | 101% |

- References

- Jung-Kyung Lee, Damian Ruiz Coll, Vikas Warudkar "AHG12: TMP using Reconstruction-Reordered for screen content coding (RR-TMP)," JVET-AC0059, Jan. 2023.

- Zhipin Deng, Kai Zhang, Li Zhang, "EE2-3.2: Reconstruction-Reordered IBC for screen content coding," JVET-AA0070, July. 2022.

- Woong Lim, Donghyun Kim, Jongho Kim, Sung-Chang Lim, "EE2-3.2: Using block vector derived from IntraTMP for IBC," JVET-AB0061, Oct. 2022.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-8.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-Y2017, 25th meeting, by teleconference, Jan. 2022.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).