| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0184-v3 |
| --- | --- |

| Title: | Non-EE2: Removal of Division Operations |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wei Jia1, Kai Zhang1, Zhipin Deng2, Li Zhang1 18910 University Center Lane, San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | wei.jia@bytedance.com zhangkai.video@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In designs of CCLM slope adjustment and chroma fusion, division operations are involved to compute the average luma value. In this contribution, it is proposed to replace the division operations in CCLM slope adjustment and chroma fusion by a look-up table-based method. Simulation results based on ECM-8.0 are reported as below:

AI: {0.00%,	-0.05%,	0.00%; 78%, 100%};

RA: {-0.01%, -0.04%, 0.05%; 80%, 84%}.

# Introduction

In the current designs of CCLM slope adjustment and the chroma fusion method, the division operations are involved in the computation of the average luma value formulated as

yr. = S / N,

where yr. is the average luma value, N represents the number of all the available samples, and S is the sum of the luma values.

# Proposed method

It is proposed to replace division operations in the CCLM slope adjustment and chroma fusion by a table-based method, which is already used to avoid division operations in DIMD, TIMD and non-adjacent affine inheritance.

# Simulation results

The proposed method is implemented on top of ECM-8.0 [1]. Simulations are performed following the common test conditions of EE2 [2]. The simulation results compared to ECM-8.0 are shown in Table 1.

Table 1: Coding performance compared to ECM-8.0

|  | AI |  |  |  |  |  |  | RA |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y |  |  | U | V | EncT | DecT |  |
| Class A1 |  | -0.01% | -0.07% | 0.05% |  |  |  |  | -0.01% | 0.03% | 0.06% |  |  |  |
| Class A2 |  | 0.00% | -0.05% | 0.05% |  |  |  |  | -0.04% | -0.08% | 0.10% |  |  |  |
| Class B |  | 0.00% | -0.06% | 0.03% |  |  |  |  | 0.01% | -0.12% | -0.03% |  |  |  |
| Class C |  | -0.01% | 0.03% | -0.08% | 100% | 99% |  |  | -0.01% | 0.04% | 0.11% | 100% | 100% |  |
| Class E |  | 0.02% | -0.14% | -0.03% | 100% | 101% |  |  |  |  |  |  |  |  |
| Overall |  | 0.00% | -0.05% | 0.00% |  |  |  |  | -0.01% | -0.04% | 0.05% |  |  |  |
| Class D |  | 0.01% | -0.01% | -0.13% | 101% | 100% |  |  | 0.01% | 0.07% | 0.02% | 100% | 100% |  |
| Class F |  | 0.03% | -0.02% | 0.04% |  |  |  |  | 0.01% | -0.09% | -0.15% | 100% | 100% |  |
| Class TGM |  | 0.01% | 0.02% | 0.06% |  |  |  |  | -0.03% | 0.04% | -0.01% |  |  |  |

# Conclusions

In this contribution, it is proposed to replace division operations in CCLM slope adjustment and chroma fusion by a lookup table-based method. It is recommended to adopt the proposed method in ECM.

# Reference

- ECM-8.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-8.0.

- M. Karczewicz, and Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing,” Document of Joint Video Experts Team, JVET-Y2017, by teleconference, January 2022.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).