| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0189-v1 |
| --- | --- |

| Title: | EE1-related: A simplified NN-based RDO model for filter set #1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Junru Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution proposes a simplified filtering design for the NN-based rate-distortion optimization (RDO) process when filter set #1 is activated. In particular, a unified model is developed for the RDO process of intra and inter slices. It is asserted that the proposed method could achieve reduced encoder complexity and improved performance compared with the existing NN-based RDO design.

On top of NNNC-4.0 with NN tools enabled (abbreviated as NNVC-4.0-NnIntra-NnFilterSet1), the BD-rate and complexity changes of the proposed NN-based RDO method and existing method are reported as below:

Proposed NN-based RDO design (test is NN-based Intra + NN-based filter set #1 + proposed NN-based RDO):

RA: %, %, %, EncT %, DecT %

LDB: -0.24%, 0.46%, 0.81%, EncT 107%, DecT 100%

AI: -0.42%, -0.35%, -0.47%, EncT 114%, DecT 99%

NNVC-4.0 NN-based RDO design (test is NN-based Intra + NN-based filter set #1 + NN-based RDO of NNVC-4.0):

RA: %, %, %, EncT %, DecT %

LDB: -0.22%, 0.26%, 0.08%, EncT 111%, DecT 100%

AI: -0.26%, -0.12%, -0.28%, EncT 123%, DecT 103%

kMAC/pixel and parameter numbers of the proposed RDO model and existing models are reported as below:

Proposed RDO model: 31.2 kMAC/pixel, 112.6k parameters

NNVC-4.0 RDO models: 166.9 kMAC/pixel, 1080.1k parameters

Introduction

In NNVC, there is a gap between the reconstruction samples used for distortion calculation during the RDO and the ultimate reconstruction samples, since the latter are eventually filtered using the NN model. To bridge this gap, a NN filter is inserted into the RDO process for partitioning mode selection. Specifically, a NN filter can be applied on the reconstruction samples before comparing them with the original samples to calculate the distortion. With filter set #1, different NN-based RDO models are used for intra and inter slices.

Proposed method

To lower the storage as well as the encoder complexity of the NN-based RDO model, this contribution proposes a simplified and unified model for the RDO process of different types of slices. Given the simplified RDO model, the constraint of applying the NN filtering in RDO is relaxed slightly.

The proposed RDO model is inspired from the NN filtering design in EE1-1.6 [1]. Fig. 1. gives the architecture of the proposed NN filter for RDO, which comprises three types of basic blocks known as HeadBlock, BackboneBlock, and TailBlock. HeadBlock is responsible for extracting features from input. The proposed model takes reconstruction, prediction, slice type, and qp as input, therefore the number of input channels, i.e. Cin, is equal to 4. C stands for the basic number of feature maps and is set as 32. {C1, C2} are set as {80, 16}. The stride of convolution, i.e. S, is set as 2 to achieve feature down-sampling. Backbone of the proposed network containing a series of BackboneBlocks. N, the number of BackboneBlocks, is set as 4. In the end, there is a TailBlock to map the features from backbone to the final output.

Fig. 2. Architecture of the proposed RDO filter model.

Experimental results

The proposed simplified NN-based RDO model is integrated into NNVC-4.0 (filter set #1) [2] and tested according to the common test conditions defined in [3].

Performance comparison

On top of NNVC-4.0-NnIntra-NnFilterSet1, the BD-rate and complexity changes of the proposed NN-based RDO method and existing method are summarized in Tables below.

Compared with NNVC-4.0-NnIntra-NnFilterSet1, the proposed NN-based RDO method brings on average {%, %, %}, {-0.24%, 0.46%, 0.81%}, and {-0.42%, -0.35%, -0.47%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI.

Compared with NNVC-4.0-NnIntra-NnFilterSet1, the existing NN-based RDO method in NNVC-4.0 brings on average {%, %, %}, {-0.22%, 0.26%, 0.08%}, and {-0.26%, -0.12%, -0.28%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI.

Proposed RDO method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.14% | -0.02% | 0.00% | 105% | 99% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.36% | -0.07% | 0.09% | 107% | 100% |
| Class C | -0.43% | 0.15% | 0.09% | 105% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.50% | 0.26% | 0.19% | 106% | 104% |
| Class F | -0.23% | 0.05% | -0.13% | 103% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.21% | 0.51% | 1.11% | 107% | 100% |
| Class C | -0.24% | 0.24% | 0.80% | 106% | 100% |
| Class E | -0.27% | 0.65% | 0.30% | 107% | 101% |
| Overall | -0.24% | 0.46% | 0.81% | 107% | 100% |
| Class D | -0.35% | 0.69% | 1.25% | 107% | 99% |
| Class F | -0.14% | 0.46% | 0.42% | 104% | 102% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.21% | -0.28% | -0.21% | 112% | 100% |
| Class A2 | -0.45% | -0.54% | -0.84% | 113% | 98% |
| Class B | -0.40% | -0.43% | -0.76% | 112% | 97% |
| Class C | -0.52% | -0.21% | -0.20% | 121% | 102% |
| Class E | -0.54% | -0.31% | -0.26% | 113% | 101% |
| Overall | -0.42% | -0.35% | -0.47% | 114% | 99% |
| Class D | -0.51% | 0.08% | -0.03% | 114% | 100% |
| Class F | -0.35% | -0.27% | -0.24% | 107% | 101% |

Existing RDO method in NNVC-4.0

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.14% | -0.10% | -0.01% | 105% | 101% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0.33% | -0.01% | -0.15% | 110% | 103% |
| Class C | -0.41% | -0.09% | -0.37% | 108% | 101% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.46% | 0.01% | -0.05% | 112% | 105% |
| Class F | -0.29% | 0.03% | -0.20% | 104% | 97% |
|  | Low delay B Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.19% | 0.33% | 0.57% | 111% | 99% |
| Class C | -0.22% | 0.19% | 0.22% | 114% | 101% |
| Class E | -0.26% | 0.25% | -0.94% | 108% | 101% |
| Overall | -0.22% | 0.26% | 0.08% | 111% | 100% |
| Class D | -0.26% | -0.24% | 0.23% | 112% | 101% |
| Class F | -0.13% | 0.72% | 0.47% | 105% | 98% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-4.0-NnIntra-NnFilterSet1 |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.13% | -0.11% | -0.09% | 120% | 103% |
| Class A2 | -0.27% | -0.21% | -0.37% | 118% | 101% |
| Class B | -0.24% | -0.12% | -0.46% | 124% | 103% |
| Class C | -0.33% | -0.04% | -0.28% | 129% | 106% |
| Class E | -0.31% | -0.18% | -0.12% | 123% | 101% |
| Overall | -0.26% | -0.12% | -0.28% | 123% | 103% |
| Class D | -0.31% | -0.06% | 0.12% | 121% | 99% |
| Class F | -0.19% | -0.26% | -0.20% | 111% | 101% |

Comparison of kMAC/pixel and memory

|  | kMAC/pixel (frame basis) | Total Parameter Number | Total parameter memory (int16) |
| --- | --- | --- | --- |
| Proposed RDO model | 31.2 | 112.6k | 225.2k |
| NNVC-4.0 RDO models | 166.9 | 1080.1k | 2160.2k |

Conclusions

This contribution presents a simplified filtering design for the RDO process when filter set #1 is activated. The proposed filtering method shows a favorable trade-off in terms of the coding performance and complexity. It is suggested to adopt the proposed method into NNVC common software.

References

- Y. Li, K. Zhang, L. Zhang, EE1-1.6: In-Loop Filter with wide activation and large receptive field, JVET-AD0106, Antalya, TR, April 2023.

- https://vcgit.hhi.fraunhofer.de/jvet-ahg-nnvc/VVCSoftware_VTM/-/tree/NNVC-4.0

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AC2016, by teleconference, January 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).