| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, by Antalya, 21–28 April 2023 | Document: JVET-AD0253-v1 |
| --- | --- |

| Title: | Cross-check of JVET-AD0160 (EE1-2.1: Deep Reference Frame Generation for Inter Prediction Enhancement) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Nicola Giuliani Elena Alshina | Email: | nicola.giuliani@huawei.com elena.alshina@huawei.com |
| Source: | Huawei |  |  |

_____________________________

# Abstract

This document verifies the performance numbers (class D, in LDB and RA configurations) reported in JVET-AD0160, which reports the results of the contribution described in JVET-AC0114. In JVET-AC0114 a deep reference frame generation method for VVC inter prediction enhancement was presented. An

# Introduction

The proposal [1] describes a method which consists of generating an additional reference frame using two reconstructed frames from the decoded picture buffer together with the QP maps, and putting it to the reference picture lists, by which the prediction is enhanced. The additional reference frame is synthesized using a neural network that consists of a three-level hierarchical feature extractor and employs PFRB [2] and IFRNet [3].

# Results

The following tables show our experimental results for the cross-check. The numbers match perfectly for RA configuration and show only slight deviations for LDB configuration. The results were generated using a python 3.7 environment with torch 1.3.0.

## Crosscheck results on EE1-2.1

| Random access Main10 Over VTM-11.0_nnvc-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences |  | Y | U | V |
| Class D | BasketballPass | -7.08% | -15.78% | -14.50% |
|  | BQSquare | -10.24% | -6.89% | -13.48% |
|  | BlowingBubbles | -4.31% | -10.13% | -8.12% |
|  | RaceHorses | -6.97% | -17.06% | -19.22% |
|  | Average | -7.15% | -12.47% | -13.83% |

| Low delay B Main10 Over VTM-11.0_nnvc-4.0 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences |  | Y | U | V |
| Class D | BasketballPass | -4.83% | -11.27% | -12.23% |
|  | BQSquare | -3.78% | 6.77% | 0.2% |
|  | BlowingBubbles | -2.02% | -16.06% | -10.53% |
|  | RaceHorses | -4.05% | -15.34% | -16.43% |
|  | Average | -3.67% | -8.97% | -9.75% |

## Difference between crosscheck result and EE1-2.1(JVET-AD0160) result

| Random access Main10 Over EE1-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences |  | Y | U | V |
| Class D | BasketballPass | 0.00% | 0.00% | 0.00% |
|  | BQSquare | 0.00% | 0.00% | 0.00% |
|  | BlowingBubbles | 0.00% | 0.00% | 0.00% |
|  | RaceHorses | 0.00% | 0.00% | 0.00% |
|  | Average | 0.00% | 0.00% | 0.00% |

| Low delay B Main10 Over EE1-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Sequences |  | Y | U | V |
| Class D | BasketballPass | 0.01% | 0.90% | -0.64% |
|  | BQSquare | -0.09% | -0.23% | 0.06% |
|  | BlowingBubbles | 0.00% | -0.20% | 0.29% |
|  | RaceHorses | -0.03% | -0.56% | -0.14% |
|  | Average | -0.03% | -0.02% | -0.11% |

# Conclusions

The proposal shows very promising results. Test EE1-2.1 was cross-checked and the BD-rates mostly match with the results reported by the proponents in JVET-AD0160.

# References

[1] J. Jia, Y. Zhang, H. Zhu, Z. Chen, Z. Liu, X. Xu, S. Liu. AHG11: Deep Reference Frame Generation for Inter Prediction Enhancement. JVET-AC0114, 29th Meeting, by teleconference, 11–20 January 2023.

[2] P. Yi, Z. Wang, K. Jiang, J. Jiang, and J. Ma. Progressive fusion video super-resolution network via exploiting non-local spatio-temporal correlations. IEEE/CVF International Conference on Computer Vision, 2019, pp. 3106-3115.

[3] L. Kong, B. Jiang, D. Luo, W. Chu, X. Huang, Y. Tai, C. Wang, and J. Yang. IFRNet: Intermediate feature refine network for efficient frame interpolation. IEEE/CVF Conference on Computer Vision and Pattern Recognition, 2022, pp. 1959-1968.

[4] E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AC2016, 29th Meeting, by teleconference, 11–20 January 2023.