| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0334 |
| --- | --- |

| Title: | Crosscheck of JVET-AD0069 (EE1-1.8: On flipping of input and output of model in NNVC filter set0) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Renjie Chang | Email: | renjiechang@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

This contribution reports the inference crosscheck results for JVET-AD0069 (JVET-AC-EE1-1.8), which proposes flipping operations with encoder optimizations applied to the inputs and output of the NNVC filter set#0 neural network. In this report, two tests based on integer inference were crosschecked including test the results with EncNnlfOpt=1 and EncNnlfOpt=0. It is reported that, the full crosscheck results for RA configuration and LDB configuration can exactly match.

Introduction

The JVET-AC-EE1-1.8 described in JVET-AD0069 [] study the flipping operations with encoder optimizations on top of NNVC filter set #0, where NNVC-4.0 anchor (NnlfOption=0), NNVC-4.0 (NnlfOption=1 EncRdoOpt=0) and NNVC-4.0 (NnlfOption=1 EncRdoOpt=1) are used as anchors for comparison. In this crosscheck report, the performance of proposed method in [1] was tested.

Results

All the experiments are performed following the common test conditions in []. The inference framework is SADL with integer precision. The crosscheck results are shown in the following tables.

- Test 1.8.1

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 (NnlfOption=1_EncRdoOpt=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.10% | -0.11% | -0.13% | 112% | 107% |
| Class A2 | -0.18% | -0.55% | -0.44% | 117% | 105% |
| Class B | -0.15% | -0.26% | -0.52% | 111% | 106% |
| Class C | -0.20% | -0.52% | -0.39% | 109% | 104% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.36% | -0.39% | 112% | 105% |
| Class D | -0.19% | -0.58% | -0.55% | 112% | 105% |
| Class F | -0.08% | -0.35% | -0.36% | 102% | 97% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 (NnlfOption=1_EncRdoOpt=1) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.38% | -0.88% | -1.64% | 112% | 107% |
| Class C | -0.49% | -1.81% | -1.40% | 111% | 105% |
| Class E | -0.48% | -0.42% | -0.59% | 117% | 111% |
| Overall | -0.44% | -1.08% | -1.30% | 113% | 107% |
| Class D | -0.45% | -2.69% | -1.97% | 113% | 103% |
| Class F | -0.52% | -1.74% | -2.02% | 114% | 110% |

- Test 1.8.2

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 (NnlfOption=1_EncRdoOpt=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.07% | -0.09% | -0.13% | 115% | 107% |
| Class A2 | -0.18% | -0.50% | -0.39% | 119% | 107% |
| Class B | -0.17% | -0.29% | -0.52% | 100% | 95% |
| Class C | -0.18% | -0.47% | -0.42% | 101% | 95% |
| Class E |  |  |  |  |  |
| Overall | -0.16% | -0.34% | -0.39% | 107% | 100% |
| Class D | -0.21% | -0.74% | -0.58% | 104% | 96% |
| Class F | -0.13% | -0.38% | -0.37% | 107% | 108% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-4.0 (NnlfOption=1_EncRdoOpt=0) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
| Class B | -0.33% | -0.73% | -1.43% | 101% | #NUM! |
| Class C | -0.50% | -1.88% | -1.25% | 101% | 91% |
| Class E | -0.50% | -0.02% | -1.53% | 113% | 100% |
| Overall | -0.43% | -0.94% | -1.39% | 104% | #NUM! |
| Class D | -0.29% | -2.25% | -1.54% | 99% | 90% |
| Class F | -0.50% | -1.31% | -1.51% | 105% | 104% |

Reference

- Z. Xie, Y. Yu, H. Yu, D. Wang. EE1-1.8: On flipping of input and output of model in NNVC filter set0. JVET-AD0069, 30th Meeting, Antalya, TR, 21–28 April 2023.

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AC2016, by teleconference, January 2023.