| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 30th Meeting, Antalya, TR, 21–28 April 2023 | Document: JVET-AD0399-v2 |
| --- | --- |

| Title: | Results of expert viewing for the spatial scalability category of the VVC multilayer VT |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien RWTH Aachen University Aachen Germany | Tel: Email: | +49-241-8027867 wien@lfb.rwth-aachen.de |
| Source: | BoG |  |  |

_____________________________

# Abstract

This document reports the results of an on-site expert viewing performed at the 30th JVET meeting in Antalya.

The evaluation targets at the spatial scalability category of the test plan document JVET-AC2021, comparing the compression performance of dual layer coding with a downscaled base layer at scaling ratios 1.5 and 2 to single layer coding.

The results indicate competitive visual quality for the reconstructed output of the dual layer bitstreams. By tendency, gains appear to be observable at the lower bitrates while stronger impairments were noted at the higher bitrates. In no case, the differences exceeded the CIs.

It is recommended to take these results into account when setting up the set of four rate points for a formal evaluation.

# Introduction

This document reports the results of an on-site expert viewing performed at the 30th JVET meeting in Antalya. The evaluation targets at the spatial scalability category of the test plan document JVET-AC2021, comparing the compression performance of dual layer coding with a downscaled base layer at scaling ratios 1.5 and 2 to single layer coding.

# Test setup

## Logistics

The setup at the meeting site included a PC with a Decklink video board for HDMI connection and SSD drives capable of stable playout of the raw YUV data at the required frame rate.

| Test Site | On-site |
| --- | --- |
| Display, size, connection (resolution setting) | Samsung 65” S95B, HDMI (3840×2160), 10bit input |
| Viewing distance | 3 viewers sitting at 1.5H, 2 views standing at 1.6H |
| Viewing angle | ±75°, 90° (at screen center) |
| Total number of viewers | 19 (6 female, 13 male) |

Participants confirmed visual acuity and normal colour vision.

The setup with three viewers sitting and two viewers standing right behind them at about the same distance was chosen to maximize the number of collected scores per session. The viewers entered their scores into the test database after each viewing session. This process was repeated with four groups.

## Test sequences and quantizer settings

For evaluation, a subset of test sequences and quantizer settings listed in JVET-AC2021 were tested. The data was made available on the JVET ftp site ftp.ient.rwth-aachen.de in the directory ./ahg/verificationtests/JVET-AD_SVVC (regular JVET login). A subset of the available streams was selected to adapt the duration of the viewing activity to the available time slot. Three rate points for four sequences were selected for viewing single layer and DL1.5x, and DL2x. The rate points are stated in the table below.

Table 1 – Selected test sequences and QP settings for the viewing

|  | Single layer |  | Double layer 1.5x |  |  | Double layer 2x |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Sequence | QP SL | Rate | QP BL | QP EL | Rate1.5x | QP BL | QP EL | Rate2x |
| CorsairJoy | 35 | 1017.0208 | 32 | 36 | 945.4624 | 29 | 36 | 1032.136 |
| CorsairJoy | 38 | 699.1296 | 35 | 40 | 653.3176 | 32 | 40 | 689.916 |
| CorsairJoy | 42 | 429.348 | 39 | 44 | 395.7264 | 36 | 44 | 415.388 |
| DrivingPOV3 | 34 | 3737.2304 | 31 | 35 | 3528.3672 | 28 | 35 | 4045.7592 |
| DrivingPOV3 | 39 | 1776.404 | 36 | 40 | 1641.956 | 33 | 40 | 1807.6368 |
| DrivingPOV3 | 46 | 676.8048 | 43 | 47 | 628.6136 | 40 | 47 | 669.2056 |
| Marathon2 | 32 | 7015.072 | 29 | 35 | 6703.6232 | 26 | 35 | 7363.5928 |
| Marathon2 | 38 | 3089.8192 | 35 | 41 | 2835.5096 | 32 | 41 | 2944.4552 |
| Marathon2 | 46 | 978.62 | 43 | 49 | 870.2376 | 40 | 49 | 895.844 |
| Procession | 38 | 2435.9408 | 35 | 40 | 2261.288 | 32 | 40 | 2420.5088 |
| Procession | 42 | 1456.0512 | 39 | 44 | 1338.4344 | 36 | 44 | 1421.1904 |
| Procession | 46 | 845.5192 | 43 | 47 | 797.168 | 40 | 47 | 842.4336 |

## Test method and test design

The assessment was performed with the reconstructed UHD output layer (VTM decoder command line parameter “-p 1”). A single DCR test session with 42 BTCs was presented to the viewers. The duration of the test session was about 22 min.

### Degradation category rating

#### Session setup

The test sequences were evaluated using the 11-grade scale as specified in Rec. ITU-R BT.500-14[3], shown in Figure 1 below.

Figure 1: Meaning of the 11 grades numerical scale as specified in Rec. ITU-R BT.500-14 Table 2-4

The test session included 42 basic test cells (BTCs) showing the uncompressed video sequence and one processed video sequence. Each BTC was structured as followed (PVS denotes the processed video sequence under evaluation):

“Original” (1sec) – [uncompressed sequence] (10sec) – “A” (1sec) – [PVS] (10sec) – “Vote <N>” (5sec)

The session had a duration of approx. 15min. It included a stabilization phase and three trapping BTCs where uncompressed sequences were to be scored.

#### Training

For the training, a modified version of the training reported in JVET-AC0267 was applied. The method is briefly summarized as follows:

- Explain procedure and scoring scale. 
- Run test session (6 BTCs, covering examples of the impairment range) 
- Let viewers note their scores. 
- Let them read out their scores. If variation is larger than 3 on the grading scale, re-run BTC 
   - Viewers shall reconsider their scoring for that sequence (not enforced to change their mind)
   - No indications where to look allowed, viewers have to make it up by themselves

# Results and analysis

## Data processing

As a first step, the complete set of votes experts was screened for the trapping sequences. None of the viewers was excluded. As a second step, isolated outliers -considered to be obvious errors- were checked. A single score for a single viewer was removed.

## Subjective results

The measured MOS values of the reconstructed video on the 11-grade scale are plotted over the bit rate of the corresponding bitstream. The ±95% confidence intervals for the MOS values are indicated.

Figure 3: MOS-over-rate plots for the DCR experiment

# Discussion

The results indicate competitive visual quality for the reconstructed output of the dual layer bitstreams. By tendency, gains appear to be observable at the lower bitrates while stronger impairments were noted at the higher bitrates. In no case, the differences exceeded the CIs.

It is recommended to take these results into account when setting up the set of four rate points for a formal evaluation.

# References

- P. de Lagrange, W. Husak, M. Radosavljević, M. Wien, “Draft plan for subjective quality testing of FGC SEI message,” Doc. JVET-AC2022, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 5, 29th meeting, online, Jan. 2022.

- Recommendation ITU-T P.910 (2008), Subjective video quality assessment methods for multimedia applications.

- Recommendation ITU-R BT.500-14 (2019), Methodologies for the subjective assessment of the quality of television images.