| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva CH, 11–19 July 2023 | Document: JVET-AE0013-v1 |
| --- | --- |

| Title: | JVET AHG report: Film grain technologies (AHG13) |  |  |
| --- | --- | --- | --- |
| Status: | AHG report input to JVET |  |  |
| Purpose: | AHG report |  |  |
| Author(s) or Contact(s): | W. Husak M. Radosavljević D. Grois Y. He P. de Lagrange A. Segall A. Tourapis A. Duenas X. Meng W. Zhang | Email: | wjh@dolby.com milos.radosavljevic@interdigital.com Dan_Grois@comcast.com yonghe@qti.qualcomm.com philippe.delagrange@interdigital.com asegall@amazon.com atourapis@apple.com alberto.duenas@wbd.com xuewei.meng@disneystreaming.com wenhao.zhang@disneystreaming.com |
| Source: | AHG13 |  |  |

_____________________________

# Abstract

This document summarizes the activities of AHG13: Film grain technologies, between the 30th Meeting (Antalya TR, 20–28 April 2023) and the 31st meeting (Geneva CH, 11-19 July 2023).

# Mandates

At the 30th meeting of the ITU-T/ISO/IEC Joint Video Experts Team (JVET), an ad hoc group on Film grain technologies was established with the following mandates:

- Study the benefits and characteristics of film grain technologies, including autoregressive and frequency-filtering technologies.

- Study alternative film grain models and their associated documentation.

- In consultation with AHG4, study and define content characteristics and test conditions that are desirable for the study and testing of film grain technologies.

- Given the study of desirable content characteristics, solicit or create new test material for further determining the operational characteristics of, testing, and developing any related technologies.

- Study preprocessing and encoder technologies for determining values for FGC (Film Grain Characteristics) SEI message syntax elements.

- Identify potential need for additional film grain technology and signalling, if needed.

- Coordinate development of film grain technology software and configuration files.

- Coordinate with AG 5 on finalizing the draft plan for subjective quality testing of the FGC SEI message JVET-AD2022, and conduct preparations for such testing.

- Coordinate with AHG3 for software support of the FGC SEI message.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). A single e-mail related to AHG13 activity was sent to the JVET reflector during the AHG period.

# Discussion

Much of this AHG period, ISO/IEC 23002-9 CDTR was out for comment.  The document was issued on May 15.  The expected response date was July 9 – just before the 31st Meeting (Geneva).  The editors met on a roughly biweekly cadence throughout the period.

The discussions were related to content for film grain, external activities on film grain technologies and additional software being developed for film grain.  There are several companies that are independently exploring content.  The content is expected to be used for demonstrations and tests.  The following characteristics were enumerated for further study:

- video representative of what could be found on streaming platforms

- 10 or 15s length

- consistent grain / no scene cut if possible

  - perhaps cross-fades?

- balanced scene lighting (not everything dark or bright)

- decent resolution and video quality, so that video quality can be rated

- reasonable motion (not too fast, not too blurry, not fully still)

- reasonable spatial content (not empty sky, not packed with excessive texture)

- "normal" grain (not just blurry blobs or blocks, or just scratches)

- color / BW?

The group agreed the content should include a “ground truth”. However, it was recognized that the “de-noising” process could be controversial.  Several content repositories were identified that could be a source of open-source content.  In addition, some companies are exploring providing content they own and can assign the rights.

Two applications SDOs have opened or are exploring study items/missions on film grain.  3GPP has started a study item that is scheduled to be completed at the end of 2023.  DVB is also exploring opening a study mission on film grain.  There is also an expectation that other SDOs will follow suit and explore film grain.  The group discussed potential support for those groups.

There were two industry demonstrations with film grain during the AHG period.  The first was at Mile High Video 2003 (MHV2003) and the second was at DVB World.  The second supported the launch and discussion of the DVB study mission.

Finally, several additional software packages were discussed.  Several software packages were updated and some new features were added, such as an autoregressive parameter translator.

# Related contributions

Three contributions related to AHG13 was identified as of 07/10/2023.

- The one contribution was the AHG report:

  - JVET-AE0013 JVET AHG report: Film grain technologies (AHG13)

- Two other contributions were added but at present

  - JVET-AE0250 Film Grain Synthesis: Material Selection for Ground Truth Content Creation

  - JVET-AE0251 Evaluating Denoising methods in the context of Film Grain Analysis/Synthesis

## Contributions

There were two contributions uploaded other than the AHG report.

### JVET-AE0250 Film Grain Synthesis: Material Selection for Ground Truth Content Creation

This document introduces some of the selected material that will be used for the creation of new film grain content based on the ground truth method presented in contribution JVET-AD0369. The material consists of either newly captured content or scenes taken from the OpenMovie project “Tears of Steel”. An initial description of the material is included in this document.

JVET contribution JVET-AD0369 [1] suggested that for the purposes of evaluating and better designing techniques for or relating to film grain analysis and synthesis it would be better to create new “synthesized film grain noise” test material. Such content would also be associated with a noise-free version of the same content, referred to as the “ground truth content”. This version would also be the material used during the creation of the film grain versions during authoring.

There are multiple benefits with such an approach. By using the ground truth version and the different film grain instances one can better study the compression impact of different types of film grain noise added onto the same source as well as design and evaluate methods for improving the compressibility of such content. In addition, one can also use such material to better study, design, and optimize new techniques for denoising, quality evaluation, and film grain noise modeling/synthesis. This can be done through the fact that one can relate any study with the performance and behavior of the ground truth content and make accurate measurements about the characteristics and type of the added noise, which is not possible for material with pre-added noise.

Finding material for such purpose, which would be of high quality, initially of low amount of noise, interesting, have desirable characteristics and also copyright properties (i.e. content that can be easily shared and used for standardization development and research) can be quite difficult. In this document we introduce new content and include material from the OpenMovie project “Tears of Steel”, which we believe fulfil our requirements. Section 2 introduces the segments that we selected from the “Tears of Steel” movie, while Section 3 introduces the new Apple material.

### JVET-AE0251 Evaluating Denoising methods in the context of Film Grain Analysis/Synthesis

This contribution proposes a methodology for evaluating the performance and appropriateness of different video denoising techniques in the context of film grain modeling/synthesis for video compression. The results from performance such an evaluation could potentially be utilized for selecting a denoising system for further work on film grain modeling/synthesis within JVET or other standardization groups within MPEG or elsewhere.

Denoising is a key component in coding systems that wish to utilize film grain synthesis. Such methods try to first remove most, if not all, of the noise while retaining as much of the video content detail possible in a video sequence. The effectiveness of such techniques can severely impact not only coding performance, after denoising, but also the accuracy and behavior of the film grain estimation and synthesis processes.

Film grain synthesis has recently become a hot topic again within the video standardization community. Studies on the efficacy and the film grain preservation accuracy of such technologies are being conducted not only by JVET/MPEG but also other organizations such as the 3GPP and the DVB. Unfortunately, there is currently no agreed denoising scheme available for the standardization community that could be used in the context of this activity. Experiments in JVET/MPEG are using the motion compensated temporal filter (MCTF) implementation embedded in the HM and the VTM reference software, which was not originally intended for the removal of film grain, and therefore current results are not ideal.

There have been serious concerns that the current MCTF method may be inadequate for handling content with stronger film grain characteristics, which may lead to the wrong conclusions about the efficacy, usage, and suitability of film grain synthesis methods in the context of video compression. It is therefore highly desirable to find more suitable filtering schemes that could be used by standards developing engineers and researchers during the evaluation of film grain technologies.

# Recommendations

The AHG recommends:

- the related input contributions are reviewed;

- testing of FGC be discussed; and

- continue the study of film grain technologies in JVET.