| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0024-v2 |
| --- | --- |

| Title: | EE2: Summary Report on Enhanced Compression beyond VVC capability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Jie Chen Roman Chernyak Karam Naser Jacob Ström Fan Wang Martin Winken Xiaoyu Xiu Kai Zhang | Email: | vseregin@qti.qualcomm.com jiechen.cj@alibaba-inc.com chernyak@tencent.com Karam.Naser@InterDigital.com jacob.strom@ericsson.com wangfan6@oppo.com martin.winken@hhi.fraunhofer.de xiaoyuxiu@kwai.com zhangkai.video@bytedance.com |
| Source: | EE coordinators |  |  |

_____________________________

# Abstract

This document provides a summary report of Exploration Experiment on Enhanced Compression beyond VVC capability. The tests are categorized as partitioning, intra prediction, inter prediction, transform and coefficient coding, and in-loop filtering.

The software basis for this EE is ECM-9.0, released at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-9.0. ECM-9.0 is used as an anchor in the tests.

Software for EE tests is released in the corresponding branches at https://vcgit.hhi.fraunhofer.de/ecm/jvet-ad-ee2/ECM/-/branches.

Test results can be found in input JVET contributions, cross-check results are uploaded to https://vcgit.hhi.fraunhofer.de/ecm/jvet-ad-ee2/simulation-results if cross-check reports are not submitted as they are optional for EE tests.

# List of tests

|  | Tests | Tester | Cross-checker |
| --- | --- | --- | --- |
| 1 Partitioning |  |  |  |
| 1.1a | Partitioning prediction | Canon G. Laroche JVET-AE0132 | InterDigital S. Puri |
| 1.1b | Partitioning prediction with unconditional MTT depth increment | Canon G. Laroche JVET-AE0132 | InterDigital S. Puri |
| 1.1c | Encoder only partitioning prediction | Canon G. Laroche JVET-AE0132 | InterDigital F. Le Léannec |
| 2 Intra prediction |  |  |  |
| 2.1a | Block vector guided CCCM with IBC BV | Nokia R. Youvalari | B<>com M. Abdoli |
| 2.1b | Block vector guided CCCM with IBC and IntraTMP BV | Nokia R. Youvalari | B<>com M. Abdoli |
| 2.2a | Extended IBC-GPM with two IBC predictions | Kwai C. Ma | withdrawn |
| 2.2b | Bi-predictive IBC-GPM | KDDI Y. Kidani JVET-AE0169 | withdrawn |
| 2.2c | Bi-predictive IBC-GPM | Kwai C. Ma KDDI Y. Kidani JVET-AE0169 | Bytedance W. Yin JVET-AE0197 |
| 2.3a | IBC BVP-merge | KDDI Y. Kidani JVET-AE0169 |  |
| 2.3b | Test 2.3a + bi-predictive IBC merge without IBC-GPM | KDDI Y. Kidani JVET-AE0169 | Ericsson R. Yu JVET-AE0205 Bytedance W. Yin JVET-AE0197 |
| 2.3c | Bi-predictive IBC merge without + Test 2.3a + Test 2.2c | KDDI Y. Kidani Kwai C. Ma | withdrawn |
| 2.4a | IBC MBVD list derivation | Qualcomm Z. Zhang JVET-AE0169 | Ofinno D. Ruiz Coll |
| 2.4b | Test 2.4a + Test 2.3a | Qualcomm Z. Zhang KDDI Y. Kidani | withdrawn |
| 2.4c | Test 2.4a + Test 2.3b + Test 2.2c | Qualcomm Z. Zhang KDDI Y. Kidani Kwai C. Ma JVET-AE0169 | Ericsson R. Yu JVET-AE0205 |
| 2.5a | Filtered IBC | Kwai H.-J. Jhu JVET-AE0159 |  |
| 2.5b | Filtering IBC predicted blocks | Qualcomm B. Ray JVET-AE0159 |  |
| 2.5c | Test 2.5a + Test 2.5b | Kwai H.-J. Jhu Qualcomm B. Ray JVET-AE0159 | OPPO Y. Yu L. Zhang JVET-AE0288 |
| 2.6a | IBC with non-adjacent spatial candidates | Kwai C. Ma JVET-AE0094 |  |
| 2.6b | Non-adjacent spatial candidates for IBC | Bytedance Y. Wang JVET-AE0094 |  |
| 2.6c | IBC with non-adjacent spatial candidates (Test 2.6a + Test 2.6b) | Kwai C. Ma Bytedance Y. Wang JVET-AE0094 | Alibaba J. Chen |
| 2.7 | Cross-component merge mode with temporal candidates | MediaTek H.-Y. Tseng | InterDigital P. Bordes |
| 2.8 | An extrapolation filter-based intra prediction mode | OPPO L. Xu | Kwai H.-J. Jhu JVET-AE0217 |
| 2.9 | Extended search areas for IntraTMP mode | OPPO Y. Yu Xidian Y. Ma, H.Zhang Kwai X. Xiu JVET-AE0077 | InterDigital K. Naser JVET-AE0210 Qualcomm P.-H Lin JVET-AE0216 Alibaba X. Li JVET-AE0245 |
| 2.9b | Extended search areas for IntraTMP mode with scan order #2 | OPPO Y. Yu Xidian Y. Ma, H.Zhang Kwai X. Xiu | withdrawn |
| 2.9c | IntraTMP mode with partial extended search areas with scan order #1 | OPPO Y. Yu Xidian Y. Ma H. Zhang Kwai X. Xiu | withdrawn |
| 2.9d | IntraTMP mode with partial extended search areas with scan order #2 | OPPO Y. Yu Xidian Y. Ma H. Zhang Kwai X. Xiu | withdrawn |
| 2.10a | IBC-LIC extension without large block-size constraint | OPPO Z. Xie JVET-AE0078 | Bytedance W. Yin JVET-AE0157 |
| 2.10b | ECM IBC-LIC without large block-size constraint | OPPO Z. Xie JVET-AE0078 | Bytedance W. Yin JVET-AE0157 |
| 2.11a | Harmonization between IBC HMVP and IBC-LIC | Bytedance N. Zhang JVET-AE0084 | Kwai C. Ma JVET-AE0222 |
| 2.11b | Test 2.11a + Test 2.10 | Bytedance N. Zhang OPPO Z. Xie JVET-AE0084 | Kwai C. Ma JVET-AE0222 |
| 3 Inter prediction |  |  |  |
| 3.1a | Cross-component residual model | Nokia P. Astola JVET-AE0059 | InterDigital F. Le Léannec Ittiam J. Raj |
| 3.1b | Cross-component residual model with complexity reductions | Nokia P. Astola JVET-AE0059 | InterDigital F. Le Léannec Ittiam J. Raj |
| 3.2 | Bi-predictive GPM | Ericsson R. Yu JVET-AE0046 | LGE Y. Ahn JVET-AE0199 KDDI Y. Kidani |
| 3.3a | Additional TM refinement for bi-prediction | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3b | 16-point diamond search pattern for TM | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3c | Enabling TM for bi-prediction under DMVR condition | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3d | Test 3.3a + Test 3.3c | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.3e | Test 3.3a + Test 3.3b + Test 3.3c | Bytedance Y. Wang JVET-AE0087 | Alibaba J. Chen JVET-AE0239 |
| 3.4 | HPel flag and BCW weight usage in OBMC | InterDigital A. Robert JVET-AE0196 | withdrawn |
| 3.5 | Iterative BDOF pass in multi-pass DMVR | Bytedance M. Salehifar Alibaba J. Chen JVET-AE0065 | vivo Z. Lv JVET-AE0200 |
| 3.6 | Affine AMVP mode with one MVD | Qualcomm H. Huang | withdrawn |
| 3.7a | RPR with new filters, scale factor 1.25x | Sharp J. Samuelsson-Allendes JVET-AE0150 | Qualcomm Z. Zhang JVET-AE0204 |
| 3.7b | RPR with new filters, scale factor 1.33x | Sharp J. Samuelsson-Allendes JVET-AE0150 | Qualcomm Z. Zhang JVET-AE0204 |
| 3.8 | Combination of Test 3.3e and Test 3.5 | Bytedance M. Salehifar Alibaba J. Chen JVET-AE0091 | Kwai C. Ma JVET-AE0223 |
| 4 Transform and coefficient coding |  |  |  |
| 4.1 | Shifting quantizer center | InterDigital M. Balcilar JVET-AE0125 | Qualcomm M. Coban JVET-AE0149 |
| 4.2 | Large NSPT | LGE M. Koo JVET-AE0086 | Qualcomm P. Garus JVET-AE0118 |
| 4.3 | Context modelling for transform coefficients for LFNST/NSPT | Qualcomm P. Nikitin JVET-AE0102 | IRT b-com M.Abdoli JVET-AE0225 |
| 4.4a | InterMTS is enabled for IBC-coded blocks in AMVP mode. | Qualcomm P. Garus JVET-AE0116 | InterDigital K. Naser JVET-AE0211 |
| 4.4b | Test 4.4a + IntraTMP using interMTS instead of intraMTS kernels | Qualcomm P. Garus JVET-AE0116 | InterDigital K. Naser JVET-AE0211 |
| 4.4c | IntraMTS disabled for IntraTMP | Qualcomm P. Garus JVET-AE0116 | InterDigital K. Naser JVET-AE0211 |
| 5 In-loop filtering |  |  |  |
| 5.1a | CCSAO with temporal history offset | Kwai C.-W. Kuo JVET-AE0151 | Qualcomm N. Hu JVET-AE0209 |
| 5.1b | Test 5.1a + extended edge classifier | Kwai C.-W. Kuo JVET-AE0151 | Qualcomm N. Hu JVET-AE0209 |
| 5.2a | Applying fixed filters to samples before DBF | Qualcomm N. Hu JVET-AE0139 | Kwai C.-W. Kuo JVET-AE0193 |
| 5.2b | Test 5.2a + extended classifiers for fixed filters | Qualcomm N. Hu JVET-AE0139 | Kwai C.-W. Kuo JVET-AE0193 |
| 5.2c | Test 5.2b + applying the second fixed filter to outputs of the first fixed filter | Qualcomm N. Hu JVET-AE0139 | Kwai C.-W. Kuo JVET-AE0193 |
| 5.3 | Combination of Test 5.1b and Test 5.2c | Kwai C.-W. Kuo Qualcomm N. Hu JVET-AE0152 | Alibaba J. Chen |

# Description of tests

## Partitioning

### Test 1.1: Partitioning prediction (JVET-AE0132)

In this test, a temporal partitioning is introduced, where for each block, the allowed partitioning splits are predicted according to the minimum QT/MTT split and the average QT/MTT split obtained from a temporal area:

- if the current QT depth is less than the temporal minimum QT depth minus 1, only the QT split is allowed,

- if the current QT depth is less than the temporal average QT depth minus 1, no split, QT and TT splits are allowed, and BT split is allowed if TT is selected in a parent node,

- if the temporal maximum MTT depth and less than the maximum MTT depth of the current block, the depth is decreased except if the current QP is less than the temporal QP,

- if the temporal maximum MTT depth is large than the maximum MTT depth of the current block and if the current depth is equal to the current QT depth, the maximum MTT depth is incremented,

- if the maximum QT depth of the current frame is larger than the maximum multi-tree depth and if the temporal maximum MTT depth is equal to the maximum MTT depth of the current frame, and if the temporal QT depth is less than the QT depth of the current frame, the maximum MTT depth of the current block is decreased.

The modifications on the maximum MTT depth are enabled only if the palette mode is disabled.

The temporal area is a collocated area from the same reference frame used for the temporal motion predictor.

JVET-AE0132 has memory requirement analysis where the largest memory which is required to store temporal depths for class A is estimated as 40.5 KB.

Test 1.1a: Partitioning prediction (as described above)

Test 1.1b: Test 1.1a where MTT depth increment is enabled unconditionally. It affects classes A and B, class A1 encoder runtime reaches 105.8%.

Test 1.1c: Use partition prediction as encoder only optimization.

## Intra prediction

### Test 2.1: Block vector guided CCCM (JVET-AE0100)

In this test, co-located luma BV is used to determine the reference area for calculating the CCCM parameters. Then the reference area in luma and corresponding area in chroma channel is used to calculate the CCCM parameters. The prediction uses the calculated model parameters and co-located luma samples to do the CCCM prediction as follows:

predChromaVal = c0C + c1N + c2S + c3E + c4W + c5P(C) + c6P(N) + c7P(S) + c8P(W) + c9P(E)+ c10B,

where spatial components are as shown in the next figure, the nonlinear term P is represented as power of two of the corresponding luma sample, and B is the bias term.

The mode is enabled only in intra slices and is controlled by SPS flag.

Test 2.1a: the mode only uses BVs of the IBC coded blocks from co-located luma area.

Test 2.1b: the mode can use BVs of both IBC and IntraTMP coded blocks from co-located luma area.

### Test 2.2: Bi-predictive GPM (JVET-AE0169)

In ECM-9.0, IBC GPM uses uni prediction for one partition and intra mode for the other one. In the test, the core GPM design is kept unchanged which uses 48 GPM modes and IBC merge candidate list, while it enables both partitions being predicted using IBC with different BVs.

Two flags are signalled to indicate the prediction modes of two partitions, the first flag indicates whether the first partition is intra predicted, and if not then the second flag is signalled to indicate whether intra prediction is used for the second partition.

The method is applied to SCC only.

### Test 2.3: IBC BVP-merge and bi-predictive IBC merge (JVET-AE0169)

In Test 2.3a, IBC-BVP-merge, which is similar to AMVP-merge, derives one BV from IBC block vector prediction (BVP) and the second BV from IBC merge to form bi-prediction for IBC. Two different indices for the IBC BVP and the IBC merge candidates are signalled.

In Test 2.3b, bi-predictive IBC merge is introduced, and it is enabled together with the existed in ECM MBVD and uni-merge (currently disabled by the encoder configuration for non-SCC classes). In bi-predictive IBC merge, two BVs from the existing IBC merge candidate list are derived, utilizing two different indices, which are signalled. Bi-predictive IBC merge is applied to IBC regular merge and IBC MBVD. Bi-predictive IBC merge, IBC MBVD, and IBC uni-merge are enabled for non-SCC classes and is tested together with Test 2.3a.

### Test 2.4: IBC MBVD list derivation (JVET-AE0169)

In the test 2.4a, adaptive BVD offsets along MVBD directions and enabled for IBC MBVD mode. The MBVD candidates search is a two-step process, which starts with checking template SAD costs of offsets added to BVP along each direction with the interval of 1-pel. The second step of the search checks template SAD costs with 1/4-pel interval for the candidates around the selected candidates from the first step. For the integer MBVD (when existed in ECM ph_fpel_mbvd_enabled_flag is 0), those intervals are multiplied by 4. The candidates with the lowest TM cost are included into the final MBVD list.

Test 2.4c is a combination of Test 2.4a, Test 2.3b, and Test 2.2c.

### Test 2.5: Filtered IBC (JVET-AE0159)

In the test, additional filtered IBC mode is introduced, where a filter is applied to IBC predictor, which is derived by minimizing MSE between current and reference template.

Output of the filter is calculated as follows:

predLumaVal = c0C + c1N + c2S + c3E + c4W + c5P + c6B

The nonlinear term P is represented as power of two of the center sample C and scaled to the sample value range of the content:

P = ( C*C + midVal ) >> bitDepth

The bias term B represents a scalar offset between the input and output and is set to middle luma value (512 for 10-bit content).

This filtered mode is used as an additional mode for non-merge IBC blocks, and it is not used together with IBC-LIC, IBC-CIIP or RR-IBC. For IBC merge modes, this filtering mode is inherited when merge mode list is constructed.

In Test 2.5a, the mode flag is signalled conditioned on the IBC-LIC flag, so when the IBC-LIC flag is true, the flag is signalled and used to indicate whether the tested mode is applied to the current block or not.

In Test 2.5b, the mode flag is signalled before IBC-LIC flag, and the model does not have non-linear term.

In Test 2.5c, the model of Test 2.5a and signalling of Test 2.5b are used, different aspects of encoder design is chosen from both tests considering the best trade-off.

### Test 2.6: IBC with non-adjacent spatial candidates (JVET-AE0094)

In the test, several new candidates obtained from the BVs of non-adjacent spatial neighboring blocks are added to the candidate lists of IBC merge modes and IBC AMVP. A pattern similar to the non-adjacent spatial candidates of regular inter is used to obtain non-adjacent BVs shown in the below figure. The obtained BVs are inserted between the adjacent spatial candidates and the HBVP candidates for both IBC merge and IBC AMVP. Additionally, the restriction that spatial candidates are not used to construct IBC merge candidate list for a 44 CU is removed.

In Test 2.6a, the same reference area as for non-adjacent regular merge is reused for the IBC, where up to 4 rows/columns of spatial neighboring blocks (in terms of the CU height/width) of the CU can be used as reference for IBC AMVP, and up to 7 rows/columns of the spatial neighboring blocks (in terms of the CU height/width) of the CU are used for IBC merge.

In Test 2.6b, for both IBC AMVP and IBC merge, the spatial neighboring blocks located up to 4 rows/columns of spatial neighboring blocks (in terms of the CU height/width) of the CU can be used as reference. Additionally, the restriction that spatial candidates are not used for the IBC merge of a 44 CU in ECM-9.0 is removed.

Test 2.6c is the combination of Test 2.6a and Test 2.6b, where the sampling method in Test 2.6a is applied. Additionally, the removal of the restriction that spatial candidates cannot be used for IBC merge of a 4x4 CU from Test 2.6b is used.

### Test 2.7: Cross-component merge mode with temporal candidates (JVET-AE0043)

In the current CCP merge mode, a CCP merge candidate list contains spatial adjacent, spatial non-adjacent, and history-based candidates coded in CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes. After including these candidates, default models can be included to fill the remaining empty positions in the merge list if necessary. To remove redundant CCP models in the merge list, pruning operations are applied. The candidates in the list are reordered based on the SAD costs obtained using the neighboring template of the current block.

In the test, two types of candidates, namely temporal candidates and shifted temporal candidates, are additionally included in the merge list for non-intra slices. Temporal candidates are added to the merge list after the spatial adjacent candidates. Shifted temporal candidates are added after the history-based candidates.

CCP merge mode is allowed to be applied in non-intra slices for chroma blocks with sizes less than or equal to 16.

Temporal candidates

Temporal candidates are selected from the collocated picture, the positions and inclusion order of the temporal candidates, which are the same as those for temporal candidates in inter merge mode of ECM-9.0, is shown in Figure 1.

Figure 1. Positions of the temporal candidates

An inclusion order of the temporal candidates is C01  C02  …  C010. If C0i is outside of the picture/slice boundary and C1i is inside of the picture/slice boundary, C1i will be used instead of C0i, where 1 ≤ i ≤ 10. Otherwise, the next inclusion position is checked.

Shifted temporal candidates

Shifted temporal candidates are also selected from the collocated picture. As depicted in Figure 2, the position of the collocated block is shifted by a selected neighboring motion vector. Consequently, the positions of C0i and C1i are shifted by the same neighboring motion vector. The inclusion order of shifted temporal candidates is the same as that of temporal candidates.

The adjacent motion vector is selected from the motion vectors of the neighboring blocks ordered as follows: L0B1  L1B1  L0A1  L1A1  L0B0  L1B0  L0A0  L1A0  L0B2  L1B2. The first motion vector, which uses the collocated picture as the reference picture, is selected. If no such motion vector is found, no shifted temporal candidate is added.

Figure 2. Positions of the shifted candidates (left) and selecting the neighboring motion vector (right)

### Test 2.8: Extrapolation filter-based intra prediction mode (JVET-AE0076)

In the test, extrapolation filter-based intra prediction is introduced with 3 types of reconstructed areas and 3 filter shapes as shown in Figure 3, a choice of reconstructed area and filter shape is signalled.

The size of reconstructed area depends on the min(blockWidth, blockHeight) and the selected filter shape. For example, when the current block is an 8x16 block and the selected filter shape is 4x4. The aboveSize of reconstructed area is min(8, 16) + 4 – 1 = 11, and the leftSize of reconstructed area is min(8, 16) + 4 – 1 = 11.

Figure 3. Three types of reconstructed areas (top) and three filter shapes (bottom)

The selected filter moves in the selected reconstructed area with a one-sample step to collect input samples and output samples, then the filter coefficients are derived using CCCM solver.

In the current block, the recursive predictor is derived from the top-left to the bottom-right position by a diagonal prediction order as shown in Figure 4, where the predicted results of the previous diagonal are used.

Figure 4. Example of generating predictions for different positions in the current block by a diagonal order

To reduce the prediction error, the min and max values from the neighboring reconstructed area are applied to restrict the output range of each predicted value.

The predicted samples are calculated as follows,

predx,y=clip(min,max,i=014(ci×(tx-xoffset, y-yoffset-offset))+offset)

where predx,y is the predicted value at (x, y) in the current block. min,  max and offset are derived in the selected reconstructed area, ci is the ith coefficient of the derived filter, tx-xoffset, y-yoffset is reconstructed or predicted value used for prediction in the current position.

The tested mode is applied in I-slices only and is used for the block sizes less than or equal to 32x32.

### Test 2.9: Extended search areas for IntraTMP mode (JVET-AE0077)

In the test, IntraTMP search areas are extended by including the bottom-left R5 and top-right R6 areas relative to the current block as they may also be available as shown in Figure 5. In ECM-9.0, the region scan order is R4, R1, R2, and R3. In the tested method, the region scan order is R4, R5, R6, R1, R2, and R3.

Figure 5. Extended IntraTMP search areas (R5 and R6)

In addition, the current ECM-9.0 always searches all available positions within the current CTU while it may be beyond the specified search range or width/height. In the test, all search areas are restricted to be within the specified search range or width/height specified by (searchRangeWidth, searchRangeHeight) as illustrated in Figure 6. The maximum horizontal and vertical search sizes, i.e., searchRangeWidth and searchRangeHeight, are set to max(5W, 64) and max(5H, 64) respectively. It is not allowed to search any positions outside the specified search range even those available positions are within the current CTU.

Figure 6. Search areas restriction

### Test 2.10: IBC-LIC extension (JVET-AE0078)

In Test 2.10a, IBC-LIC is extended by including 3 additional modes:

- Use only left or use only above LIC template in addition to the current L-shape template,

- Extend the MMLM to IBC-LIC, which allows IBC-LIC to have two linear models in one CU for L-shape template.

Large block-size constraint for ECM IBC-LIC, tested separately in Test 2.10b, is additionally removed in Test 2.10a.In Test 2.10b, large block-size constraint for ECM IBC-LIC is removed, so IBC-LIC can be applied to the CU whose block size is larger than or equal to 32.

### Test 2.11: Harmonization of IBC HMVP and IBC-LIC (JVET-AE0084)

In ECM-9.0, the inter LIC flag can be inherited from a HMVP candidate. However, the IBC-LIC flag is not inherited when the motion information from an IBC HMVP candidate is used.

In Test 2.11a, IBC-LIC flag is inherited from an IBC HMVP candidate, which is similar to the inter LIC case.

Test 2.11b: Test 2.11a + Test 2.10a

## Inter prediction

### Test 3.1: Cross-component residual model for inter prediction (JVET-AE0059)

Cross-component residual model to predict chroma samples from reconstructed luma samples when the block uses inter prediction or intra block copy (IBC) is tested. As illustrated in Figure 7, the cross-component filters are derived using the prediction blocks of luma and chroma. The derived filters are applied to the reconstructed luma block and blended with the prediction blocks of chroma to produce the final chroma prediction blocks. In the blending process the filtered reconstructed luma blocks use blending weight of 0.75 and chroma prediction blocks use blending weight of 0.25.

Figure 7. Cross-component residual model

Model uses 8-tap filter consisting of 6 spatial luma samples shown in Figure 8, a nonlinear term, and a bias term as follows:

predChromaVal = c0 L0+ c1L1 + c2L2 + c3L3 + c4L4 + c5L5 + c6 nonlinear((L0+L3+1) >> 1) + c7 B,

Figure 8. 2 Luma samples L0,..,L5 in relation to the chroma sample C.

The filter coefficients are derived using ECM’s division-free Gaussian elimination method and the necessary offsets are applied to samples prior to filter derivation.

For filter coefficient derivation at most 256 chroma samples are used.

The mode flag is only signalled if the TU’s luma Cbf is non-zero and the CU’s predMode is either MODE_INTER or MODE_IBC.

Test 3.1a: Cross-component residual model, where offsets for division free operation are obtained by averaging the samples in the block.

Test 3.1b: Cross-component residual model, where offsets for division free operation are obtained by averaging the four points correspond to the top-left, top-right, bottom-left and bottom-right corners of the blocks.

### Test 3.2: Bi-predictive GPM (JVET-AE0046)

In the test, the existing uni-predictive GPM is extended to allow usage of bi-predictive motion vectors for generating motion compensated prediction samples for inter partitions. The method consists of the following elements:

The first element conditionally invokes the existing extraction process that extracts uni-predictive motion vectors from the initial list. The extraction process is invoked only for small blocks 8x8, 16x8 and 8x16. For other larger blocks, the extraction process is bypassed, so the initial list (which may contain merged Bi-MVs) is directly used as the final GPM merge list. The generation of the initial list is the same as before (i.e., the normal merge list generation without any candidate reordering) except that when generating the initial list for larger blocks (i.e., blocks with the extraction process bypassed), the motion vector difference threshold for controlling whether a candidate can be added into the initial list is increased to be one full sample distance.

The second element modifies GPM-MMVD to support bi-predictive motion vector as the base vector. For low-delay pictures, the signalled MVD is applied on top of the L0 and L1 motion vector as in the existing merge MMVD design. For non-low-delay pictures, the bi-predictive motion vector is converted into a uni-predictive motion vector first and then the MVD is applied on top.

The third element modifies GPM-TM to also support bi-predictive motion vectors.

The last element is to enable the BDOF based mv refinement as in the multi-pass DMVR on top of the associated bi-predictive motion vectors for each inter partition.

### Test 3.3: High-Accuracy template matching (JVET-AE0087)

In ECM, template matching is a decoder-side MV derivation method to refine the motion information of the current CU by finding the closest match between a template (i.e., top and/or left neighbouring blocks of the current CU) in the current picture and a block (i.e., same size to the template) in a reference picture.

When TM is used for bi-prediction, the following steps are applied:

- The initial motion vector of list 0 (MV0) is refined to derive a refined MV (MV’0) and a TM cost C0 is calculated;

- The initial motion vector of list 1 (MV1) is refined to derive a refined MV (MV’1) and a TM cost C1 is calculated;

- If C0 is larger than C1, MV’1 is used to derive a further refined MV (MV’’0) by refining MV’0; Otherwise, MV’0 is used to derive a further refined MV (MV’’1) by refining MV’1 .

In high-accuracy template matching, three aspects are tested:

- In Test 3.3a, an additional refinement is applied to TM for bi-prediction, when MV’0 is refined in step 3), MV’’0 is used to derive MV’’1 by refining MV’1; Otherwise, MV’’1 is used to derive MV’’0 by refining MV’0.

- In Test 3.3b, the diamond search pattern used in TM is modified from 8-point to 16-point.

- In Test 3.3c, TM for bi-prediction is enabled when DMVR condition is satisfied.

Test 3.3d: Test 3.3a + Test 3.3c.

Test 3.3e: Test 3.3a + Test 3.3b + Test 3.3c.

### Test 3.4: OBMC with HPel flag and BCW weights (JVET-AE0196)

In the current OBMC design, the motion compensation of the neighboring blocks and subblocks performed by OBMC only considers the motion vectors and reference pictures.

In the test, HPel flags and BCW weights of the neighboring blocks and subblocks are considered in the OBMC. When processing the top and left borders, the neighboring BCW weights and HPel flags are used in addition to the motion vectors and reference pictures. The neighboring HPel flag allows setting the correct AMVR index. For the internal subblock boundaries, the BCW and AMVR indexes of the current CU are also used.

It has to be noticed that the integration of adopted contribution JVET-AD0213 (Bi-prediction LIC) turns out to already include main aspect of this test (BCW weights in OBMC) in the ECM-9.0 which was not the case in ECM-8.0.

### Test 3.5: Iterative BDOF pass in multi-pass DMVR (JVET-AE0065)

In ECM, a multi-pass DMVR has three passes of motion refinement. In the first pass, bilateral matching (BM) is applied to the coding block (CB) to get the block level refined MV pair. In the second pass, the block is divided into 16×16 subblocks and on top of block level refined MV pair obtained in the first pass, BM is applied on each 16×16 subblock to get a 16×16 subblock level refined MV pair. In the third pass, BDOF based MV refinement is applied on the 4×4 or 8×8 subblocks depending on the CB size.

In the test, the current multi-pass DMVR is extended by adding another BDOF-based motion refinement pass as the 4th pass of the multi-pass DMVR (i.e., the 2nd round of BDOF-based DMVR).

Also, the subblock size of the proposed round of the BDOF-based DMVR is adaptively selected depending on the area (i.e., width×height) of the coding block. For blocks smaller than 1024 pixels, the subblock size of 4×4, and otherwise 8×8 is used.

### Test 3.7: RPR with new filters for scale factor 1.25x and 1.33x (JVET-AE0150)

In the test, additional set of 8-bit precision RPR filters consisting of three tables (respectively 12-tap for luma, 6-tap for chroma, and 10-tap for affine) is introduced with the target to improve the performance for scaling ratios 1.25x and 1.33x. These new set of filters is used for scaling ratios in between 1.1x and 1.35x.

In the tests, two PSNR values are used:

- PSNR1: it is measured on the decoded picture size by calculating MSE for each picture, accumulating MSEs for all pictures and calculating average PSNR from the accumulated MSE.

- PSNR2: it is measured after upsampling of the decoded picture if the decoded picture size is in smaller resolution, the existed ECM interpolation filters are used for the upsampling.

### Test 3.8: Combination of Test 3.3e and Test 3.5 (JVET-AE0091)

## Transform and coefficient coding

### Test 4.1: Shifting quantizer center (JVET-AE0125)

In the test, a quantization offset is added to the quantized level, the offset is quantization index dependent, a look-up table is used to derive the offset T[yi].

xi=1024-T[yi]*Q-1yi+T[yi]*Q-1y'i>>10,

where x is the dequantized coefficient, y and y’ are the quantization indices, Q-1 is the dequantization operation, T is a look-up table

T=[0,63, 31, 21, 15, 12, 10, 9, 7, 7, 6, 5, 5, 4, 4, 4, 3, 3, 3, 3, 3, 3, 2, 2, 2, 2, 2, 2, 2, 2, 2, 2, 1,…,1].

### Test 4.2: Large NSPT (JVET-AE0086)

The large NSPT kernels are tested for 4x32/32x4 and 8x32/32x8 blocks, of which kernel matrix dimensions are 20x128 and 24x256. Therefore, 20 and 24 transform coefficients are generated by applying the two types of kernel matrices, respectively, which are placed from DC position following scan order. The remaining 108 and 232 positions in each transform block are zeroed-out, respectively.

Large NSPT kernels are applied in the same way as for other block sizes 4x4, 4x8/8x4, 4x16/16x4, 8x8, and 8x16/16x8 and those NSPT kernels are not changed.

### Test 4.3: Context modeling for transform coefficients for LFNST/NSPT (JVET-AE0102)

In ECM, the causal 2D neighborhood of coefficients is used to model the context to parse the sig_coeff_flag, gt1_flag, gt2_flag as shown in Figure 9. For LFNST, DCT-II coefficients are placed into the coefficient block using diagonal reordering.

Figure 9. Context modeling for LFNST coefficients in ECM

In the test, when LFNST/NSPT is applied, the previous 5 coefficients in the coding order are used for context derivation instead of 2D neighborhood.

Figure 10. Context modeling for LFNST coefficients in TEST4.3

In ECM, the lfnstIdx is signalled after all the coefficients in a CU. In the test, lfnstIdx is signalled after all last_sig_coeff_pos syntax elements in a CU since lfsntIdx is required for parsing the transform coefficients.

### Test 4.4: InterMTS for IBC and IntraTMP (JVET-AE0116)

InterMTS enabled for IBC and IntraTMP is tested. The following tests are performed:

Test 4.4a: InterMTS is enabled for IBC-coded blocks in AMVP mode. MTS is not enabled for IBC merge.

Test 4.4b: InterMTS is enabled for IBC-coded blocks in AMVP mode as in Test 4.4a. In addition, IntraTMP is utilizing InterMTS instead of IntraMTS. Consequently, the number of MTS candidates for IntraTMP has been fixed to 4 instead of adaptively setting it to 1, 4 or 6 as in IntraMTS.

Test 4.4c: IntraMTS is disabled for IntraTMP coded blocks.

## In-loop filtering

### Test 5.1: CCSAO with extended edge classifiers and history offsets (JVET-AE0151)

In ECM-9.0, CCSAO uses band and edge classifiers switched at CTB level. Classifier parameters/offsets are derived at encoder and signaled for each slice independently.

In Test 5.1a, to reduce the signaling overhead, CCSAO inheritance scheme is introduced, where the offsets/parameters of some coded pictures are stored at both encoder and decoder which are allowed to be used as the CCSAO offsets/classifiers of future pictures. An index is signaled in SH to indicate which candidate in the CCSAO parameter storage is selected for the current slice. The candidates in the CCSAO parameter storage are updated in a FIFO manner and refreshed at IDR pictures.

In Test 5.1b, on top of Test 5.1a, beside the existing CCSAO edge classification, one new edge classification is added, which is a subset of the original one with less edge range divisions.

orginal: iE=d<0?d<-T?0:1:d<T?2:3added:  iE=                     d<T?0:1

where iE is calculated by comparing a sample difference d in one direction with a threshold T.

Additionally, the component used for edge classification can be selected from one of all three components. Same to the existing CCSAO design, the selected edge classifier and edge component are decided by encoder and signaled in SH.

### Test 5.2: Improved fixed filters for ALF (JVET-AE0139)

In ECM-9.0, two Laplacian-based classifiers (one for each fixed filter) are applied to a 2x2 block. In each classifier, activity and directionality values are derived based on vertical, horizontal, and diagonal gradients using a window surrounding each 2x2 block. Then a class index is determined based on the activity and directionality values. Two 13x13 diamond shaped fixed filters are selected from the two filter sets by using the derived two class indices. Then the two fixed filters (fi with i=0, 1) are applied to the ALF input samples. Finally, a signalled filter is applied to the ALF input samples, samples before the deblocking filter (DBF), outputs of the two fixed filters, output of a gaussian filter and the residual data.

In Test 5.2a, both fixed filters are applied to samples before DBF and ALF input, where additional diamond 9x9 filter is used for the samples before DBF. The shape of the first fixed filter applied to the ALF input samples is reduced from 13x13 to 9x9, and the shape of the second fixed filter, which is 13x13, applied to ALF input is unchanged as shown in the below table.

|  | ECM-9.0 | Proposed method |  |
| --- | --- | --- | --- |
|  | ALF input | Samples before DBF | ALF input |
| Fixed filer f0 | 13x13 | 9x9 | 9x9 |
| Fixed filer f1 | 13x13 | 9x9 | 13x13 |

In Test 5.2b, on top of Test 5.2a, the classifiers of the fixed filters are extended. For each 2x2 block, the mean value of a surrounding window is calculated. Then, for each sample of this window, the difference between the sample value and the mean value is calculated. A scaling factor is determined based on the activity value derived from a Laplacian classifier. The square root of the sum of the squared differences is further quantized to C' by a scaling factor. The value of C' is an integer between 0 and 7, inclusively. With i=0, 1, let Ci denote the classifier from the classifier of i-th fixed filter in ECM-9.0. Then the proposed class index Ci' is derived as

Ci'= C'*896+Ci.

The total number of the fixed filters is not changed.

In Test 5.2c, on top of Test 5.2b, fixed filter f1 is applied to outputs of f0 (instead of ALF input) and samples before DBF.

### Test 5.3: Combination of Test 5.1b and Test 5.2c (JVET-AE0152)

# Test results

See the accompanied Excel table.