| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0047-v1 |
| --- | --- |

| Title: | AHG12: On GPM-MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

GPM-MMVD provides a way to adjust motion vectors for inter partitions of GPM blocks. Similarly to merge-MMVD, GPM-MMVD explicitly signals MVD which are applied on top of merged motion vectors. The MVD magnitudes that can be signaled in GPM-MMVD is limited within sets which always include factional MVDs. It is proposed to let GPM-MMVD follow the merge-MMVD design to allow possibilities of disabling those fractional MVDs. The proposed modification is reported to be implemented in ECM-9.0 and tested for ClassA1 and ClassA2 under the ECM RA CTC. The BD-rate impact is reported to be -0.01% for ClassA1 and -0.01% for ClassA2 (estimated numbers, final numbers will be updated).

# Introduction

In VVC and ECM, merge-MMVD applies explicitly signaled MVDs to merged motion vectors of normal merge blocks. In the merge-MMVD design, fractional MVDs can be disabled for certain type of pictures. For example, in VTM and ECM CTC, the fractional MVDs in merge-MMVD are disabled for 4K sequences in ClassA1 and ClassA2.

In ECM, GPM-MMVD explicitly signals MVD on top of merged motion vectors for GPM inter partitions. The magnitude of MVD that can be signaled is within set (¼-pel, ½-pel, 1-pel, 2-pel, 3-pel, 4-pel, 6-pel, 8-pel, 16-pel) or set (¼-pel, ½-pel, 1-pel, 2-pel, 4-pel, 8-pel, 16-pel, 32-pel). It is noticed that the fractional MVDs such as ¼-pel and ½-pel are always present in both sets.

It is proposed to let GPM-MMVD follow the merge-MMVD design to allow the possibilities of disabling fractional MVDs. In other words, when fractional MVDs are disabled, the magnitude of MVDs within the sets are all multiplied by 4.

# Code change

Only two lines of code in ECM are modified, as in the patch file “JVET-AE0047.patch” associated with the contribution.

# Results

The proposed modification is reported to be tested under ECM RA CTC for ClassA1 and ClassA2 sequences (since only those sequences are affected). The encoding and decoding numbers are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,17% | -0,03% | 101% | 104% |
| Class A2 | -0,01% | -0,18% | 0,00% | 103% | 104% |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.