| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0047-v2 |
| --- | --- |

| Title: | AHG12: On GPM-MMVD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ruoyang Yu | Email: | ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

GPM-MMVD provides a way to adjust motion vectors for inter partitions of GPM blocks. Similarly to merge-MMVD, GPM-MMVD explicitly signals MVD which are applied on top of merged motion vectors. The MVD magnitudes that can be signaled in GPM-MMVD is limited within sets which always include factional MVDs. It is proposed to let GPM-MMVD follow the merge-MMVD design to allow possibilities of disabling those fractional MVDs. The proposed modification is reported to be implemented in ECM-9.0 and tested for ClassA1 and ClassA2 under the ECM RA CTC. The BD-rate impact is reported to be -0.01% for ClassA1 and 0.01% for ClassA2.

In v2 of the contribution, additional results for disabling fractional MVD for GPM-MMVD on screen contents are provided.

# Introduction

In VVC and ECM, merge-MMVD applies explicitly signaled MVDs to merged motion vectors of normal merge blocks. In the merge-MMVD design, fractional MVDs can be disabled for certain type of pictures. For example, in VTM and ECM CTC, the fractional MVDs in merge-MMVD are disabled for 4K sequences in ClassA1 and ClassA2.

In ECM, GPM-MMVD explicitly signals MVD on top of merged motion vectors for GPM inter partitions. The magnitude of MVD that can be signaled is within set (¼-pel, ½-pel, 1-pel, 2-pel, 3-pel, 4-pel, 6-pel, 8-pel, 16-pel) or set (¼-pel, ½-pel, 1-pel, 2-pel, 4-pel, 8-pel, 16-pel, 32-pel). It is noticed that the fractional MVDs such as ¼-pel and ½-pel are always present in both sets.

It is proposed to let GPM-MMVD follow the merge-MMVD design to allow the possibilities of disabling fractional MVDs. In other words, when fractional MVDs are disabled, the magnitude of MVDs within the sets are all multiplied by 4.

# Code change

Only two lines of code in ECM are modified, as in the patch file “JVET-AE0047.patch” associated with the contribution.

# Results

The proposed modification is reported to be tested under ECM RA CTC for ClassA1 and ClassA2 sequences (since only those sequences are affected). The encoding and decoding numbers are not accurate.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,18% | -0,02% | 100% | 107% |
| Class A2 | 0,01% | -0,15% | 0,03% | 104% | 112% |

# Additional Results

In the current ECM, for screen contents, fractional MVDs for merge-MMVD are enabled. An attempt was made in this contribution to disable the usage of fractional MVDs for merge-MMVD based on block hash value matching percentage of each picture (i.e., based on the existing already calculated “hashBlkHitPerc” variable in ECM). The corresponding code for this modification is provided in the patch file “JVET-AE0047-additional.patch” associated with the contribution.

## Additional results for disabling fractional MVDs in merge-MMVD for screen contents

The following table shows the results for Class F and Class TGM under RA.

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0,01% | 0,04% | 0,04% | 101% | 103% |
| Class TGM | -0,03% | -0,04% | -0,04% | 99% | 106% |

## Additional results for disabling fractional MVDs in both merge-MMVD and GPM-MMVD for screen contents

The following table shows the results for Class F and Class TGM under RA when further disabling fractional MVDs for GPM-MMVD (i.e., disabling fractional MVDs for both merge-MMVD and GPM-MMVD based on block hash match percentage value for each picture)

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0,01% | 0,03% | 0,01% | 99% | 100% |
| Class TGM | -0,08% | -0,10% | -0,08% | 95% | 100% |

## Additional VTM results for disabling fractional MVDs in merge-MMVD for screen contents

The modification (corresponding to 4.1) which disables fractional MVDs in merge-MMVD based on block hash value matching percentage is applied on top of VTM-20.0. The following table shows the results for Class F and Class TGM under RA comparing to VTM-20.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-20.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | 0,00% | 0,00% | -0,01% | 101% | 101% |
| Class TGM | -0,09% | -0,12% | -0,09% | 102% | 106% |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.