| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0049-v2 |
| --- | --- |

| Title: | AHG9: Miscellaneous VVC changes on neural-network post-filter SEI messages |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Maria Santamaria Nokia Technologies, Tampere, Finland | Email: | miska.hannuksela@nokia.com francesco.cricri@nokia.com maria.santamaria_gomez@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This input document proposes miscellaneous items for the VVC text on the neural-network post-filter characteristics (NNPFC) and activation (NNPFA) SEI messages. The proposed changes are relative to JVET-AD2005. The following items are proposed:

- Allowing multiple NNPFC and NNPFA SEI messages with different content in the same picture unit by excluding NNPFC and NNPFA SEI messages from the existing constraint in JVET-AD2005, subclause D.3.1.

Depending on the outcome of JVET-AE0048, item 5, the NNPFA SEI message is constrained in a manner that the payload shall be identical for the same nnpfa_target_id value, or an NNPFA SEI message cancelling the previous NNPFA SEI message followed by an NNPFA SEI with the same nnpfa_target_id for activating an NNPF are allowed for the same picture unit.

- Constraining the repeated inference to generate interpolated pictures up to the end of the bitstream only to NNPFs that perform only picture rate upsampling.

- Correcting the index range in input picture assignment for repeated inference at the end of a bitstream: it is proposed to remove "− j" from "When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i:".

# Allowing multiple NNPFC and NNPFA SEI messages with different content for the same picture unit

## Problem statement

The design intent is to allow

- multiple NNPFC SEI messages with different nnpfc_id values in the same picture unit,

- multiple NNPFA SEI messages with different nnpfa_target_id values in the same picture unit, and

- two NNPFC SEI messages with the same nnpfc_id value, one defining the base filter and another defining a filter update, in the same picture unit.

Thus, the following constraint in JVET-AD2005, subclause D.3.1 is too strict for NNPFC and NNPFA SEI messages:

The following applies on the content of scalable-nested and non-scalable-nested SEI messages applying to the same OLS or layer:

–	When there are multiple SEI messages with a particular value of payloadType not equal to any of 4, 5, and 133 that are associated with a particular AU or DU and apply to a particular OLS, layer, or subpicture OLS or layer, regardless of whether some or all of these SEI messages are scalable-nested, the SEI messages shall have the same SEI payload content.

The content of NNPFC SEI message has already been constrained in JVET-AD2005, subclause D.12.11 as follows:

There shall not be more than two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id. When there are two NNPFC SEI messages present in a picture unit with the same value of nnpfc_id, these SEI messages shall have different content.

It is asserted in JVET-AE0048, item 5 that the JVET-AD2006 text for NNPFA persistence and cancel flags cause situations where two NNPFA SEI messages with the same nnpfa_target_id value are present in the same picture unit, the first cancelling the persistence of a previous NNPFA SEI message with the same nnpfa_target_id value and the second activating a base filter (when an updated filter was active before) or an updated filter (when the base filter was active before).  JVET-AE0048, item 5 proposes a change that avoids the first of these NNPFA SEI messages in the same picture unit.

## Proposed changes

### Overview

It is proposed to exclude NNPFC and NNPFA SEI messages from the existing constraint in JVET-AD2005, subclause D.3.1. For the constraint on the content of NNPFA SEI message, different options are proposed depending on whether JVET-AE0048, item 5 is adopted.

The highlighted changes in green are proposed:

### Common part for both options

The following applies on the content of scalable-nested and non-scalable-nested SEI messages applying to the same OLS or layer:

–	When there are multiple SEI messages with a particular value of payloadType not equal to any of 4, 5, and 133, 210, and 211 that are associated with a particular AU or DU and apply to a particular OLS, layer, or subpicture OLS or layer, regardless of whether some or all of these SEI messages are scalable-nested, the SEI messages shall have the same SEI payload content.

### Proposal if JVET-AE0048, item 5 is adopted

The following bullet is proposed to be added next to the bullet above. The proposed text requires all NNPFA SEI messages with the same nnpfa_target_id, present in the same AU, and applying to the same OLS or layer to have the same content.

–	When there are multiple SEI messages with payloadType equal to 211 and the same nnpfa_target_id value that are associated with a particular AU or DU and apply to a particular OLS or layer, regardless of whether some or all of these SEI messages are scalable-nested, the SEI messages shall have the same SEI payload content.

### Proposal if JVET-AE0048, item 5 is not adopted

The following bullet is proposed to be added next to the bullet above. The proposed text requires all NNPFA SEI messages with the same nnpfa_target_id, having nnpfa_cancel_flag equal to 0, present in the same AU, and applying to the same OLS or layer to have the same content and follow in decoding order any NNPFA SEI message with the same nnpfa_target_id, having nnpfa_cancel_flag equal to 1, present in the same AU, and applying to the same OLS or layer.

–	When there are multiple SEI messages with payloadType equal to 211, nnpfa_cancel_flag equal to 0, and the same nnpfa_target_id value that are associated with a particular AU or DU and apply to a particular OLS or layer, regardless of whether some or all of these SEI messages are scalable-nested, these SEI messages shall have the same SEI payload content and shall follow, in decoding order, the SEI messages, if any, that have payloadType equal to 211, nnpfa_cancel_flag equal to 1, and the same nnpfa_target_id value and are associated with the same particular AU or DU and apply to the same particular OLS or layer.

# Correction of the condition for repeated inference

## Problem statement

The VVC text for the use of NNPF SEI messages includes repeated inference at the end of a bitstream to generate interpolated pictures up to the end of the bitstream. It is asserted that when an NNPF performs both picture rate upsampling and another filtering purpose, the repeated inference could cause output a filtered picture that is beyond the end of the bitstream (i.e., another filtered version of the last picture of the bitstream in output order). It is proposed to constrain the repeated inference to NNPFs that perform only picture rate upsampling.

## Proposal

–	If pictureRateUpsamplingFlag is equal to 1  nnpfc_purpose is equal to 8 (i.e., the only purpose for the NNPF is picture rate upsampling), the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, nnpfc_interpolated_pics[ i ] is greater than 0 only for a single value of i that is greater than 0, and currPic is the last picture of the bitstream in output order that has nuh_layer_id equal to currLayerId, the variable numPostRoll is set equal to the value of i such that nnpfc_interpolated_pics[ i ] is greater than 0 and the variable numInferences is set equal to 1 + numPostRoll.

# Correction of the index range in input picture assignment for repeated inference at the end of a bitstream

## Problem statement

The VVC text for the use of NNPF SEI messages includes repeated inference at the end of a bitstream to generate interpolated pictures up to the end of the bitstream. When a persistently activated NNPF is applied at the end of the bitstream, the number of inferences, numInferences, may be greater than 1 and some input pictures are repetitions of the last picture of the bitstream in output order.

It is asserted that there is a mistake, highlighted below in yellow, that results into less input pictures being selected for the NNPF than indicated by the NNPFC SEI message (with nnpfc_num_input_pics_minus1).

The variable numInputPics is set equal to nnpfc_num_input_pics_minus1 + 1.

…

For each value of j in the range of 0 to numInferences − 1, inclusive, the following applies:

–	The arrays inputPic[ i ] and inputPresentFlag[ i ] for i in the range of 0 to numInputPics − 1, inclusive, representing all the input pictures and the presence of input pictures within the bitstream, respectively, are specified as follows:

–	When j is greater than 0, for each value of k in the range of 0 to j − 1, inclusive, inputPic[ k ] is set to be currPic and inputPresentFlag[ k ] is set equal to 0.

–	The j-th input picture, inputPic[ j ], is set equal to be currPic and inputPresentFlag[ j ] is set equal to 1.

–	When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i:

–	… (selection of the i-th input picture among pictures that are present in the bitstream)

## Proposal

It is proposed to remove "− j" from "When numInputPics is greater than 1, the following applies for each value of i in the range of j + 1 to numInputPics − j − 1, inclusive, in increasing order of i:".

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).