| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0059-v1 |
| --- | --- |

| Title: | EE2-3.1: Cross-component residual model (CCRM) for inter prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pekka Astola Jani Lainema | Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports test results of EE2-3.1a and EE2-3.1b, where cross-component residual model (CCRM) is investigated. The tests were implemented on top of ECM-9.0 software. Test EE2-3.1b uses a simpler method for obtaining the offsets used by the division-free Gaussian elimination in solving CCRM filter coefficients but is otherwise identical to test EE2-3.1a. The summarized results are reported as follows.

For test EE2-3.1a the impact on coding efficiency and runtimes over ECM-9.0 is reportedly {for Y, U, V, EncT, DecT}: RA {-0.17%, -1.59%, -1.82%, 102%, 99%}, LB {0.11%, -4.21%, -3.86%, 102%, 99%}.

For test EE2-3.1b the impact on coding efficiency and runtimes over ECM-9.0 is reportedly {for Y, U, V, EncT, DecT}: RA {}, LB {0.08%, -4.29%, -3.84%, 99%, 97%}.

The gains are asserted to be larger for high resolution sequences for both tests. The chroma gains are asserted to be larger for both tests in low delay configuration.

# Proposed method

## Introduction

It is proposed to apply cross-component residual model (CCRM) to predict chroma samples from reconstructed luma samples when the block uses inter prediction or intra block copy (IBC). Fig. 1 illustrates the decoder side of the method. The cross-component filters are derived using the prediction blocks of luma and chroma. The derived filters are applied to the reconstructed luma block and blended with the prediction blocks of chroma to produce the final chroma prediction blocks. In the blending process the filtered reconstructed luma blocks use blending weight of 0.75 and chroma prediction blocks use blending weight of 0.25.

Fig. 1 The proposed method on the decoder.

## Convolutional filter and calculation of filter coefficients

The proposed 8-tap filter consist of 6 spatial luma samples, a nonlinear term, and a bias term. The spatial luma samples (L0,…,L5) are obtained from the luma grid selecting the 6 luma samples closest to the chroma position C without down sampling as shown in Fig. 2. The predicted chroma value is obtained as,

predChromaVal = c0 L0+ c1L1 + c2L2 + c3L3 + c4L4 + c5L5 + c6 nonlinear((L0+L3+1) >> 1) + c7 B,

where nonlinear is CCCM’s nonlinear operator and B is bias.

Fig. 2 Luma samples L0,..,L5 in relation to the chroma sample C.

The filter coefficients are derived using ECM’s division-free Gaussian elimination method and the necessary offsets are applied to samples prior to filter derivation.

For filter coefficient derivation at most 256 chroma samples are used.

## Bitstream signalling

Usage of the mode is signalled with a CABAC coded TU level flag. One new CABAC context was included to support this. The CCRM flag is only signalled if the TU’s luma Cbf is non-zero and the CU’s predMode is either MODE_INTER or MODE_IBC.

## Encoder operation

The encoder performs an RD decision in the transform selection loop for the chroma components when luma Cbf is non-zero and the CU’s predMode is either MODE_INTER or MODE_IBC.

## Description of EE2 Test 3.1a

EE2 Test 3.1a tests the CCRM method as described in above subsections. In this test the offsets for ECM’s division-free Gaussian elimination method (for solving the CCRM filter coefficients) are obtained for luma and chroma prediction blocks using ECM’s buffer.computeAvg() method.

## Description of EE2 Test 3.1b

In this test the offsets for ECM’s division-free Gaussian elimination method (for solving the CCRM filter coefficients) are obtained using a four-point average of the luma and chroma prediction blocks, where the four points correspond to the top-left, top-right, bottom-left and bottom-right corners of the blocks. Otherwise, this test is identical to EE2 Test 3.1a.

# Experimental results

## Test 3.1a

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26 % | -2.32 % | -2.90 % | 103 % | 100 % |
| Class A2 | -0.81 % | -2.41 % | -3.32 % | 101 % | 99 % |
| Class B | 0.06 % | -1.20 % | -1.15 % | 101 % | 99 % |
| Class C | 0.08 % | -0.90 % | -0.72 % | 102 % | 99 % |
| Class E |  |  |  |  |  |
| Overall | -0.17 % | -1.59 % | -1.82 % | 102 % | 99 % |
| Class D | 0.09 % | -0.90 % | -0.55 % | 103 % | 100 % |
| Class F | 0.05 % | -0.62 % | -0.66 % | 100 % | 97 % |
| Class TGM | -0.03 % | -0.49 % | -0.57 % | 101 % | 100 % |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11 % | -8.02 % | -6.23 % | 101 % | 99 % |
| Class C | 0.12 % | -2.28 % | -2.76 % | 104 % | 99 % |
| Class E | 0.10 % | -0.43 % | -1.36 % | 100 % | 99 % |
| Overall | 0.11 % | -4.21 % | -3.86 % | 102 % | 99 % |
| Class D | -0.03 % | -2.81 % | -2.76 % | 100 % | 98 % |
| Class F | -0.22 % | -4.31 % | -4.68 % | 103 % | 96 % |
| Class TGM | -0.21 % | -3.08 % | -4.32 % | 105 % | 99 % |

## Test 3.1b

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.07 % | -1.27 % | -1.05 % | 100 % | #NUM! |
| Class C | 0.08 % | -0.73 % | -0.73 % | 101 % | #NUM! |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.13 % | -0.89 % | -0.75 % | 103 % | #NUM! |
| Class F | 0.05 % | -0.59 % | -0.72 % | 100 % | #NUM! |
| Class TGM | -0.02 % | -0.50 % | -0.56 % | 100 % | #NUM! |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 Full |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.11 % | -8.43 % | -6.21 % | 99 % | 98 % |
| Class C | 0.09 % | -2.21 % | -2.75 % | 101 % | 96 % |
| Class E | 0.02 % | -0.15 % | -1.32 % | 98 % | 97 % |
| Overall | 0.08 % | -4.29 % | -3.84 % | 99 % | 97 % |
| Class D | -0.11 % | -1.88 % | -3.14 % | 103 % | 96 % |
| Class F | -0.08 % | -4.75 % | -5.42 % | 100 % | 94 % |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).