| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 31st Meeting, Geneva, CH, 11–19 July 2023 | Document: JVET-AE0073-v2 |
| --- | --- |

| Title: | Non-EE2: IBC-LIC Model Merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang, Yue Yu, Haoping Yu, Dong Wang | Email: | zhanglai@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution proposes an IBC-LIC model merge mode that inherits IBC-LIC model parameters from spatial adjacent and non-adjacent neighbors, history candidates, or default models. The proposed method is implemented on top of ECM-9.0 software, it shows coding gains as follows:

AI: { -0.56% Y, -0.50% U, -0.45% V, 101.9% EncT, 100.9% DecT } for class F

{ -0.17% Y, -0.16% U, -0.15% V, 101.9% EncT, 101.3% DecT } for class TGM

RA: { -0.39% Y, -0.37% U, -0.35% V, 100.6% EncT, 102.3% DecT } for class F

{ -0.04% Y, -0.08% U, -0.09% V, 101.1% EncT, 102.4% DecT } for class TGM

Introduction

Intra block copy with local illumination compensation (IBC-LIC) is an adopted tool in the current ECM. It compensates the local illumination variation within a picture between the CU coded with IBC and its prediction block with a linear equation, that is α*p[x]+β. The parameters of the linear equation are derived using the same method as LIC for inter prediction except that the reference template is generated with a block vector in IBC-LIC. IBC-LIC can be applied to the IBC AMVP mode and the IBC merge mode. For the IBC AMVP mode, an IBC-LIC flag is signalled to indicate the usage of IBC-LIC. For the IBC merge mode, the IBC-LIC flag is inferred from the merge candidate.

Proposed methods

In this contribution, the IBC-LIC model merge mode is proposed to inherit IBC-LIC model parameters from previously coded blocks. More specifically, a LIC model for an IBC-LIC model merge mode can be obtained as follows:

- Construct a model candidate list which consists of model parameters from spatial adjacent and non-adjacent neighbors, history candidates, and default models. The size of the candidate list is twelve.

Similar to the CCMerge method, LIC models are collected from the previously coded IBC-LIC and IBC-LIC model merge adjacent and non-adjacent positions. Besides, a history IBC-LIC model table with a size of six is maintained similar to the HMVP table. The LIC models from spatial neighbors and the history IBC-LIC model table are added to the IBC-LIC model merge candidate list. If the list is not full, a default model and the scaled models are then added to the list. To avoid redundant models, a pruning operation is also applied.

- Calculate the offset of each model candidate

For a certain inherited IBC-LIC model parameter set (α and β), an offset can be calculated from the template as:

totalDiff=(Rec-(α*Ref+β))

offset = totalDiff / N

where Rec and Ref are pixels from the template of the current block and reference block, N is the total number of pixels in the template area. β is then modified as β=β+offset.

- Select an IBC-LIC model from the candidate list and signal its index in the bitstream.

A flag is signaled to indicate whether the IBC-LIC model merge mode is applied or not. If this flag is true, an index is further signaled to indicate which candidate model is used by the current block. More specifically, the proposed mode is explicitly signaled both in the IBC-AMVP and the IBC-Merge mode. In the IBC-AMVP mode, the flag is signaled when the IBC-LIC flag is true. In the IBC-Merge mode, the flag is only signaled when the current block isn’t coded as IBC-CIIP, IBC-GPM, TM-Merge, or skip mode. If the IBC-LIC model merge flag is true, the regular inheritance of IBC-LIC flag won’t be applied, and the current block is treated as regular IBC-LIC by other blocks.

Experimental results

The proposed method is implemented on top of ECM-9.0 [1]. The test results are obtained following the common test conditions [2]:

Table 1. The proposed method on top of ECM-9.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.56% | -0.50% | -0.45% | 101.9% | 100.9% |
| Class TGM | -0.17% | -0.16% | -0.15% | 101.9% | 101.3% |

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-9.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class F | -0.39% | -0.37% | -0.35% | 100.6% | 102.3% |
| Class TGM | -0.04% | -0.08% | -0.09% | 101.1% | 102.4% |

Conclusion

This contribution proposes an IBC-LIC model merge mode. The results show that the proposed method can improve the coding efficiency of ECM with minor complexity increases. It is suggested to further study this method in EE2.

References

- ECM9.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-9.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).